| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0103-v1 |
| --- | --- |

| Title: | CE11.3.4: Parallel deblocking for 4 x N and N x 4 block boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM2.0.1 deblocking is only considered on 8x8 grids. In this contribution, it is proposed to apply deblocking for 4xN and Nx4 block boundaries on not only 8x8 grids but also non-8x8 grids. In addition, deblocking processes are modified for 4xN and Nx4 block boundaries to allow parallel deblocking of all parallel block boundaries. Comparing with VTM2.0.1, simulation results reportedly show 0.00%, -0.15%, -0.26%, and -0.25% luma BD-rates for AI, RA, LB, and LP configurations, respectively, with negligible run-time changes. Comparing with VTM2.0.1 with ALF switched off, simulation results reportedly show -0.02%, -0.25%, -0.22%, and -0.39% luma BD-rates for AI, RA, LB, and LP, respectively, with negligible run-time changes. It is asserted that the subjective quality especially for low resolution sequences is improved.

# Introduction

In HEVC and VTM2.0.1 [1], deblocking filtering decisions are made separately for each boundary of four-sample length that lies on the grids dividing the picture into blocks of 8×8 samples. Only boundaries on the 8×8 grids, which are PU or TU boundaries, are deblocked. However, as binary-tree and ternary-tree partitioning types are used in VTM2.0.1, many 4×N and N×4 blocks are generated and no deblocking is applied if these block boundaries are not on the 8x8 grids. In this contribution, a deblocking method for 4xN and Nx4 block boundaries is proposed.

Some definitions in this contribution are described as follows. The left side and right side of a vertical block edge are called p-side and q-side, respectively. The upper side and lower side of a horizontal block edges are called p-side and q-side, respectively. In addition, each block edge is of 4-sample length by definition, and pji and qji (i=0, 1, 2, or 3; and j=0, 1, 2, …) represent the p-side and q-side samples on the i-th line perpendicular to the block edge, j equal to 0 means the sample is adjacent to the block edge, and a larger j means the sample is at a larger distance from the block edge. The CB length refers to CB width and CB height for horizontal filtering across vertical block edge and vertical filtering across horizontal block edge, respectively.

# Proposed method

To support deblocking all 4xN and Nx4 block boundaries and achieve parallel deblocking for all parallel block boundaries, the following changes are made to HEVC deblocking filters (DF):

- When HEVC filtering decision and filtering operations are performed, if the CB length at p-side is equal to four, then p1i is used to replace p2i and p3i.

- When HEVC filtering decision and filtering operations are performed, if the CB length at q-side is equal to four, then q1i is used to replace q2i and q3i.

- When HEVC normal/weak filtering is used and CB length at p-side or q-side is equal to four, then only samples p0i and q0i are modified.

- When HEVC strong filtering is used and CB length at p-side or q-side is equal to four, then only samples p1i, p0i, q0i, and q1i are modified.

# Experimental results

The proposed deblocking method for 4×N and N×4 block boundaries has been implemented on VTM2.0.1 [1], and the objective results are summarized in the Table 1 and Table 2. The simulations were performed following the JVET common test conditions [2].

# Conclusions

This contribution proposes to support deblocking for all 4xN and Nx4 block boundaries while keeping the feasibility of parallel deblocking all parallel block boundaries. The proposed DF leads to small BD-rate savings with negligible run-time changes. It is asserted that blocky artifacts are reduced by the proposed DF especially for low resolution sequences with many 4xN and Nx4 blocks.

# References

- VTM2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Objective results of the proposed deblocking method on top of VTM2.0.1 for CTC configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.04% | 0.00% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class E | -0.13% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.02% | -0.01% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.08% | 0.04% | 100% | 103% |
| Class A2 | -0.25% | 0.01% | -0.07% | 100% | 99% |
| Class B | -0.10% | -0.04% | -0.02% | 100% | 100% |
| Class C | -0.21% | 0.03% | 0.00% | 100% | 100% |
| Overall | -0.15% | 0.01% | -0.01% | 100% | 100% |
| Class D | -0.19% | -0.15% | -0.22% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.11% | -0.06% | -0.16% | 100% | 100% |
| Class C | -0.25% | -0.04% | -0.25% | 100% | 101% |
| Class E | -0.53% | 0.29% | -0.35% | 101% | 101% |
| Overall | -0.26% | 0.03% | -0.24% | 101% | 100% |
| Class D | -0.19% | 0.53% | 0.08% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.15% | 0.19% | 0.23% | 100% | 100% |
| Class C | -0.23% | -0.24% | -0.26% | 100% | 101% |
| Class E | -0.44% | -0.13% | 0.06% | 100% | 100% |
| Overall | -0.25% | -0.03% | 0.02% | 100% | 101% |
| Class D | -0.14% | -0.11% | -0.26% | 100% | 100% |

Table 2.  Objective results of the proposed deblocking method on top of VTM2.0.1 for CTC with ALF switched off configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Class C | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class E | -0.16% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.01% | 0.00% | 0.01% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.21% | -0.05% | 100% | 101% |
| Class A2 | -0.44% | -0.09% | -0.04% | 101% | 102% |
| Class B | -0.19% | -0.05% | -0.06% | 100% | 101% |
| Class C | -0.27% | -0.02% | -0.02% | 100% | 101% |
| Overall | -0.25% | -0.08% | -0.04% | 100% | 101% |
| Class D | -0.22% | -0.03% | -0.12% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.17% | 0.09% | -0.02% | 101% | 99% |
| Class C | -0.28% | -0.10% | -0.16% | 100% | 101% |
| Class E | -0.23% | -0.04% | 0.06% | 101% | 99% |
| Overall | -0.22% | -0.01% | -0.05% | 100% | 100% |
| Class D | -0.20% | -0.16% | -0.40% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.33% | -0.17% | 0.12% | 100% | 99% |
| Class C | -0.34% | 0.20% | -0.06% | 100% | 100% |
| Class E | -0.55% | 0.08% | 0.11% | 101% | 102% |
| Overall | -0.39% | 0.02% | 0.06% | 100% | 100% |
| Class D | -0.34% | -0.65% | -0.62% | 100% | 100% |