| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0105 |
| --- | --- |

| Title: | CE4-related: A second ATMVP candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang 8910 University Center Ln, Suit 400 San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-705-2600 yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, a new ATMVP-like merge candidate is generated and inserted into the merge candidate list in addition to the original ATMVP. The derivation process of the new ATMVP is similar to the original ATMVP, with slightly different rules used in selecting motion vectors (MV) from corresponding reference sub-blocks. More specifically, the MV selection rules for TMVP from a reference block are used in deriving MVs for each subblock under the new ATMVP mode. Simulation results show that the new ATMVP mode provides -0.23% BD-rate saving under RA with negligible encoding/decoding run time difference.

# Introduction

In the original ATMVP, for each subblock within a CU, its motion vectors are derived from corresponding MVs in a collocated picture. First, based on a temporal vector identified, a corresponding reference subblock in the collocated picture is located, as shown in Figure 1. Then its motion vectors are derived based on procedures listed in Table 1 for selecting its reference MV, followed by appropriate MV scaling if needed.

Figure 1: ATMVP motion prediction for a CU

Table 1: Reference MV selection for ATMVP

If the current picture has no backward prediction,

If the L[X] MV of the reference subblock is available,

L[X] MV of the reference subblock is selected.

Otherwise,

L[1-X] MV of the reference subblock is selected.

Otherwise,

L[X] MV of the reference subblock is selected if available.

In the table, L[X] (with X being 0 or 1) denotes reference list X. According to Table 1, in case that the current picture has backward prediction, the derived MVs for a subblock can be either uni-directional or bi-directional.

# Proposed method

In this proposal, we propose to generate and insert a new ATMVP-like merge candidate into merge candidate list, in addition to the original ATMVP. The derivation process of the new ATMVP is similar to the original ATMVP, except that slightly different rules are used in selecting the reference MVs.

As illustrated in the Table 2, the reference MV selection for the proposed ATMVP is the same as that used for the conventional TMVP. In Table 2, listCol denotes the collocated picture list index.

It is worth noting that when a current picture has no backward prediction, the MV selection process of the new ATMVP and the original ATMVP are identical. Therefore, the proposed ATMVP is only derived and inserted into the merge list when the current picture has backward prediction.

Table 2: Reference MV selection for the new ATMVP

If current picture has no backward prediction,

If the L[X] MV of the reference block is available,

L[X] MV of the reference block is selected.

Otherwise,

L[1-X] MV of the reference block is selected.

Otherwise,

If the L[1- listCol] MV of the reference block is available,

L[1- listCol] MV of the reference block is selected.

Otherwise,

L[listCol] MV of the reference block is selected.

# Simulation results

The table below shows the BD-rate performance and processing time of the method proposed. The simulations are conducted following the common test conditions [1] on top of reference software VTM-2.0.1 [2] and the anchor is VTM-2.0.1 using the VTM configurations.

Table 1. Performance of the proposed scheme under Random Access

# Conclusions

In this proposal, a new ATMVP candidate is proposed to be added into merge candidate list for the case when a current picture has backward prediction. The new ATMVP provides -0.23% BD-rate saving with negligible encoding/decoding run time difference. It is recommended to adopt the proposed second ATMVP into VVC to improve the coding performance with negligible complexity impact.

# Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-K1010, July 2018.

- VTM-2.0.1 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).