| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0109-v2 |
| --- | --- |

| Title: | Non-CE3: (LM only)+ (CCCLM) coding performance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Jianglin Wang Xiaoyan Chai Shuai Wan Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn jlwang_2@stu.xidian.edu.cn xychai@stu.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

Abstract

This contribution presents the coding performance by adding CCCLM (proposed in JVET-L108) as an additional LM mode. CCLM and CCCLM are two LM methods which are decided through RD optimization process. The simulation results reportedly show that -0.13%, -2.27% and -2.52% BD rate savings on Y, Cb and Cr components for All Intra configuration.

Introduction

In VTM2.0.1, LM only has been adopted. The linear model parameters α and β are derived based on the neighbouring reconstructed luma and chroma samples.

To explore the statistics in the current luma, CCCLM is proposed which uses the reconstructed luma and the temporary chroma of the current block to derive the parameters α and β. In JVET-L0108, CCCLM replaces the LM only method to conduct the CCLM.

Proposed method

In this proposal, CCCLM is used as an additional LM mode. For the chroma block, both LM and CCCLM are checked; the one with minimum RDcost will be selected as the LM mode. The mode index of CCCLM is set to 68.

Experimental results

The proposed scheme is implemented based on BMS 2.0.1 with the VTM configuration. Simulations are performed under All Intra configuration. The TemporalSubsampleRatio is set to 24.

Test #1:

Anchor: VTM2.0.1

Tested: CCCLM

Table 1. Summary of experimental results (CCCLM vs. VTM2.0.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.13% | -0.06% |  | 133% |
| Class A2 | -0.07% | -0.55% | -0.51% |  | 115% |
| Class B | -0.10% | -0.62% | -0.83% |  | 117% |
| Class C | -0.21% | -0.82% | -0.99% |  | 112% |
| Class E | -0.06% | -0.58% | -0.51% |  | 108% |
| Overall | -0.14% | -0.57% | -0.63% |  | 116% |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Test #2:

Anchor: VTM2.0.1

Tested: LM only + CCCLM

Table 2. Summary of experimental results(LM only + CCCLM vs. VTM2.0.1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -3.65% | -3.40% |  | 136% |
| Class A2 | -0.15% | -2.27% | -1.50% |  | 116% |
| Class B | -0.06% | -1.94% | -2.88% |  | 119% |
| Class C | -0.13% | -2.35% | -2.89% |  | 110% |
| Class E | -0.07% | -1.32% | -1.59% |  | 112% |
| Overall | -0.13% | -2.27% | -2.52% |  | 118% |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

In this contribution, it is proposed to add CCCLM as an additional LM mode for the chroma block. The proposed method gives -0.13%, -2.27% and -2.52% BD rate reduction on Y, Cb and Cr components for All Intra configuration.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).