| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0116-v1 |
| --- | --- |

| Title: | CE5: Per-context CABAC initialization with single window (Test 5.1.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Jie Dong, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for Test CE5.1.7. In this test, the CABAC engine uses a single window to estimate the binary symbol probabilities, where the window parameters are different for different contexts and are stored in a pre-defined look-up table (LUT). Under the CE5 test conditions [1], the average coding gains are 0.07% AI, -0.03% RA, and -0.02% LDB.

# Introduction

The CABAC probability estimation method of this test was proposed in JVET-K0385 [2]. Denote P[k] as the probability for binary arithmetic coding and P[k] has 15-bit precision. The single-window probability estimation is implemented using the following recursive equations.

If the current bin is 1, P[k] is updated to P[k+1] by (1),

P[k+1] = P[k] + ((215 – P[k]) >> a)                                                    (1)

otherwise, if the current bin is 0, P[k] is updated to P[k+1] by (2),

P[k+1] = P[k] – (P[k] >> a)                                                           (2)

where a denotes window shift parameter which can be different for each context. Since the range of parameters are restricted to 2 ≤ a ≤ 9, the proposed method requires 3 bits of ROM per-context.

# Results

The method is integrated into the modified BMS2.0.1 software, called CEM1, where CABACEngine 1 is used, the parameters for double windows are 4 and 7, respectively, and the interval subdivision table size is 512×64. VTM configuration is used.

Table 1 Comparison of CEM1 with the proposed method integrated against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.24% | 0.33% | 89% | 94% |
| Class A2 | 0.03% | 0.35% | 0.20% | 95% | 95% |
| Class B | 0.06% | 0.16% | 0.15% | 89% | 96% |
| Class C | 0.07% | -0.03% | -0.07% | 106% | 100% |
| Class E | 0.11% | 0.09% | -0.02% | 100% | 96% |
| Overall | 0.07% | 0.15% | 0.11% | 95% | 96% |
| Class D | -0.03% | -0.23% | -0.32% | 117% | 99% |
| Class F (optional) | 0.19% | 0.20% | 0.35% | 100% | 97% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.73% | 0.68% | 100% | 98% |
| Class A2 | 0.07% | 1.16% | 0.60% | 102% | 98% |
| Class B | -0.06% | 0.82% | 0.53% | 104% | 95% |
| Class C | -0.13% | 0.31% | 0.05% | 113% | 100% |
| Overall | -0.03% | 0.73% | 0.45% | 105% | 97% |
| Class D | -0.37% | 0.16% | 0.12% | 127% | 100% |
| Class F (optional) | 0.04% | 0.26% | 0.42% | 121% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.07% | 1.02% | 0.70% | 100% | 102% |
| Class C | -0.09% | 0.73% | 0.36% | 101% | 95% |
| Class E | -0.09% | -0.36% | 0.09% | 100% | 93% |
| Overall | -0.02% | 0.58% | 0.43% | 100% | 97% |
| Class D | -0.28% | 0.41% | -0.18% | 112% | 97% |
| Class F (optional) | 0.06% | 0.53% | 0.40% | 100% | 92% |

In addition, the performance of the proposed method using an 8×8 table (CE5.2.4) is also tested and compared with CEM1 anchor, as shown in Table 2.

Table 2 Comparison of the proposed method using an 8×8 table against CEM1 anchor

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.21% | 0.30% | 89% | 96% |
| Class A2 | 0.01% | 0.34% | 0.18% | 95% | 95% |
| Class B | 0.05% | 0.15% | 0.14% | 91% | 98% |
| Class C | 0.07% | -0.03% | -0.07% | 101% | 103% |
| Class E | 0.12% | 0.11% | -0.01% | 100% | 98% |
| Overall | 0.06% | 0.14% | 0.10% | 95% | 98% |
| Class D | -0.01% | -0.20% | -0.30% | 101% | 101% |
| Class F (optional) | 0.20% | 0.21% | 0.36% | 98% | 101% |
|  | Random Access |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.72% | 0.67% | 101% | 100% |
| Class A2 | 0.06% | 1.15% | 0.59% | 105% | 100% |
| Class B | -0.07% | 0.81% | 0.52% | 104% | 97% |
| Class C | -0.12% | 0.32% | 0.06% | 105% | 94% |
| Overall | -0.04% | 0.73% | 0.44% | 104% | 97% |
| Class D | -0.35% | 0.17% | 0.14% | 109% | 93% |
| Class F (optional) | 0.06% | 0.27% | 0.43% | 117% | 94% |
|  | Low Delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.05% | 0.99% | 0.67% | 102% | 99% |
| Class C | -0.09% | 0.73% | 0.36% | 102% | 104% |
| Class E | -0.07% | -0.35% | 0.10% | 101% | 97% |
| Overall | -0.03% | 0.57% | 0.43% | 102% | 100% |
| Class D | -0.26% | 0.42% | -0.17% | 102% | 100% |
| Class F (optional) | 0.07% | 0.54% | 0.41% | 99% | 93% |

# References

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-K1025, 11th Meeting, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, Y.-H. Chao, M. Karczewicz, V. Seregin, “CE5-related: context state memory reduction,” JVET-K0385, 11th Meeting, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).