| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0130-v1 |
| --- | --- |

| Title: | CE3-3.1.1 : Interpolation filter selection regarding intra mode and block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com jin78.heo@lge.com jangwon84.choi@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a method to select a suitable interpolation filter for intra prediction. In the proposed method, appropriate interpolation filter is selected between cubic and Gaussian filters considering intra prediction direction and the block size. In Test CE3-3.1.1, the interpolation filters in JEM were used. The simulation results reportedly show that Y BD-rate decreases 0.41% and 0.18% for VTM-AI, VTM-RA, respectively.

Proposed method

At the last meeting, an interpolation selection algorithm was proposed in JVET-K0097 [1]. In the proposed method, a four-tap or six-tap intra interpolation filter is used for directional intra prediction filtering. Two types of interpolation filters are used: (i) Cubic interpolation filters for blocks with the size smaller than or equal to 64 samples with the intra modes whose absolute angles are less than 11 (±6 modes from horizontal and vertical modes), and (ii) Gaussian interpolation filters for blocks with the size larger than 64 samples with the intra modes whose absolute angles are equal to or larger than 11.

In Test 3.1.1 in CE3, the interpolation filters identical to the ones in JEM are used.

Experimental results

The proposed method has been implemented using BMS-2.0.1 as software base and experimentally evaluated both for VTM and BMS test according to the common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1023 [3].

Table 1 Test 3.1.1 VTM results

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 AI |  |  | Over VTM-2.0.1 RA |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.18% | -0.16% | -0.15% | -0.13% | -0.02% | -0.02% |
| Class A2 | -0.15% | -0.07% | -0.14% | -0.06% | -0.10% | -0.04% |
| Class B | -0.41% | -0.41% | -0.40% | -0.21% | -0.15% | -0.08% |
| Class C | -0.65% | -0.78% | -0.69% | -0.28% | -0.15% | -0.11% |
| Class E | -0.59% | -0.71% | -0.64% |  |  |  |
| Overall | -0.41% | -0.44% | -0.42% | -0.18% | -0.11% | -0.07% |
| Class D | -0.53% | -0.45% | -0.69% | -0.25% | -0.53% | -0.13% |
| Enc Time [%] | 101% |  |  | 100% |  |  |
| Dec Time [%] | 101% |  |  | 100% |  |  |

References

- S. Yoo, J. Heo, J. Choi, L. Li, J. Lim, “CE3-2.3.3 and CE3-2.3.4:Interpolation filter selection regarding intra mode and block size,” Document of Joint Video Experts Team (JVET), JVET-K0097, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-K1023, 11th Meeting: Ljubljana, SI, 10–18 July 2018

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).