| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0143_r2 |
| --- | --- |

| Title: | CE4: Slice-level 4/6 parameters affine model switching (Test 4.1.8) and bypass coding of 4/6 parameter indication flag (Test 4.1.14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeho Lee Junghak Nam Naeri Park Hyeongmun Jang Jaeho Lee Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {jhlee.lee, junghak.nam, naeri.park, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This document reports the results of slice-level 4/6 parameters affine model switching (Test 4.1.8) and bypass coding of affine type flag (Test 4.1.14) in JVET-K0095.

In Test 4.1.8, test result shows that slight loss for random access and low delay B, respectively, compared to VTM-2.0.1. However encoding time is reduced.

In Test 4.1.14, test results shows that no coding loss, compared to VTM-2.0.1.

Introduction

Slice-level 4/6 parameters affine model switching (Test 4.1.8)

The affine types are adaptively selected at slice level as follows;

Slice header flags affine_type_4_flag and affine_type_6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied or not. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. To determine the affine_type_4_flag and affine_type_6_flag for the current slice the statistics from the previous slice are used.

Bypass coding of affine type flag (Test 4.1.14)

Affine type flag is bypass coded to reduce the CABAC throughput.

Experimental results

The proposed methods have been implemented on the top of VTM-2.0.1 and tested based on JVET SDR common test condition.

Test 4.1.8

Table 1 summarizes the results of test 4.1.8.

Table 1: Experimental results of test 4.1.8

Test 4.1.14

Table 2 summarizes the results of test 4.1.14.

Table 2: Experimental results of test 4.1.14

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).