| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0144_r2 |
| --- | --- |

| Title: | CE4-related: Composite merge candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaeho Lee Junghak Nam Naeri Park Hyeongmun Jang Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {jhlee.lee, junghak.nam, naeri.park, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution presents the average candidate for the merge candidate list construction. A-candidate is generated as the average of A1 and B1 candidates, and two B-candidates are generated as the average of A1 and T, and B1 and T, if the reference indices of two candidates are same. For A-candidate, the coding performances are -0.30% in RA and -0.10% in LDB. For two B-candidates, the coding performances are -0.25% in RA and -0.08% in LDB. When combine A-candidate and B-candidate, the coding gains are -0.42% in RA and -0.12% in LDB. The proposed scheme performs without scaling process and pruning check.

Technical description

In VTM-2.0.1, the merge candidate list construction is performed as the following order, and A0, A1, B0, B1, B2, and T are shown in Figure 1.

- {A1, B1, B0, A0} candidates

- ATMVP

- {B2} candidate

- TMVP {T}

- Combined bi-predictive

- Zero

Figure 1: Positions for merge candidate list construction

The following candidates are generated and used for the merge list construction.

1) A-candidate: after spatial candidates {A1, B1} are considered, A-candidate is generated as the average of A1 candidate and B1 candidate if two candidates are available and the reference indices of two candidates are same.

2) B-candidates: after TMVP (T) is checked, the maximum two B-candidates are generated and inserted into the candidate list. The first B-candidate is generated as the average of A1 and T if A1 and T are available and the reference indices of two candidates are same, and the second B-candidate is generated as the average of B1 and T, if B1 and T candidates are available and the reference indices of two candidates are same.

3) C-candidates: after TMVP, the maximum six C-candidates are generated and inserted into the candidate list. The first 4 candidates in the merge list is used to generate C-candidates. If two candidates in the merge list have same reference indices, the average candidates are considered as C-candidate.

Experimental results

The proposed methods have been implemented on the top of VTM-2.0.1, and tested based on the common test conditions.

Three tests have been performed as listed in Table 1 and results are summarized in Table 2.

Table 1: Test specification of proposed candidate

| Test # | Test Description |
| --- | --- |
| 1 | A-candidate |
| 2 | B-candidates |
| 3 | Test 1 + Test 2 |
| 4 | C-candidates |

Table 2: Experimental results of test 1, test 2, test 3, and test 4.

Conclusion

Composite merge candidate based on average of two merge candidates is proposed. Experimental results shows BD-rate savings for random access and low delay B, respectively even though the proposed method does not introduce the computational complexity in terms of scaling process and pruning check. It is recommended to study the proposed method in CE.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).