| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0145-v1 |
| --- | --- |

| Title: | CE7-related: Constraints on context-coded bins for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shih-Ta Hsiang, Zhi-Yi Lin, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, shih-ta.hsiang, zhi-yi.lin, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the VTM2.0.1 coefficient coding, the averaged maximum number of context-coded bins per 16 samples is 2.56 times of that in HEVC. In this contribution, it is proposed to constrain the maximum number of context-coded bins to 32 for luma 4x4 sub-blocks, 16 for chroma 4x4 sub-blocks, and 4 for chroma 2x2 sub-blocks. The averaged maximum number of context-coded bins per 16 samples is reduced to 1.07 times of that in HEVC. The constraints on the maximum number of context-coded bins per coefficient sub-block are increased according to the last significant sub-block position without increasing the worst-case averaged maximum number of context-coded bins per 16 samples. To improve the parsing throughput and reduce the number of coding passes, greater than 2 flags are moved to the first coding pass after the parity bit. The proposed method is implemented on top of CE7.1.3.b. The simulation results reportedly show that, the average luma BD-rates are -0.18%, -0.08%, and -0.xx% in AI, RA, and LB cases respectively, and the average chroma BD-rates are 0.05%, 0.18%, and 0.xx% in AI, RA, and LB cases respectively.

# Introduction

In HEVC, for a 4x4 coefficient sub-block, the maximum number of context-coded bins per 16 samples is 24 or 25, including 15 or 16 significant flags depending on whether the last significant coefficient is in the sub-block, eight greater than 1 (gt1) flags, and one greater than 2 (gt2) flag. In VTM2.0.1, the dependent quantization in JVET-K0072 is adopted for coefficient coding. In the dependent quantization, for each coefficient, at most four context-coded bins are used for significant flag, parity flag, gt1 flag, and gt2 flag (sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag). For a 4x4 coefficient sub-block, the maximum number of context-coded bins is 64 if the sub-block does not contain the last significant coefficient and is 63 otherwise. For a 64-sample luma block (in four 4x4 sub-blocks) and two 16-sample chroma blocks (in two 4x4 sub-blocks), the worst case happens when none of these sub-blocks contain the last significant coefficient. The worst case maximum number of context-coded bins is 384, which is 2.56x of the averaged maximum number of context-coded bins of HEVC.

In CE7.1.3.b, a constraint on the maximum number of context-coded bins is proposed. For a coefficient coding sub-block, at most 32 context-coded bins can be used. In the 32 context-coded bins, four of them are reserved for the second coding pass of the gt2 flag coding. The same constraint is applied for luma 4x4 coefficient sub-block, chroma 4x4 coefficient sub-block, and chroma 2x2 coefficient sub-block. In CE7.1.3b, the worst case happens when every coding unit (CU) shape is Nx4 or 4xN, where the chroma transform block (TB) size is (N/2)x2 or 2x(N/2). The 2x2 coefficient sub-block is used for the chroma TBs. For the 2x2 chroma coefficient sub-block, at most 16 context-coded bins can be used (four significant flags, four gt1 flags, four parity flags, and four gt2 flags) except the last significant sub-block which has at most 15 context-coded bins (with one inferred significant flag). The averaged maximum number of context-coded bins per 16 samples in CE7.1.3.b is 42.66 bins, which is 1.71x of that of HEVC. Table 1 shows a comparison of HEVC, VTM2.0.1, and CE7.1.3.b.

Table 1. The averaged maximum number of context-coded bins per 16 samples in HEVC, VTM2.0.1, and CE7.1.3.b

| Number of maximum context-coded bins per sub-block | 4x4 luma sub-block | 4x4 chroma sub-block | 2x2 chroma sub-block | Maximum number of context-coded bins in one 64-sample luma block and two 16-sample chroma blocks | Averaged maximum number of context-coded bins per 16 samples |
| --- | --- | --- | --- | --- | --- |
| HEVC | 24 or 25 | 24 or 25 | N/A | 25*4 + 25*2 = 150 | 25 (1.00x) |
| VTM2.0.1 | 63 or 64 | 63 or 64 | 15 or 16 | 64*4 + 64*2 = 384 | 64 (2.56x) |
| CE7.1.3.b | 32 | 32 | 15 or 16 | 32*4 + 16*8 = 256 | 42.66 (1.71x) |

In this contribution, it is proposed to use different constraints according to color components and sub-block sizes. To improve coding efficiency, the constraints can be loosened if not all coefficient sub-blocks in a TB need to be coded. According to the parsed last significant coefficient position, the constraint can be loosen by increasing the allowed maximum number of context-coded bins. Besides, the gt2 flag is moved to the first coding pass after the parity bit coding. Including the gt2 flag in the first coding pass can improve the parsing throughput and reduce the number of coding passes.

# Proposed methods

## Different constraints according to color components and sub-block sizes

In this method, it is proposed to apply constraints depending on color components and coefficient sub-block sizes. In CE7.1.3.b, the worst case happens in (N/2)x2 or 2x(N/2) chroma TBs. To reduce the averaged maximum number of context-coded bins, the constraint value of 2x2 chroma sub-block is proposed to be one quarter of that of 4x4 chroma sub-block for CE7.1.3.b. However, from the analysis result in Table 2, the averaged maximum number of context-coded bins per 16 samples is still 28% larger than that in HEVC. Therefore, it is proposed to reduce the constraint values on 4x4 chroma sub-blocks and 2x2 chroma sub-blocks to 16 and 4, respectively. From the analysis, the averaged maximum number of context-coded bins per 16 samples is 26.66 bins and is only 7% larger than that in HEVC.

Table 2. The averaged maximum number of context-coded bins per 16 samples in HEVC, VTM2.0.1, CE7.1.3.b, and the proposed method

| Number of maximum context-coded bins | 4x4 luma sub-block | 4x4 chroma sub-block | 2x2 chroma sub-block | Maximum number of context-coded bins in one 64-sample luma block and two 16-sample chroma blocks | Averaged maximum number of context-coded bins per 16 samples |
| --- | --- | --- | --- | --- | --- |
| HEVC | 24 or 25 | 24 or 25 | N/A | 25*4 + 25*2 = 150 | 25.00 (1.00x) |
| VTM2.0.1 | 63 or 64 | 63 or 64 | 15 or 16 | 64*4 + 64*2 = 384 | 64.00 (2.56x) |
| CE7.1.3.b | 32 | 32 | 15 or 16 | 32*4 + 16*8 = 256 | 42.66 (1.71x) |
| Proposed modified CE7.1.3.b | 32 | 32 | 8 | 32*4 + 8*8 = 160 | 32.00 (1.28x) |
| Proposed Constraint | 32 | 16 | 4 | 32*4 + 4*8 = 160 | 26.66 (1.07x) |

## Last significant sub-block position dependent constraint

The constraint on the number of context-coded bins is to limit the worst-case number of context-coded bins. When not all the sub-blocks in a TB need to be coded, the constraint value can be loosened. According to the last significant sub-block position, the number of to-be-coded sub-blocks in a TB can be derived. The constraint value is determined according to the color component and the coefficient sub-block size. With the number of to-be-coded sub-blocks (NumToBeCodedSb) and the number of total coefficient sub-blocks in a TB (NumTotalSb), the constraint value (ConstraintValue) is modified as follows.

if (NumToBeCodedSb * 4 <= NumTotalSb)

ConstraintValue = ConstraintValue * 4;

else if (NumToBeCodedSb * 2 <= NumTotalSb)

ConstraintValue = ConstraintValue * 2;

else if (NumToBeCodedSb * 3 <= NumTotalSb * 2)

ConstraintValue = (ConstraintValue * 3) >> 1;

else if (NumToBeCodedSb * 5 <= NumTotalSb * 4)

ConstraintValue = (ConstraintValue * 5) >> 2;

## Include gt2 flag into the first coding pass

In VTM2.0.1, in the first coding pass, the significant flag is first parsed. If the significant flag is 1, a parity bit and a gt1 flag are parsed in order. In the second coding pass, the gt2 flags are parsed. The third coding pass is remaining level coding and the forth coding pass is sign bit coding. The context set selection of significant flag coding depends on the parity bit of the previous parsed coefficient. After parsing the parity bit, the time of parsing gt1 flag can be used to calculate the state and prepare the context variable set of the significant flag of the next coefficient. In CE7.1.3.b, the parity bit is moved after the gt1 flag to reduce the code word length of coefficient level equal to 1. However, there is no “hidden” time to calculate the state and prepare the context variable set of the significant flag of the next coefficient.

In this contribution, the gt2 flag is proposed to be moved to the first coding pass after the parity bit. By doing this, the time of parsing gt2 flag can be used to calculate the state and prepare the context set of the significant flag of the next coefficient. Also, the number of coding passes can be reduced from 4 to 3. The parsed gt2 flag is used to calculate the locSumAbsPass1in the context modeling of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag. Compared with CE7.1.3.b, the reserved number of context-coded bins for gt2 flag in the second coding pass is also merged into the number of context-coded bins for the first coding pass.

# Simulation results

The proposed methods are implemented on top of CE7.1.3.b. Five tests are implemented as follows.

Test-A: Set constraint values to 32 for luma 4x4 sub-block, 16 for chroma 4x4 sub-block, and 4 for chroma 2x2 sub-block

Test-B: Test-A + last significant sub-block position dependent constraint

Test-C: Test-B + include gt2 flag into the first coding pass

Test-D: Set constraint values to 32 for luma 4x4 sub-block, 32 for chroma 4x4 sub-block, and 8 for chroma 2x2 sub-block (proposed modified CE7.1.3.b).

Test-E: Test-D + include gt2 flag into the first coding pass

The common test conditions [1] are used for evaluation. The results of Test-A are shown in Table 3. The results of Test-B are shown in Table 4. The results of Test-C, Test-D, and Test-E are shown in Tables 5, 6, and 7, respectively.

Table 3.  Results of Test-A: Set constraint values to 32 for luma 4x4 sub-block, 16 for chroma 4x4 sub-block, and 4 for chroma 2x2 sub-block

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.37% | 0.59% | 109% | 103% |
| Class A2 | -0.80% | 0.61% | 0.49% | 110% | 102% |
| Class B | -0.04% | 0.77% | 0.34% | 110% | 101% |
| Class C | 0.07% | 0.49% | 0.39% | 111% | 101% |
| Class E | 0.15% | 0.16% | 0.11% | 110% | 101% |
| Overall | -0.10% | 0.51% | 0.38% | 110% | 101% |
| Class D | 0.11% | 0.43% | 0.22% | 111% | 101% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.68% | 0.67% | 103% | 103% |
| Class A2 | -0.24% | 1.25% | 1.28% | 104% | 98% |
| Class B | -0.14% | 0.66% | 0.41% | 104% | 101% |
| Class C | 0.08% | 0.84% | 0.60% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.83% | 0.69% | 104% | 100% |
| Class D | 0.11% | 0.34% | 0.72% | 105% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | 1.92% | 1.42% | 104% | 101% |
| Class C | -0.14% | 1.21% | 0.40% | 104% | 101% |
| Class E | -0.16% | -0.16% | -0.08% | 103% | 99% |
| Overall | -0.21% | 1.16% | 0.71% | 104% | 101% |
| Class D | -0.10% | 1.64% | 0.46% | 105% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 4.  Results of Test-B: Test-A + last significant sub-block position dependent constraint

LB data will be updated

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.34% | 0.27% | 110% | 104% |
| Class A2 | -0.84% | -0.21% | -0.31% | 111% | 103% |
| Class B | -0.02% | 0.19% | 0.12% | 111% | 101% |
| Class C | 0.04% | 0.24% | 0.38% | 112% | 100% |
| Class E | 0.16% | 0.07% | 0.08% | 110% | 100% |
| Overall | -0.11% | 0.14% | 0.13% | 111% | 102% |
| Class D | 0.07% | 0.22% | -0.03% | 112% | 101% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.42% | 0.27% | 103% | 105% |
| Class A2 | -0.17% | 0.59% | 0.40% | 104% | 97% |
| Class B | -0.11% | -0.05% | 0.24% | 104% | 104% |
| Class C | 0.07% | 0.32% | 0.36% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.27% | 0.31% | 104% | 101% |
| Class D | 0.07% | 0.56% | 0.44% | 106% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -1.36% | -1.60% | 5% | 35% |
| Class C | -0.07% | 0.57% | 0.11% | 105% | 99% |
| Class E | -0.23% | -0.35% | -0.12% | 102% | 98% |
| Overall | -0.16% | -0.47% | -0.66% | 30% | 64% |
| Class D | -0.06% | 0.29% | 0.07% | 105% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 5.  Results of Test-C: Test-B + include gt2 flag into the first coding pass

LB data will be updated

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.22% | 0.15% | 107% | 101% |
| Class A2 | -0.89% | -0.35% | -0.34% | 108% | 102% |
| Class B | -0.07% | 0.08% | 0.13% | 108% | 100% |
| Class C | -0.03% | 0.11% | 0.24% | 108% | 99% |
| Class E | 0.07% | 0.11% | 0.11% | 107% | 99% |
| Overall | -0.18% | 0.04% | 0.07% | 108% | 100% |
| Class D | 0.01% | 0.09% | 0.00% | 108% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | 0.26% | 103% | 103% |
| Class A2 | -0.18% | 0.55% | 0.21% | 103% | 102% |
| Class B | -0.17% | -0.18% | 0.11% | 103% | 102% |
| Class C | 0.04% | 0.32% | 0.23% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | 0.18% | 0.19% | 103% | 101% |
| Class D | 0.05% | 0.13% | 0.58% | 104% | 99% |
| 0 |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.56% | -0.78% | 0.39% | 5% | 34% |
| Class C | -0.13% | 0.38% | -0.15% | 103% | 98% |
| Class E | -0.20% | 0.64% | -0.92% | 102% | 99% |
| Overall | -0.32% | -0.04% | -0.12% | 30% | 64% |
| Class D | -0.12% | 0.53% | 0.38% | 104% | 99% |
| 0 |  |  |  |  |  |

Table 6.  Results of Test-D: Set constraint values to 32 for luma 4x4 sub-block, 32 for chroma 4x4 sub-block, and 8 for chroma 2x2 sub-block

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.02% | 0.03% | 110% | 102% |
| Class A2 | -1.02% | -0.56% | -0.65% | 111% | 102% |
| Class B | -0.04% | 0.08% | 0.11% | 111% | 101% |
| Class C | 0.03% | 0.15% | 0.32% | 112% | 100% |
| Class E | 0.13% | 0.03% | 0.07% | 110% | 100% |
| Overall | -0.17% | -0.03% | 0.01% | 111% | 101% |
| Class D | 0.07% | 0.26% | 0.14% | 112% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.10% | 0.20% | 104% | 105% |
| Class A2 | -0.21% | -0.09% | -0.19% | 104% | 104% |
| Class B | -0.09% | -0.10% | 0.08% | 104% | 100% |
| Class C | 0.07% | 0.25% | 0.17% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.03% | 0.07% | 104% | 102% |
| Class D | 0.10% | -0.13% | 0.46% | 106% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -0.32% | -0.48% | 104% | 100% |
| Class C | -0.10% | -0.18% | -0.35% | 104% | 100% |
| Class E | -0.24% | 0.03% | -0.36% | 103% | 103% |
| Overall | -0.20% | -0.19% | -0.41% | 104% | 101% |
| Class D | -0.11% | 0.87% | -0.30% | 105% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 7.  Results of Test-E: Test-D+ include gt2 flag into the first coding pass

AI, RA, and LB data will be updated

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.47% | 0.40% | 43% | 79% |
| Class A2 | -1.15% | 0.00% | -0.32% | 17% | 56% |
| Class B | -0.11% | -0.01% | 0.04% | 108% | 100% |
| Class C | -0.05% | 0.01% | 0.20% | 109% | 99% |
| Class E | 0.06% | -0.07% | 0.10% | 108% | 99% |
| Overall | -0.30% | -0.09% | 0.08% | 68% | 87% |
| Class D | -0.02% | 0.15% | -0.01% | 109% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -1.08% | -0.76% | 4% | 31% |
| Class A2 | 0.13% | -1.33% | -1.61% | 4% | 29% |
| Class B | -0.16% | -0.13% | 0.12% | 103% | 101% |
| Class C | 0.02% | 0.22% | 0.22% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.47% | -0.37% | 29% | 62% |
| Class D | 0.05% | -0.07% | 0.04% | 104% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class C | -0.13% | 0.07% | -0.28% | 104% | 99% |
| Class E | -0.21% | 0.43% | -1.00% | 102% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 2% | 28% |
| Class D | -0.08% | -0.05% | -0.15% | 104% | 99% |
| Class F (optional) |  |  |  |  |  |

# Conclusions

In this contribution, it is proposed to constrain the maximum number of context-coded bins to 32 for luma 4x4 sub-block, 16 for chroma 4x4 sub-block, and 4 for chroma 2x2 sub-block. The constraint values can be increased based the last significant sub-block position. To improve parsing throughput and reduce the number of coding passes, the gt2 flags are moved to the first coding pass after the parity bit. The simulation results reportedly show that, the average luma BD-rates are -0.18%, -0.08%, and -0.xx% in AI, RA, and LB cases respectively, and the average chroma BD-rates are 0.05%, 0.18%, and 0.xx% in AI, RA, and LB cases respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, August 2018.