| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0149 |
| --- | --- |

| Title: | CE6-related: Complexity reduction method based on skipping high frequency coefficients for inter MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the current VTM, coefficient region other than top-left 32x32 is skipped (zeroed-out) for inter predicted residual when inter MTS is turned on.

In this contribution, the following three restrictions (RMTS_1, RMTS_2, and RMTS_3) based on the skipping with inter MTS on are proposed, all of which are stronger than the current one:

- RMTS_1

- If height ≥ width && height ≥ 16, then (width, height/2) region is kept.

- If width > height && width ≥ 16, then (width/2, height) region is kept.

- RMTS_2: If width (height) ≥ 32, then left (top) half of coefficients are kept.

- RMTS_3: If width (height) ≥ 16, then left (top) half of coefficients are kept.

The experimental results of the above three methods are summarized as follows:

- RMTS_1: 96%/100% (RA), and 95%/99% (LD) encoding/decoding time compared to VTM anchor, BD-rate degradation is 0.06% (RA), and 0.12% (LD).

- RMTS_2: 96%/98% (RA), and 96%/98% (LD) encoding/decoding time compared to VTM anchor, BD-rate degradation is 0.03% (RA), and 0.03% (LD).

- RMTS_3: 94%/99% (RA), and 92%/97% (LD) encoding/decoding time compared to VTM anchor, BD-rate degradation is 0.08% (RA), and 0.18% (LD).

Proposed Method

In this contribution, the proposed methods already introduced in the Abstract section are only applied to inter predicted TU. In [1], RMTS_2 is applied to both intra and inter predicted TU. According to the associated result presented in [1], RMTS_2 causes little performance impact in RA and LD conditions, which can be a strong motivation to apply further skipping such as RMTS_1 or RMTS_3. In this contribution, basically, it is assumed that inter MTS is turned on for all of anchor and tests. For the region where must be zero, residual coding is skipped in such a way that the associated subblock flags are implicitly derived to be 0, and truncated unary binarization of last coefficient position is adjusted considering maximum feasible position.

Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration with inter MTS ON according to common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1026 [3].

2.1) RMTS_1

Anchor is BMS 2.0.1 with VTM configuration with inter MTS on:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.20% | 0.07% | 96% | 100% |
| Class A2 | 0.02% | 0.11% | -0.05% | 97% | 99% |
| Class B | 0.08% | -0.07% | -0.03% | 96% | 100% |
| Class C | 0.08% | -0.07% | -0.03% | 96% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.06% | -0.01% | 96% | 100% |
| Class D | 0.06% | -0.01% | 0.05% | 96% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | -0.05% | -0.23% | 95% | 99% |
| Class C | 0.24% | 0.10% | 0.09% | 94% | 98% |
| Class E | -0.02% | 0.17% | -0.05% | 97% | 100% |
| Overall | 0.12% | 0.06% | -0.08% | 95% | 99% |
| Class D | 0.23% | 0.35% | -0.08% | 95% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.2) RMTS_2

Anchor is BMS 2.0.1 with VTM configuration with inter MTS on:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.14% | 0.08% | 96% | 99% |
| Class A2 | 0.03% | 0.11% | -0.07% | 97% | 99% |
| Class B | 0.03% | -0.05% | -0.09% | 96% | 98% |
| Class C | 0.03% | -0.06% | -0.05% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.04% | -0.04% | 96% | 98% |
| Class D | 0.00% | 0.13% | 0.12% | 97% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.08% | 0.27% | 95% | 98% |
| Class C | 0.08% | 0.11% | 0.06% | 95% | 97% |
| Class E | -0.09% | -0.08% | 0.01% | 96% | 98% |
| Overall | 0.03% | -0.02% | 0.13% | 96% | 98% |
| Class D | 0.02% | 0.09% | 0.40% | 96% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) RMTS_3

Anchor is BMS 2.0.1 with VTM configuration with inter MTS on:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.11% | 0.03% | 95% | 99% |
| Class A2 | 0.07% | 0.03% | -0.08% | 95% | 99% |
| Class B | 0.10% | -0.09% | -0.07% | 94% | 99% |
| Class C | 0.10% | -0.06% | -0.04% | 94% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.06% | -0.05% | 94% | 99% |
| Class D | 0.13% | -0.04% | -0.10% | 94% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | -0.06% | -0.12% | 92% | 96% |
| Class C | 0.30% | 0.21% | 0.10% | 91% | 96% |
| Class E | 0.01% | -0.17% | -0.22% | 94% | 99% |
| Overall | 0.18% | 0.00% | -0.07% | 92% | 97% |
| Class D | 0.26% | 0.19% | 0.09% | 92% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

Consequently, RMTS_2 showed little BD-rate degradation as expected. RMTS_1 led slight performance drop but seems to be a reasonable trade-off considering complexity reduction from it. Basically, it is expected that both computational complexity and memory usage can be reduced according to how severe the skipping restriction would be. It is recommended that one of our proposed methods should be incorporated into the next VTM or be studied in next CE.

Reference

- M. Koo et al., “CE 6-1.1 (c,d): Fast DST-7/DCT-8 based on DFT and 32 point MTS based on skipping high frequency coefficients”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0132, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).