| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0153 |
| --- | --- |

| Title: | CE6-related: NSST modification for wide angle intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In 11th JVET meeting, Wide Angle Intra Prediction (WAIP) was adopted, where intra prediction direction is change from upward-right to downward-left or vice versa according to block shape. When this change occurs, NSST mapping and input residual arrangement need to be adjusted to reflect actual residual statistics based on the WAIP modes. In this contribution, NSST mapping for extended wide angle modes is added and input residual data is transposed according to WAIP mode (not original prediction mode). With this modification, 0.03% (AI), 0.01% (RA), and -0.01% (LD) BD-rate improvements were observed for BMS 4x4 NSST under VTM configuration. For two RST (Reduced Secondary Transform)  configurations proposed in [1], 0.08%/0.07% (AI), 0.05%/0.05% (RA), and 0.06%/0.06% (LD) BD-rate improvements were obtained, respectively.

Proposed Method

In WAIP, 10 upward-right directions are added, whose mode indices are in the range of [-10, -1]. Also, 10 downward-left directions which are symmetrical with the 10 upward-right ones are added, whose mode indices are in the range of [67, 76].

In NSST, transform sets for intra prediction modes of which indices are ranging from 0 to 34 are prepared and other modes share their transform sets with symmetrically directed modes (specifically, symmetric to mode 34 direction). Therefore, for intra prediction modes ranging from 35 to 66, a 2D residual block is transposed before applying its symmetrically shared transform set.

If prediction direction is changed to the opposite one due to the WAIP, it should be followed accordingly whether or not the transpose operation should be performed. But, currently the transpose is being fulfilled according to original intra prediction mode whose index is parsed from intra prediction mode coding. Therefore, in this contribution, WAIP direction determines whether or not the transpose should be performed.

Moreover, currently NSST mapping based on the original intra prediction mode is still used. Therefore, for 20 extended WAIP modes, inappropriate NSST could be applied not exactly reflecting residual statistics for the WAIP modes. But NSST sets specific for the extended modes, say, [-10, -1] are not provided for current BMS, so that the same transform set assigned for mode 2 is mapped to the modes [-10, -1]. This mapping is also applied to the other extended modes, [67, 76], along with transpose operation. The resultant NSST set mapping with 35 transform sets is as follows:

If we use RST with reduced number of transform sets proposed in [1], the associated map could be as the following:

Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration according to common test conditions defined in JVET-K1010 [2] and Core Experiment description in JVET-K1026 [3].

2.1) BMS NSST

Anchor is BTM 2.0.1 with VTM configuration:

Comparing 2.1.2) with 2.1.1), BD-rate increases by 0.03%, 0.01%, and -0.01% on AI, RA, and LD conditions, respectively, due to the proposed modifications.

2.1.1) With the modifications for WAIP OFF

BMS 2.0.1 with BMS 4x4 NSST on, under VTM configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.67% | -2.25% | -1.99% | 290% | 98% |
| Class A2 | -0.63% | -2.29% | -1.71% | 289% | 100% |
| Class B | -0.76% | -2.03% | -3.10% | 304% | 99% |
| Class C | -1.23% | -2.18% | -2.95% | 295% | 101% |
| Class E | -1.37% | -2.53% | -4.17% | 300% | 100% |
| Overall | -0.93% | -2.23% | -2.83% | 296% | 100% |
| Class D | -1.30% | -2.09% | -2.87% | 301% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -1.74% | -2.02% | 155% | 101% |
| Class A2 | -0.37% | -1.83% | -1.43% | 144% | 99% |
| Class B | -0.39% | -2.40% | -3.22% | 154% | 100% |
| Class C | -0.53% | -1.61% | -2.12% | 157% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -1.94% | -2.33% | 153% | 99% |
| Class D | -0.50% | -1.97% | -2.12% | 154% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.86% | -1.26% | 130% | 100% |
| Class C | -0.13% | -0.40% | -0.89% | 131% | 99% |
| Class E | -0.39% | -1.54% | -2.40% | 109% | 100% |
| Overall | -0.18% | -0.88% | -1.42% | 125% | 100% |
| Class D | -0.09% | 0.10% | -0.73% | 123% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.1.1)  With the modifications for WAIP ON

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -2.27% | -1.98% | 289% | 98% |
| Class A2 | -0.65% | -2.35% | -1.80% | 291% | 100% |
| Class B | -0.81% | -2.06% | -3.15% | 303% | 100% |
| Class C | -1.25% | -2.19% | -2.98% | 294% | 102% |
| Class E | -1.40% | -2.61% | -4.28% | 299% | 100% |
| Overall | -0.96% | -2.26% | -2.88% | 296% | 100% |
| Class D | -1.34% | -2.14% | -2.98% | 301% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -1.86% | -2.20% | 155% | 100% |
| Class A2 | -0.39% | -1.79% | -1.51% | 144% | 99% |
| Class B | -0.38% | -2.33% | -3.26% | 153% | 100% |
| Class C | -0.53% | -1.54% | -1.99% | 157% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -1.92% | -2.36% | 153% | 99% |
| Class D | -0.53% | -1.45% | -1.87% | 154% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.94% | -1.14% | 130% | 100% |
| Class C | -0.11% | -0.65% | -0.98% | 130% | 99% |
| Class E | -0.38% | -1.49% | -2.83% | 109% | 100% |
| Overall | -0.16% | -0.98% | -1.51% | 125% | 100% |
| Class D | -0.03% | -0.77% | -0.65% | 123% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.2) RST configuration of CE 6-2.1a

Anchor is BMS 2.0.1 with VTM configuration:

Comparing 2.2.2) with 2.2.1), BD-rate increases by 0.08%, 0.05%, and 0.06% on AI, RA, and LD conditions, respectively, due to the proposed modifications.

2.2.1)  With the modifications for WAIP OFF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.36% | -2.15% | -2.03% | 95% | 94% |
| Class A2 | -0.54% | -1.77% | -1.12% | 99% | 97% |
| Class B | -0.69% | -1.82% | -2.46% | 95% | 97% |
| Class C | -1.04% | -1.88% | -2.27% | 93% | 100% |
| Class E | -1.24% | -1.92% | -2.96% | 95% | 98% |
| Overall | -0.95% | -1.90% | -2.20% | 95% | 97% |
| Class D | -0.72% | -1.59% | -2.12% | 93% | 97% |
| Class F (optional) | -0.99% | -1.55% | -1.66% | 94% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -1.45% | -1.81% | 103% | 99% |
| Class A2 | -0.43% | -1.29% | -1.06% | 100% | 99% |
| Class B | -0.44% | -1.64% | -2.20% | 100% | 101% |
| Class C | -0.41% | -0.83% | -1.27% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -1.31% | -1.65% | 101% | 100% |
| Class D | -0.28% | -0.95% | -1.45% | 100% | 93% |
| Class F (optional) | -0.35% | -0.98% | -1.15% | 99% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.65% | -1.10% | 101% | 97% |
| Class C | -0.22% | -0.54% | -0.81% | 102% | 101% |
| Class E | -0.31% | -1.11% | -1.67% | 102% | 103% |
| Overall | -0.22% | -0.73% | -1.15% | 101% | 100% |
| Class D | -0.16% | 0.53% | -0.30% | 101% | 99% |
| Class F (optional) | -0.26% | -0.32% | -0.70% | 100% | 99% |

2.2.2)  With the modifications for WAIP ON

The same results as in [1]

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.46% | -2.15% | -2.09% | 94% | 92% |
| Class A2 | -0.62% | -1.88% | -1.24% | 98% | 96% |
| Class B | -0.77% | -1.87% | -2.62% | 94% | 95% |
| Class C | -1.08% | -1.98% | -2.38% | 91% | 99% |
| Class E | -1.33% | -2.08% | -3.17% | 95% | 96% |
| Overall | -1.02% | -1.98% | -2.34% | 94% | 96% |
| Class D | -0.79% | -1.58% | -2.16% | 92% | 126% |
| Class F (optional) | -1.04% | -1.61% | -1.88% | 94% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.43% | -1.83% | 103% | 100% |
| Class A2 | -0.50% | -1.30% | -1.12% | 100% | 99% |
| Class B | -0.48% | -1.50% | -2.34% | 100% | 100% |
| Class C | -0.46% | -1.19% | -1.36% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -1.37% | -1.73% | 101% | 99% |
| Class D | -0.32% | -1.10% | -1.47% | 99% | 92% |
| Class F (optional) | -0.39% | -0.95% | -1.35% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.72% | -1.04% | 101% | 99% |
| Class C | -0.19% | -0.87% | -0.99% | 100% | 98% |
| Class E | -0.53% | -0.99% | -1.47% | 100% | 100% |
| Overall | -0.28% | -0.84% | -1.13% | 100% | 99% |
| Class D | -0.12% | 0.10% | -0.10% | 100% | 96% |
| Class F (optional) | -0.32% | -0.29% | -1.08% | 99% | 98% |

2.3) RST configuration of CE 6-2.1b

Anchor is BTM 2.0.1 with VTM configuration:

Comparing 2.3.2) with 2.3.1), BD-rate increases by 0.07%, 0.05%, and 0.06% on AI, RA, and LD conditions, respectively, due to the proposed modifications.

2.3.1)  With the modifications for WAIP OFF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.60% | -2.03% | -1.94% | 124% | 95% |
| Class A2 | -0.79% | -1.56% | -1.00% | 127% | 98% |
| Class B | -0.82% | -1.61% | -2.29% | 126% | 98% |
| Class C | -1.06% | -1.63% | -2.07% | 121% | 99% |
| Class E | -1.41% | -1.95% | -2.74% | 122% | 98% |
| Overall | -1.10% | -1.73% | -2.04% | 124% | 98% |
| Class D | -0.84% | -1.46% | -1.91% | 122% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.47% | -1.77% | 111% | 100% |
| Class A2 | -0.48% | -1.03% | -0.97% | 107% | 99% |
| Class B | -0.46% | -1.51% | -2.24% | 108% | 100% |
| Class C | -0.43% | -0.86% | -1.18% | 109% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -1.23% | -1.61% | 109% | 100% |
| Class D | -0.30% | -0.95% | -1.27% | 108% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.75% | -0.72% | 106% | 100% |
| Class C | -0.16% | -0.70% | -0.82% | 105% | 100% |
| Class E | -0.44% | -1.46% | -0.86% | 102% | 101% |
| Overall | -0.23% | -0.91% | -0.79% | 104% | 100% |
| Class D | -0.07% | -0.29% | -0.08% | 104% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3.2)  With the modifications for WAIP ON

The same results as in [1]

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -2.22% | -2.04% | 125% | 95% |
| Class A2 | -0.86% | -1.76% | -1.15% | 127% | 98% |
| Class B | -0.90% | -1.68% | -2.41% | 126% | 98% |
| Class C | -1.10% | -1.67% | -2.20% | 122% | 100% |
| Class E | -1.49% | -2.13% | -3.11% | 123% | 98% |
| Overall | -1.16% | -1.86% | -2.21% | 124% | 98% |
| Class D | -0.87% | -1.41% | -1.88% | 123% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.04% | -1.48% | -1.95% | 112% | 100% |
| Class A2 | -0.54% | -1.38% | -1.08% | 107% | 99% |
| Class B | -0.50% | -1.68% | -2.33% | 109% | 100% |
| Class C | -0.47% | -0.93% | -1.24% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -1.38% | -1.71% | 109% | 100% |
| Class D | -0.36% | -1.24% | -1.23% | 108% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.68% | -1.26% | 106% | 100% |
| Class C | -0.19% | -0.63% | -0.82% | 106% | 100% |
| Class E | -0.55% | -0.76% | -1.81% | 102% | 101% |
| Overall | -0.30% | -0.69% | -1.25% | 105% | 100% |
| Class D | -0.06% | -0.25% | -0.71% | 104% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

With the advent of WAIP, it seems to be natural that NSST which has to be affected by intra prediction should be aligned with WAIP. In this contribution, it is evident that extended NSST mapping for wide angle modes and performing transpose operation in accordance with real (wide angle) direction should be beneficial. It is recommended that the proposed modifications should be incorporated into the next VTM or BMS.

Reference

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).