| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0159 |
| --- | --- |

| Title: | Non-CE8: Block vector predictor for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes block vector predictor (BVP) for Intra block copy (IBC) before zero candidates which are always for invalid for IBC mode. Two default candidates for BVP are introduced as follows, (-2*width, 0) and (0, -2*height). The luma BD-rate results for the proposed method achieves -0.11% and -0.69% bit-saving for CTC and classF under BMS-AI configuration, respectively.

Introduction

In the both AMVP and merge mode, candidate list are constructed using motion vectors of the spatial and temporal neighboring blocks according to defined order. If the number of candidates in the constructed list is smaller than the maximum number of candidates for AMVP and merge mode, zero candidates are added at the candidate list until the number of candidates reach maximum number. However, block vector of IBC mode using current picture referencing must be larger than width and height of the current coding block. Therefore, zero candidates are always invalid for IBC mode.

Proposed method

This contribution proposes block vector predictor (BVP) for IBC mode. Two default candidates, (-2*width, 0) and (0, -2*height), for the BVP are proposed. It can be added in front of zero candidates for both AMVP and merge mode.

For AMVP mode, when reference picture of current coding block indicated by reference index is same as current picture and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are inserted at MVP candidate list.

For Merge mode, when the current picture exists in the reference picture list and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are added at merge candidate list.

If one candidate for AMVP and merge candidate list is allowed, the first one of default candidates is only added. The maximum number of candidates for AMVP and merge mode are unchanged and there is no syntax changes introduced by the proposed method.

Experimental results

The proposed method has been implemented on top of the BMS2.1. The experiments are tested under common test conditions in [1]. Only CPR tool with the VTM-2.0.1 configuration is switched on.

Table 1 Experimental results of the proposed BV predictor on BMS-2.1 with CPR on

Conclusion

We observed that the proposed method is a solution for useless zero candidates when current picture referencing is used. We recommend that the proposed block vector predictor for IBC mode be studied in CE.

Reference

- J. Boyce, et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).