| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 08–12 Oct. 2018 | Document: JVET-L0159r1 |
| --- | --- |

| Title: | Non-CE8: Block vector predictor for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In merge and AMVP modes, candidate list is first filled from spatial and temporal candidates and zero candidates are then inserted to candidate list until candidate list includes maximum candidates. However, in current picture referencing, block vector for intra block copy cannot become zero. Therefore this contribution proposes, instead of zero candidates, two alternative candidates (-2*width, 0) and (0, -2*height) for block vector prediction candidate for intra block copy (IBC) are introduced. Experimental results reportedly show 0.11%, 0.61%, and 1.18% BD-rate savings for CTC, classF, and SCC over VTM-2.0.1 with CPR under AI configuration.

Introduction

Both in AMVP and merge mode, candidate list is constructed using motion vectors of the spatial and temporal neighboring blocks according to predefined order. If the number of candidates in the constructed list is smaller than the maximum number of candidates for AMVP and merge mode, then zero candidates are added until the number of candidates reach the maximum candidate number. However, block vector of IBC mode using current picture referencing must be larger than width and height of the current coding block. Therefore, zero candidates are always become invalid for IBC mode.

Proposed method

This contribution proposes block vector predictor (BVP) for IBC mode. Two default candidates, (-2*width, 0) and (0, -2*height), for the BVP are proposed. It is added in front of zero candidates for both AMVP and merge mode.

For AMVP mode, when reference picture of current coding block indicated by reference index is same as current picture and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are inserted into MVP candidate list.

For Merge mode, when the current picture exists in the reference picture list and the number of candidates in the constructed list is smaller than the maximum number of candidates, default candidates are added int merge candidate list.

If only one candidate for AMVP and merge candidate list is allowed, the first one of default candidates is added. The maximum number of candidates for AMVP and merge mode are unchanged and there is no syntax changes introduced by the proposed method also.

Experimental results

The proposed method has been implemented on top of the BMS2.1 and tested under common test conditions in [1]. Only CPR tool with the VTM-2.0.1 configuration is switched on.

Table 1 Experimental results of the proposed BV predictor on BMS-2.1 with CPR on

|  | All Intra Main10 |  |  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1+CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.10% | 152% | 101% | -0.03% | -0.01% | -0.03% | 105% | 100% |
| Class A2 | -0.40% | -0.17% | -0.29% | 148% | 101% | -0.08% | -0.03% | -0.03% | 105% | 100% |
| Class B | -0.63% | -0.23% | -0.21% | 159% | 104% | -0.12% | -0.03% | -0.13% | 106% | 101% |
| Class C | -0.32% | -0.28% | -0.07% | 156% | 108% | -0.13% | -0.09% | -0.01% | 106% | 100% |
| Class E | -1.05% | -0.68% | -0.66% | 162% | 103% | -0.20% | -0.14% | -0.05% | 106% | 100% |
| Overall | -0.49% | -0.26% | -0.21% | 156% | 104% | -0.11% | -0.06% | -0.06% | 105% | 100% |
| Class D | -0.21% | 0.13% | 0.32% | 148% | 108% | -0.12% | -0.03% | -0.01% | 106% | 100% |
| Class F | -18.03% | -17.84% | -17.80% | 189% | 98% | -0.61% | -0.29% | -0.29% | 103% | 99% |
| SCC | -53.35% | -52.37% | -52.70% | 149% | 85% | -1.18% | -0.87% | -0.88% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1+CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.06% | 103% | 103% | -0.03% | -0.06% | -0.16% | 101% | 100% |
| Class A2 | -0.24% | -0.06% | -0.16% | 105% | 104% | -0.11% | -0.13% | -0.17% | 101% | 100% |
| Class B | -0.30% | -0.37% | -0.14% | 108% | 102% | -0.12% | -0.17% | 0.02% | 102% | 100% |
| Class C | -0.09% | -0.20% | -0.06% | 116% | 108% | -0.16% | -0.01% | 0.01% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.19% | -0.19% | -0.11% | 108% | 104% | -0.11% | -0.10% | -0.06% | 102% | 100% |
| Class D | -0.04% | -0.08% | 0.41% | 117% | 114% | -0.14% | -0.01% | -0.17% | 102% | 100% |
| Class F | -14.80% | -14.73% | -14.72% | 122% | 105% | -0.49% | -0.49% | -0.47% | 101% | 100% |
| SCC | -36.07% | -36.03% | -36.37% | 107% | 97% | -0.79% | -0.74% | -0.63% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1+CPR |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | -0.03% | -0.30% | 114% | 104% | 0.02% | -0.07% | -0.47% | 101% | 97% |
| Class C | 0.12% | 0.46% | -0.14% | 121% | 111% | 0.05% | 0.38% | -0.08% | 102% | 100% |
| Class E | 0.25% | 0.92% | 0.32% | 112% | 106% | 0.29% | 0.19% | 0.87% | 102% | 100% |
| Overall | 0.13% | 0.37% | -0.09% | 116% | 107% | 0.10% | 0.15% | 0.00% | 101% | 99% |
| Class D | 0.13% | 0.18% | 0.49% | 125% | 117% | -0.06% | -0.11% | -0.08% | 102% | 100% |
| Class F | -8.61% | -8.47% | -9.25% | 118% | 108% | -0.23% | -0.08% | 0.19% | 101% | 99% |
| SCC | -25.58% | -26.76% | -26.77% | 113% | 103% | -0.26% | -0.42% | -0.36% | 101% | 99% |

Conclusion

In this contribution, two default candidates are proposed as a solution for invalid zero candidates insertion when current picture referencing is used. It is recommended to study the proposed block vector predictor for IBC mode in CE.

Reference

- J. Boyce, et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).