Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 77 | INTRA_CCLM |

NOTE – : The intra prediction mode INTRA_CCLM is only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb+ cbHeight - 1  ) and ( xCb+ cbWidth - 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- candModeList[ 0 ] = candIntraPredModeA                                                                                               (8-1)

candModeList[ 1 ] = ! candIntraPredModeA                                                                                            (8-2)

candModeList[ 2 ] = INTRA_ANGULAR50                                                                                            (8-3)

candModeList[ 3 ] = INTRA_ANGULAR18                                                                                            (8-4)

candModeList[ 4 ] = INTRA_ANGULAR46                                                                                            (8-5)

candModeList[ 5 ]= INTRA_ANGULAR54                                                                                             (8-6)

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is larger than 1, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-7)

candModeList[ 1 ] = INTRA_PLANAR                                                                                       (8-8)

candModeList[ 2 ] = INTRA_DC	                                                                                                 (8-9)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 62 ) % 65 )                                               (8-10)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA - 1 ) % 65 )                                                  (8-11)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 61 ) % 65 )                                               (8-12)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	                                                                                   (8-13)

candModeList[ 1 ] = candIntraPredModeB	                                                                                   (8-14)

  - Set the variable biggerIdx is as follows

biggerIdx = candModeList[ 0 ] >  candModeList[ 1 ] ? 0 : 1                                                            (8-15)

  - If both of candIntraPredModeA and candIntraPredModeB are larger than 1, candModeList[ x ] with x = 2..5 is derived as follows:

candModeList[ 2 ] = INTRA_PLANAR                                                                                           (8-16)

candModeList[ 3 ] = INTRA_DC                                               	                                        (8-17)

- If candModeList[biggerIdx] - candModeList[ !biggerIdx] is equal to neither 64 nor 1, the following applies:

candModeList[ 4 ] = 2 + ( ( candModeList[biggerIdx] + 62 ) % 65 )                                    (8-18)

candModeList[ 5 ] = 2 + ( ( candModeList[biggerIdx] - 1 ) % 65 )                                       (8-19)

- Otherwise, the following applies:

candModeList[ 4 ] =2 + ( ( candModeList[biggerIdx] + 61 ) % 65 )                                     (8-20)

candModeList[ 5 ] =2 + (   candModeList[biggerIdx]  % 65 )                                               (8-21)

  - Otherwise, if sum of candIntraPredModeA and candIntraPredModeB is larger or equal to 2, the following applies:

candModeList[ 2 ] = ! candModeList[ !biggerIdx]                                                                           (8-22)

candModeList[ 3 ] = 2 + ( (candModeList[biggerIdx] + 62 ) % 65 )                                               (8-23)

candModeList[ 4 ] = 2 + ( (candModeList[biggerIdx] - 1 ) % 65 )                                                   (8-24)

candModeList[ 5 ] = 2 + ( (candModeList[biggerIdx] + 61 ) % 65 )                                               (8-25)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..5 is modified by the following ordered steps:

  - For i sequentially equals to 0 to 4, inclusive, apply:

  - For j sequentially equals to i + 1 to 5, inclusive, compare candModeList[ i ] to candModeList[ j ], When candModeList[ i ] is greater than candModeList[ j ], both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ])     (8-26)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

