| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0165-v1 |
| --- | --- |

| Title: | CE3-6.2.1: Extended MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Seunghwan Kim Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com jin78.heo@lge.com jangwon84.choi@lge.com jungah.choi@lge.com sunmi.yoo@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes an extended 6 MPM list with similar complexity compared to 3 MPM in VTM 2.0.1. In the proposed method, two neighboring intra modes (left and above) are deployed for MPM list generation. Following the structure of the 3 MPM generation in VTM2.0.1, MPM lists are generated. The following three tests are performed in this investigation.

- CE3-6.2.1a: CE3-6.2.1 with CTU boundary restriction

- CE3-6.2.1b: CE3-6.2.1 without CTU boundary restriction

The experiment results reportedly show -0.29%, -0.11% and -0.33% and -0.13% bit rate changes for VTM-AI, VTM-RA for CE3-6.2.1a, and VTM-AI and VTM-RA for CE3-6.2.1b, respectively.

Proposed Method

At the last meeting, HEVC style 3 MPM is adopted in VVC. The rationale behind 3 MPM is simplicity and throughput advantage. 6 MPM existing in JEM is too complex: including exhaust neighbor position search and pruning process, three steps for list generation, line buffer requirement and parsing dependency. Hence following philosophy of 3 MPM, an extended 6 MPM list method is proposed.

In the proposed method, as shown in Figure 1, only intra mode of neighbor position A and B denoted as LEFT and ABOVE are used for MPM list generation. Pseudo code is as below. For the non MPM coding, truncated binary coded is applied.

Table 1. Pseudo code for proposed method

Determine LEFT and ABOVE intra modes

Set MPM as MPM_ordering_0

If (LEFT==ABOVE)

If (LEFT>=DC_idx), then set MPM as MPM_ordering_1

Else if (LEFT>DC_idx and ABOVE>DC_idx), then set MPM as MPM_ordering_2

Else if (LEFT+ABOVE> DC_idx), then set MPM as MPM_ordering_3

Figure 1. Neighbor blocks of current block

Complexity Analysis

|  | Max number of neighbors to access | Line buffer required? | Max layer of if | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increment | Max number of bit operation | Parsing dependency? | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3MPM | 2 | N | 3 | 2 | 4 | 15 | 0 | 0 | N | 1 | 1 or 2 | N | - |
| CE3-6.2.1a | 2 | N | 3 | 6 | 8 | 26 | 0 | 0 | N | 1 | 1 or 2 | N | - |

Draft Specification Text

Changes compared to JVET-K1001-v7 are highlighted.

- Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 77 | INTRA_CCLM |

NOTE – : The intra prediction mode INTRA_CCLM is only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb+ cbHeight - 1  ) and ( xCb+ cbWidth - 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- candModeList[ 0 ] = candIntraPredModeA                                           (8-1)

candModeList[ 1 ] = ! candIntraPredModeA                                          (8-2)

candModeList[ 2 ] = INTRA_ANGULAR50                                          (8-3)

candModeList[ 3 ] = INTRA_ANGULAR18                                          (8-4)

candModeList[ 4 ] = INTRA_ANGULAR46                                          (8-5)

candModeList[ 5 ] = INTRA_ANGULAR54                                          (8-6)

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is larger than 1, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA                                      (8-7)

candModeList[ 1 ] = INTRA_PLANAR                                        (8-8)

candModeList[ 2 ] = INTRA_DC	                                             (8-9)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 62 ) % 65 )                    (8-10)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA - 1 ) % 65 )                     (8-11)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 61 ) % 65 )                    (8-12)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	                                                                                   (8-13)

candModeList[ 1 ] = candIntraPredModeB	                                                                                   (8-14)

  - Set the variable biggerIdx is as follows

biggerIdx = candModeList[ 0 ] > candModeList[ 1 ] ? 0 : 1                          (8-15)

  - If both of candIntraPredModeA and candIntraPredModeB are larger than 1, candModeList[ x ] with x = 2..5 is derived as follows:

candModeList[ 2 ] = INTRA_PLANAR                                         (8-16)

candModeList[ 3 ] = INTRA_DC                 	                       (8-17)

- If candModeList[biggerIdx] - candModeList[ !biggerIdx] is equal to neither 64 nor 1, the following applies:

candModeList[ 4 ] = 2 + ( ( candModeList[biggerIdx] + 62 ) % 65 )              (8-18)

candModeList[ 5 ] = 2 + ( ( candModeList[biggerIdx] – 1 ) % 65 )               (8-19)

- Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( candModeList[biggerIdx] + 61 ) % 65 )              (8-20)

candModeList[ 5 ] = 2 + ( candModeList[biggerIdx] % 65 )                    (8-21)

  - Otherwise, if sum of candIntraPredModeA and candIntraPredModeB is larger or equal to 2, the following applies:

candModeList[ 2 ] = ! candModeList[ ! biggerIdx]                                 (8-22)

candModeList[ 3 ] = 2 + ( ( candModeList[biggerIdx] + 62 ) % 65 )                   (8-23)

candModeList[ 4 ] = 2 + ( ( candModeList[biggerIdx] – 1 ) % 65 )                    (8-24)

candModeList[ 5 ] = 2 + ( ( candModeList[biggerIdx] + 61 ) % 65 )                   (8-25)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..5 is modified by the following ordered steps:

  - For i sequentially equals to 0 to 4, inclusive, apply:

  - For j sequentially equals to i + 1 to 5, inclusive, compare candModeList[ i ] to candModeList[ j ], When candModeList[ i ] is greater than candModeList[ j ], both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ])     (8-26)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

Experimental Results

The proposed method is implemented on top of BMS 2.0.1, and evaluated for VTM configuration according to the common test conditions [2] and Core Experiment [3].

Table 1 Experiment result for CE3-6.2.1a all intra (AI) using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.16% | -0.17% | 99% | 100% |
| Class A2 | -0.25% | -0.24% | -0.25% | 100% | 100% |
| Class B | -0.28% | -0.19% | -0.22% | 100% | 100% |
| Class C | -0.30% | -0.26% | -0.08% | 100% | 100% |
| Class E | -0.40% | -0.38% | -0.35% | 99% | 99% |
| Overall | -0.29% | -0.24% | -0.21% | 100% | 100% |
| Class D | -0.19% | 0.10% | -0.37% | 100% | 100% |

Table 2 Experiment result for CE3-6.2.1a random access (RA) using BMS 2.0.1 with VTM config as anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.03% | -0.06% | 100% | 100% |
| Class A2 | -0.12% | 0.00% | -0.07% | 100% | 100% |
| Class B | -0.09% | -0.11% | 0.14% | 100% | 100% |
| Class C | -0.10% | -0.08% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.05% | 0.01% | 100% | 100% |
| Class D | -0.08% | -0.31% | 0.38% | 100% | 100% |

Table 3 Experiment result for CE3-6.2.1b all intra (AI) using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.22% | -0.20% | 100% | 99% |
| Class A2 | -0.29% | -0.30% | -0.33% | 99% | 99% |
| Class B | -0.31% | -0.25% | -0.29% | 99% | 100% |
| Class C | -0.34% | -0.35% | -0.20% | 100% | 101% |
| Class E | -0.49% | -0.49% | -0.55% | 99% | 99% |
| Overall | -0.33% | -0.32% | -0.30% | 100% | 100% |
| Class D | -0.19% | 0.03% | -0.39% | 100% | 101% |

Table 4 Experiment result for CE3-6.2.1b random access (RA) using BMS 2.0.1 with VTM config as anchor

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS 2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | -0.05% | 100% | 100% |
| Class A2 | -0.14% | -0.18% | -0.15% | 100% | 100% |
| Class B | -0.12% | -0.15% | 0.10% | 100% | 99% |
| Class C | -0.11% | 0.03% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.09% | 0.00% | 100% | 100% |
| Class D | -0.09% | -0.45% | 0.35% | 100% | 99% |

Summary

The proposed extended MPM list has 0.29% bit rate reduction over VTM-2.0.1, but as simple as existing 3 MPM.

References

- BMS-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, Document of Joint Video Experts Team (JVET), JVET-K1023, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).