| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0166_v1 |
| --- | --- |

| Title: | CE13-related: Subjective Quality Improvement for RSP |
| --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anubhav Singh Akhil Konda Chirag Pujara | Tel: Email: | Anubhav.23@samsung.com akhil.konda@samsung.com chirag.p@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution compares the effects of padding and blending and post filtering. The simulations are run on RSP format. Subjective quality is estimated using the viewports extracted according 360 CTC document. Seam artifact in the viewports are observed to be reduced by both: a) padding and blending with 8 pixels and b) post filtering at the discontinuous boundaries of RSP. The effect of padding and blending is observed to be more.

# Introduction

Projection formats (RSP, HEC, etc) have discontinuous edges where the content from non-neighboring locations on the sphere arranged together. Application of in-loop filters at these locations, with discontinuous neighbours, result in poor quality of viewports. To solve this problem, two major approaches have been experimented which do not require any normative changes, namely:

- Padding of projection format and blending while converting to viewport/ ERP.

- Post filtering of discontinuous edges.

In this document we have carried out experiments to compare the effects of above two methods on RSP. The subjective quality is compared using the evil viewports mentioned in 360 CTC [3].

# Technical description

## Padding and Blending

In blending, boundary content is updated with weighted average of itself and equivalent content inside padded region. In RSP, circular boundary of the lobes is filled by using spherically neighboring content and those pixels are used for blending. On top of spherical padding, we add some extra padding for aligning boundary/seams with CU. This is represented as step padding in Figure 1.

Figure 1. Representative image of RSP, showing color mapping between RSP content and their counterpart in spherical padding which is used for blending

For each pixel inside the colored region of RSP content, we find its corresponding pixel values in spherical padding region using bilinear interpolation. Denote Pold as pixel intensity value inside RSP content before blending, Ppad as pixel intensity mapped inside spherical padding, Pnew as pixel intensity value inside RSP content after blending, we compute Pnew by the following

| Pnew= Pold*d+ Ppad*(D-d) D | (21) |
| --- | --- |

where d refers to distance of the pixel from boundary while D is number of pixels to be modified after blending within same column. We have observed significant improvement in subjective quality but loss in BD bitrate gain after introducing padding and blending for the discontinuous regions.

## Post Filtering

Figure 2. Representative image of RSP, showing color mapping between RSP content and their counterpart for post filtering

The post filtering of RSP is implemented similar to the one proposed in JVET-K0183 [4]. Figure 2 shows the corresponding locations for post filtering. De-blocking and ALF are switched off at the discontinuous boundaries (post filtering locations).

# Simulation results

The software is implemented based on BMS-2.0.1 and 360Lib-7.0. The encoding was done under 360° video CTC [3]. Only E2E WS PSNR is reported for experimentations, Table 1.

Table 1. BD-rate for RSP with 8 sample padding and blending and post filtering compared to PHEC.

The results reported in Table 1 is with respect to PHEC. Padding and blending causes a loss of 1.14% in Luma for RSP compared to PHEC. Though applying only post filtering reduces the loss to 0.59% for Luma, the subjective quality of the viewports are better in case of padding and blending. Moreover applying post filtering on top of blending doesn’t give any significant gain, Figure 4.

# Observation

Applying post filtering alone, though reduces the loss with respect to PHEC, it is not enough to eliminate the seam artifact. As can be seen in figure 3, the content in RSP improves after applying post filtering.

- Without post filtering                                                            (b) With post filtering

Figure 3. Snapshot taken from RSP image at the face boundary for Harbor at QP 37.

- Only Post filtering              (c) padding and blending    (b) Post filtering and padding and blending

Figure 4. Snapshots taken from Viewports of balboa after encoding with RSP at QP 37.

# Conclusion

This contribution reports a study of the two methods of seam removal, namely, the padding and blending and post filtering for RSP. The objective results show that post filtering is better than padding and blending by ~0.8%, but the subjective analysis shows that only post filtering does not remove the seam artifact completely. A combination of padding and blending with post filtering can help in reducing the seam artifact as well as give a slight (~0.1% for Luma) gain for RSP.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 7)”, JVET-K1004, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

- Xuchang Huangfu ,Yule Sun, Bin Wang, Lu Yu,  “Face boundary filtering for 360° video”, JVET-K0183, July 2018, Ljublijana, SI.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).