| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0167-v2 |
| --- | --- |

| Title: | AHG7: Subjective Quality Evaluation of VVC HDR sequences on UHD TV |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Amith Dsouza Chirag Pujara Raj Narayana Gadde Kiho Choi Kwang Pyo Choi | Tel: Email: | amith.ds@samsung.com chirag.p@samsung.com raj.gadde@samsung.com kiho14.choi@samsung.com kp5.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This information document presents the method to generate HDR/WCG sequences from VVC bit streams, to view HDR/WCG video on commercially available UHD TV’s.

# Introduction

VVC decoders are not yet available on commercial devices. To view HDR VVC sequences on current devices (TVs) the VVC bit streams should be converted to a format that is understandable by commercially available devices.  The Samsung 4K UHD TV [1] can display HDR/WCG H.265 bit-stream if HDR related VUI and SEI messages are properly inserted. More information on VUI and SEI message can be found in [2].

# Ffmpeg software

## Download ffmpeg software and build it

- Download ffmpeg from http://www.ffmpeg.org/download.html

- Compile ffmpeg along with x265 encoder as described in https://trac.ffmpeg.org/wiki/Encode/H.265

Or

- Download static build of ffmpeg from https://johnvansickle.com/ffmpeg/

## Encode a bitstream with below command line

./ffmpeg -f rawvideo -s 1920x1080 -pix_fmt yuv420p10le -i input.yuv -c:v hevc -r 50 -profile:v main10 -pix_fmt yuv420p10le -y -x265-params master-display=G(13250,34500)B(7500,3000)R(34000,16000)WP(15635,16450)L(40000000,47):max-cll=4000,0:colorprim=bt2020:transfer=smpte2084:colormatrix=bt2020nc:lossless=1 output.mp4

# Results

Figures in this section are photos captured during our subjective evaluations with Samsung MU7000. Test content was encoded using ffmpeg with and without HDR related metadata. As result, for HDR videos encoded by libx265 without metadata, TV uses default values and displays wrong colors and brightness as shown on Figure 1).

HDR content was also provided in form of 10 bits 4:2:0 YUV video encoded using x265 with VUI information and Mastering Display Color Volume SEI. With the metadata, TV correctly chooses transfer function and properly interprets colors (as shown on Figure 2).

## Snapshots of videos with and without HDR metadata

## Figure 1 Market sequence without HDR metadata

## Figure 2 Market sequence without HDR metadata

## Figure 3 Balloon sequence without HDR metadata

## Figure 4 Balloon Sequence with HDR metadata

Figure 5 Showgirl sequence without HDR metadata

## Figure 6 Showgirl sequence with HDR metadata

# Conclusions

This contribution outlines method for encoding test sequences with HDR metadata in order to display in commercially available televisions. It is also demonstrated that proper encoding VUI and Mastering Display Color Volume SEI has critical effect on visual quality of HDR video decoded by commercial TV.

# References

[1] http://www.samsung.com/us/video/tvs/UN65JS9500FXZA

[2] JCTVC-U0045 : “High Dynamic Range with HEVC Main10”, Warsaw meeting, June 2015.

# Patent rights declaration(s)

Samsung Electronics does not have any current or pending patent rights relating to the technology described in this contribution.