| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0170 |
| --- | --- |

| Title: | CE11.3.5 Parallel deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

- Abstract

This proposal presents a parallel deblocking filter based on current block size. The proposed method is introduced for QTBT as JVET-D0044[1] in Chengdu meeting. The performance of the proposed parallel deblocking method is evaluated in terms of subjective quality, objective quality, and computational complexity in CE11[2]. The proposed method doesn’t shows any noticeable subjective quality degradation even though it gives less computational complexity in terms of operation. Moreover, experimental results reportedly show no remarkable BD-rate changes in RA, LD-B, and LD-P configurations.

Proposed method

For the luma component, deblocking filter is not applied on block boundary of 4xN block for vertical edge and Nx4 block for horizontal edge.

For the chroma component, deblocking filter is not applied on block boundary of 2xN block for vertical edge and Nx2 block for horizontal edge.

Experimental results

With ALF

Table 1. Random access configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.27% | -0.20% | 100% | 101% |
| Class A2 | 0.00% | 0.21% | 0.23% | 100% | 100% |
| Class B | -0.01% | 0.16% | 0.06% | 100% | 100% |
| Class C | -0.01% | -0.03% | -0.20% | 100% | 99% |
| Overall | -0.01% | 0.14% | -0.03% | 100% | 100% |
| Class D | 0.01% | 0.16% | 0.11% | 100% | 98% |

Table 2. Low delay configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.32% | -0.44% | 101% | 99% |
| Class C | 0.05% | -0.29% | -0.34% | 101% | 103% |
| Class E | -0.09% | 0.20% | -0.08% | 101% | 105% |
| Overall | 0.00% | -0.18% | -0.32% | 101% | 101% |
| Class D | -0.05% | 0.36% | -0.40% | 101% | 98% |

Table 3. Low delay P configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Without ALF

Table 4. Random access configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | -0.39% | 101% | 100% |
| Class A2 | 0.04% | 0.18% | 0.47% | 101% | 100% |
| Class B | 0.00% | 0.32% | 0.20% | 101% | 101% |
| Class C | 0.01% | 0.12% | -0.02% | 101% | 99% |
| Overall | 0.02% | 0.17% | 0.08% | 101% | 100% |
| Class D | 0.03% | 0.32% | 0.21% | 101% | 95% |

Table 5. Low delay configuration

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | 0.01% | -0.38% | 100% | 99% |
| Class C | 0.03% | -0.17% | -0.21% | 101% | 101% |
| Class E | 0.09% | -0.48% | -0.34% | 102% | 104% |
| Overall | 0.03% | -0.17% | -0.31% | 101% | 101% |
| Class D | 0.06% | -0.28% | -0.44% | 101% | 98% |

Table 6. Low delay P configuration.

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Conclusion

In this contribution, block size based parallel deblocking filter was proposed. The proposed method gives less computational complexity in terms of operations without any noticeable subjective quality degradation and objective BD-rate change. It is recommended to consider the proposed method for the next version of VTM.

Reference

- H. Jang, J. Nam, J. Lim, “Parallel deblocking filter for QTBT,” JVET-D0044, Chengdu, CH, October 2016.

- A. Norkin, A. Kotra, “Description of Core Experiment 11 (CE11): Deblocking,” JVET-K1031, Ljubljana, SI, July 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).