| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0174-v2 |
| --- | --- |

| Title: | Non-CE9: Simplifications related to cost function in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal (relates to CE9) |  |  |
| Author(s) or Contact(s): | S. Sethuraman Consulate-1, #1 Richmond Road Bangalore-560025 India | Tel: Email: | +91-80-66601000 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_______`______________________

# Abstract

In this proposal, two simplifications related to the cost function used in DMVR are proposed and results related to individual methods and the combined methods are presented. The first simplification relates to restricting the bit-depth of the interpolated samples on which the cost functions are evaluated during DMVR. The second simplification relates to replacing the mean removed sum of absolute differences (MR-SAD) with a refinement position independent MR-SAD (P-I-MR-SAD) cost function. Results show that the bit-depth restriction has hardly any impact on BDRATE with EncT ratio of xxx% and DecT ratio of yyy% compared to the reference method. The P-I-MR-SAD results in a 0.08% BDRATE drop when compared to MR-SAD, but offers an EncT ratio of xxx% and DecT ratio of yyy%. The two simplifications together allow EncT ratio of xxx% and DecT ratio of yyy% for DMVR Tool-ON over VTM2.

# Introduction

The cost function evaluations constitute a significant portion of DMVR related complexity. Though the normative motion compensation works with unclipped data at a higher bit-depth till the final bi-directional averaging is performed, such a high bit-depth may not be required for cost function evaluations. In hardware designs, when DMVR is performed in a different pipeline stage from normative motion compensation, the interpolated buffer requirements can double. In this proposal, the bit-depth requirements for cost function evaluation are constrained without any impact to the coding gains.

It has been shown in the DMVR adopted in BMS-2.0 that MR-SAD provides consistent coding gains over SAD as a cost function. However, MR-SAD requires evaluation of block means at every refinement position in each of the references. Also, it requires adding the mean-difference to the each of the sample difference before evaluating the absolute value and thus adds to the complexity when compared to a regular SAD that has single cycle instructions on most processors and simpler hardware. In this proposal, a position independent MR-SAD is proposed that eliminates the need for computing the mean at every refinement position and enables the use of regular SAD after an one-time mean adjustment step.

The combination of the constrained bit-depth and P-I-MR-SAD offer significant throughput increase on software platforms and reduced complexity and storage in hardware implementations.

# Proposed algorithms

## Constrained bit-depth for refinement (CBDR)

It has been shown in JVET-K0041 that bilinear interpolated samples cause no major BDRATE impact when compared to DCTIF based samples for refinement. When explicit sub-pixel distance refinement is performed as in the BMS-2.0 adopted DMVR or with JVET-K0041 proposed parametric error surface, the bilinear interpolation reduces the interpolation complexity as compared to performing 2 or more sets of DCTIF, one for every sub-pixel phase. It also offers the ability to work with arbitrary bit-depths, instead of keeping the interpolated samples at the 14-bit unclipped internal bit-depth required for bi-prediction.

In this proposal, we propose to restrict the bilinear interpolated samples to a bit-depth of 8 bits that can be combined with the right-shift performed to correct for the bit-depth at which the sum of the bilinear interpolation filter weights is represented.

This bit-depth allows all subsequent refinement operations such as block mean computation and mean removed SAD computation to be performed using 8-bit samples. In software SIMD, byte-wise SIMD can improve the throughput by a factor of 2x compared to the existing bit-depth of 14 (or the bit-depth that depends on the sample bit-depth). In hardware, this bit-depth restriction can translate to fewer bit-planes and hence a lower gate count.

## Position-independent MR-SAD (P-I-MR-SAD)

MRSAD helps to emphasize the level of alignment between the AC signals in the L0 and L1 patches used for bi-prediction. Instead of computing the block mean at every position of refinement, in this proposal, a single block mean is computed for each reference using a subset of samples used for refinement. Specifically, the block mean is computed on the (CU_width + 2)*(CU_height + 2) samples at the center of the refinement range in L0 and L1. Each interpolated sample in the refinement range of the block with the lower block mean is adjusted with the difference between the block with the higher block mean and the lower block mean. The resulting sample values are clipped to be within the interpolated sample bit-depth that is employed in MRSAD.

During the refinement iterations, one unadjusted buffer of samples and another mean difference adjusted buffer of samples are used along with a regular sum of absolute differences cost function at all the refinement positions.

Hence, the proposed method eliminates the need to compute block means for every refinement position considered in L0 and L1. It also eliminates the need to add a block mean-difference to every sample difference value before computing absolute values for summation of the cost function. Though the block mean can be computed in an incremental manner, the address calculations tend to add a lot of address generation and control circuitry which are now eliminated. The use of regular SAD offers improved throughput on most processors with single cycle SIMD instructions for SAD. It also reduces the hardware requirements. From a hardware timing perspective, the block means can be computed when the bilinear interpolated samples are computed.

## Combination of CBDR and P-I-MRSAD

In this proposal, the bit-depth restriction of the bilinear interpolated samples is combined with the position independent MR-SAD cost function. The advantage is that the block mean calculation and the mean difference adjustment can all be performed with 8-bit samples. Also, the use of regular 8-bit SAD can improve the throughput significantly. Since many mobile chips have both 8-bit encoding as well as multi-format decoding capability, the use of 8-bit SAD can help leverage any SAD engines employed for encoding on these chipsets.

# Complexity analysis

Compared to MR-SAD that requires ~288 additions for mean calculation along with several address calculation overheads and 3 operations per sample per refinement position for the worst case block size of 8x8, the P-I-MR-SAD requires only 200 additions for mean calculation, 64 additions for mean adjustment, and 2 operations per sample per refinement position. In addition, the bit-depth reduction to 8-bits allows these calculations to be performed using 8-bit instead of 14-bit samples that are used currently, which reduces the ALU widths and requires 6 fewer bits in the accumulators.

# Test Results

The test results are reported as per the CE requirements. BMS-2.1rc is the code base used. The results compare proposed DMVR Tool ON against the VTM-2.0 anchor (run through BMS configuration file). The tests are performed with –SIMD=SSE42.

- Base version: BMS-DMVR + Bilinear Interpolation ((JVET-K0041, JVET-K0359, JVET-K0361)) + Padded MC to only pre-fetch (CU_w + 7)*(CU_h + 7) (JVET-K0275) + CE9.1.4 + CE9.2.5 elements (JVET-K0041)

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.14% | -1.80% | -2.16% | 103% | 108% |
| Class A2 | -2.82% | -2.80% | -2.86% | 102% | 107% |
| Class B | -1.61% | -1.61% | -1.70% | 104% | 111% |
| Class C | -1.56% | -1.71% | -1.80% | 104% | 115% |
| Class E |  |  |  |  |  |
| Overall | -1.95% | -1.91% | -2.05% | 103% | 111% |
| Class D | -1.93% | -1.98% | -2.14% | 104% | 115% |

- Impact of constraining the bilinear interpolated samples to 8-bits

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.13% | -1.82% | -2.17% | 103% | 107% |
| Class A2 | -2.84% | -2.81% | -2.90% | 102% | 106% |
| Class B | -1.60% | -1.68% | -1.71% | 104% | 111% |
| Class C | -1.57% | -1.74% | -1.78% | 103% | 114% |
| Class E |  |  |  |  |  |
| Overall | -1.95% | -1.95% | -2.06% | 103% | 110% |
| Class D | -1.91% | -2.02% | -2.12% | 104% | 115% |

There is no BDRATE impact due to the 8-bit restriction. The decoding time has reduced by 1%.

- Impact of changing the cost function to P-I-MR-SAD

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | -1.73% | -2.10% | 103% | 108% |
| Class A2 | -2.77% | -2.78% | -2.81% | 101% | 105% |
| Class B | -1.56% | -1.67% | -1.70% | 103% | 110% |
| Class C | -1.51% | -1.66% | -1.73% | 103% | 114% |
| Class E |  |  |  |  |  |
| Overall | -1.89% | -1.90% | -2.01% | 103% | 110% |
| Class D | -1.90% | -2.06% | -2.04% | 103% | 113% |

There is a 0.06% BDRATE impact due to replacing MR-SAD with P-I-MR-SAD. The decoding time has reduced by 1%.

- Impact of (b) + (c) together

|  | Over VTM 2.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.07% | -1.79% | -2.12% | 103% | 107% |
| Class A2 | -2.76% | -2.71% | -2.78% | 102% | 105% |
| Class B | -1.55% | -1.64% | -1.69% | 104% | 110% |
| Class C | -1.51% | -1.67% | -1.76% | 103% | 114% |
| Class E |  |  |  |  |  |
| Overall | -1.89% | -1.89% | -2.01% | 103% | 109% |
| Class D | -1.93% | -2.04% | -2.11% | 104% | 114% |

The combination has the same BDRATE impact as in (c) and the overall average decoding time has shown a 2% reduction.

# Summary

The two proposed simplifications of interpolated sample bit-depth restriction and position independent mean-removed SAD cost function have clear benefits in both hardware and software implementations. The BDRATE impact is minimal (0.06%). We request that these additional elements be studied further in ongoing core experiments on DMVR.

# References

- S. Esenlik, Y. Chen, F. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-K1029, July 2018.

- S. Sethuraman, Jeeva Raj A., S. Kotecha, “Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques”, JVET-K0041, July 2018.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).