| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0175 |
| --- | --- |

| Title: | CE 4: Extended Non-adjacent Spatial Merge Candidates (Test 4.4.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents extended spatial merge candidates based on VTM-2.0.1. The extended spatial merge candidates are added into the merge candidates list. The proposed technologies can provide 0.19%, 0.37% gain for merge list size equal to 6 and 10 respectively compared to VTM-2.0.1 anchor with no encoding and decoding time increase.

Introduction

In current VTM, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates. Figure 1 shows the positions of five spatial candidates. For each candidate position, the availability is checked according to the numerical order {1, 2, 3, 4, 5}.

Figure 1. Positions of spatial candidates in HEVC.

Proposed improvements

- Extended Spatial Merge Candidates

In this proposed method, more spatial positions are checked as shown in Figure 2. The extended spatial positions from 6 to 14 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, the spatial candidates beyond the line above the current CU excluded. In order to save the local memory buffer, all the spatial candidates are restricted within two or one CTU lines.

Figure 2. Positions of extended spatial candidates in the proposed method.

The number of candidates within the merge candidate list is controlled by NumMrgCands. In VTM2.0.1, NumMrgCands=6. In this proposed method, the NumMrgCands is added by 0/4, so the NumMrgCands equals to 6/10.

The whole process of adding candidates will stop as soon as the number of candidates reaches NumMrgCandsduring the merge candidate list construction process. The redundancy check is done for all the merge candidates except the generated ones. That is, only unique motion candidate from spatial positions, ATMVP, and temporal candidate, can be included the merge candidate list. The merge candidate list is constructed as follows:

- Add the spatial positions from position 1 to 5, ATMVP candidates, and temporal candidate, which is the same as in current JEM; (include redundancy check)

- Add the extended spatial positions from position 6 to 14; (include redundancy check)

Experimental Results

The proposed method has been implemented on VTM-2.0.1[1], and allthe tests were conducted under the common testconditions[2]. The experimental results of extended spatial mergecandidatescompared to VTM-2.0.1 are shown in Table1.

CASE 1 (Merge list size = 6)

Table 1. Experimental results of Extended Spatial Merge Candidates(Merge list size = 6)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.04% | -0.18% | 98% | 99% |
| Class A2 | -0.15% | -0.30% | -0.28% | 99% | 99% |
| Class B | -0.36% | -0.34% | -0.47% | 99% | 100% |
| Class C | -0.09% | -0.01% | -0.23% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.18% | -0.31% | 99% | 99% |
| Class D | 0.02% | -0.15% | -0.09% | 99% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | -0.39% | -0.34% | 99% | 99% |
| Class C | 0.04% | 0.01% | -0.10% | 100% | 101% |
| Class E | -0.12% | 0.30% | 0.14% | 100% | 103% |
| Overall | -0.14% | -0.08% | -0.14% | 100% | 101% |
| Class D | 0.09% | 0.42% | 0.10% | 100% | 102% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.61% | -0.45% | -0.38% | 100% | 99% |
| Class C | -0.34% | -0.60% | -0.54% | 100% | 102% |
| Class E | -0.07% | -0.16% | 0.58% | 100% | 99% |
| Overall | -0.39% | -0.43% | -0.19% | 100% | 100% |
| Class D | -0.22% | -0.03% | -0.44% | 100% | 101% |

CASE 2 (Merge list size = 10)

Table 2. Experimental results of Extended Spatial Merge Candidates(Merge list size = 10)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.24% | -0.40% | 100% | 99% |
| Class A2 | -0.38% | -0.60% | -0.71% | 101% | 100% |
| Class B | -0.53% | -0.55% | -0.60% | 101% | 99% |
| Class C | -0.24% | -0.23% | -0.40% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.41% | -0.53% | 101% | 100% |
| Class D | -0.13% | -0.29% | -0.08% | 102% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.45% | -0.53% | 101% | 98% |
| Class C | -0.14% | -0.21% | -0.22% | 103% | 102% |
| Class E | -0.18% | 0.05% | -0.53% | 102% | 100% |
| Overall | -0.30% | -0.24% | -0.43% | 102% | 100% |
| Class D | -0.05% | 0.37% | 0.08% | 103% | 102% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -0.52% | -0.39% | 102% | 100% |
| Class C | -0.32% | -0.42% | -0.22% | 103% | 103% |
| Class E | -0.18% | -0.26% | 0.31% | 103% | 100% |
| Overall | -0.41% | -0.42% | -0.16% | 103% | 101% |
| Class D | -0.20% | 0.00% | -0.65% | 104% | 102% |

CASE 3 (Merge list size = 10 with max 30 prunning for list construction)

Table 3. Experimental results of Extended Spatial Merge Candidates(Merge list size = 10, max 30 prunning for list construction)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.23% | -0.36% | 101% | 100% |
| Class A2 | -0.32% | -0.50% | -0.56% | 101% | 99% |
| Class B | -0.51% | -0.51% | -0.55% | 101% | 99% |
| Class C | -0.20% | -0.22% | -0.34% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.37% | -0.46% | 101% | 100% |
| Class D | -0.12% | -0.23% | -0.18% | 102% | 99% |

Table1-3 show the gain for extended spatial merge candidates.

Reference

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM.

- F. Bossen,J. Boyce, X. Li, V. Seregin,K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010,Ljubljana, SI, 10–18July 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).