| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0176_WD |
| --- | --- |

| Title: | Proposed WD for CE 4: Merge Offset Extension (Test 4.4.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Proposed Working Draft Changes

This document provides the proposed text of VVC Working Draft. The changes are highlighted in yellow.

8.3.2.8  Derivation process for merge offset candidates

Inputs to this process are:

- first available candidate in mergeCandList,

- merge_idx[ x0 ][ y0 ].

Output of this process is the merge offset candidate.

The merge offset candidate is derived in the following ordered steps:

- construct first available candidate in mergeCandList.

- merge offset candidate is derived as follows:

merge offset candidate _IDX0 = Candidate0_IDX0;

merge offset candidate _IDX1 = Candidate0_IDX1;

If refIdxL0N is over 0,

merge offset candidate _MVL0 = Candidate0_MVL0 + Mv_offset[i];

merge offset candidate _IDX0 = Candidate0_IDX0;

If refIdxL1N is over 0,

merge offset candidate _MVL1 = Candidate0_MVL1 + Mv_offset[i];

merge offset candidate _IDX1 = Candidate0_IDX1;

If both refIdxL0N and refIdxL1N are over 0 and ( DiffPicOrderCnt( RefPicList0[ refIdxL0 ], currPic) * DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] ) ) is negative,

merge offset candidate _MVL0 = Candidate0_MVL0 + Mv_offset[i];

merge offset candidate _MVL1 = Candidate0_MVL1 - Mv_offset[i];

If both refIdxL0N and refIdxL1N are over 0 and ( DiffPicOrderCnt( RefPicList0[ refIdxL0 ], currPic) * DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] ) ) is positive,

merge offset candidate _MVL0 = Candidate0_MVL0 + Mv_offset[i];

merge offset candidate _MVL1 = Candidate0_MVL1 + Mv_offset[i];

Candidate0 is the first available candidate in mergeCandList. i is set equal to merge_idx[ x0 ][ y0 ] - MaxmergeCand. MaxmergeCand is equal to merge list size.

Mv_offset[i] is predefined as follow:

Mv_offset[0] = (4, 0); Mv_offset[1] = (0, 4); Mv_offset[2] = (-4, 0); Mv_offset[3] = (0, -4); Mv_offset[4] = (4, 4); Mv_offset[5] = (-4, 4); Mv_offset[6] = (-4, -4); Mv_offset[7] = (4, -4); Mv_offset[8] = (8, 0); Mv_offset[9] = (0, 8); Mv_offset[10] = (-8, 0); Mv_offset[11] = (0, -8); Mv_offset[12] = (8, 8); Mv_offset[13] = (-8, 8); Mv_offset[14] = (-8, -8); Mv_offset[15] = (8, -8);

- Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).