| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0176 |
| --- | --- |

| Title: | CE 4: Merge Offset Extension (Test 4.4.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xu Chen Jianhua Zheng | Tel: Email: | +86 13480990067 anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution presents merge offset extension based on VTM-2.0.1. More MV offsets are added to be merge candidate. For solution 1, it can provide 0.49%, 0.19%, 0.11% gain for RA, LB, LP respectively compared to VTM-2.0.1 anchor with around 8% encoding time and no decoding time increase. For solution 2, it can provide 0.78%, 0.17%, 0.13% gain for RA, LB, LP respectively compared to VTM-2.0.1 anchor with around 7% encoding time and no decoding time increase.

Introduction

In current VTM, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates. Figure 1 shows the positions of five spatial candidates. For each candidate position, the availability is checked according to the numerical order {1, 2, 3, 4, 5}.

Figure 1. Positions of spatial candidates in HEVC.

Proposed improvements

- Merge offset extension

In current VTM, the set of possible candidates in the merge mode consists of spatial neighbour candidates, ATMVP candidates, a temporal candidate, and generated candidates.

In Merge offset extension, more candidates are checked based on existed candidate. As Figure 2 shows, new candidates has mv offset to the mv of existed candidate.

Figure 2.  New MV candidate

- Solution 1

The number of candidates within the merge candidate list is controlled by NumMrgCands. In VTM, NumMrgCands=6. Formerge offset extension, 4new MV candidates with oneluma sample offset, so the NumMrgCands is added by 4. In this proposed solution, the NumMrgCands equals to 10.

Figure 3New merge candidate list construction

As Figure 3 shows, new extended mv offset candidates are constructed only based on the first candidate of merge candidate list. New extended mv offset candidates only has mv offset to the current mv of first candidate and other prediction information is same to first candidate.

Figure 4Offset candidates (red ones)

In the proposed solution, 4 new MV candidates include MV' surrounding candidates with one luma sample offset to the first candidate MV in either the horizontal or vertical direction. Finally, the best refined MV' are used for generating the final prediction results. For bi-prediction with one MV from a reference picture in the past and another from a reference picture in the future case, add the mv offset to first candidate MV0, and add the reverse mv offset to first candidate MV1. Other bi-prediction cases add same mv offset to first candidate MV0 and MV1 respectively.

- Solution 2

For solution 2, 8MV candidates with one luma sample offset and 8 8 MV candidates with twoluma sample offset.The NumMrgCands is added by 1. When one of offset candidates is selected, offset index is needed for 16 offset candidates.

Figure 5New merge candidate list construction

As Figure 5 shows, new extended mv offset candidates are constructed only based on the first candidate of merge candidate list. New extended mv offset candidates only has mv offset to the current mv of first candidate and other prediction information is same to first candidate.

Figure 6Offset merge candidates(red and blue ones)

In the proposed solution, 16 new MV candidates include MV' surrounding candidates with one and two luma sample offset to the first candidate MV in either the horizontal or vertical direction, and both. Finally, the best refined MV' are used for generating the final prediction results. For bi-prediction with one MV from a reference picture in the past and another from a reference picture in the future case, add the mv offset to first candidate MV0, and add the reverse mv offset to first candidate MV1. Other bi-prediction cases add same mv offset to first candidate MV0 and MV1 respectively.

Experimental Results

The proposed method has been implemented on VTM-2.0.1[1], and all the tests were conducted under the common test conditions[2]. The experimental results of themerge offset extensioncompared to VTM-2.0.1 are shown in Table1.

- Solution 1

Table 1. Experimental results of Merge offset extension(Solution 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | -0.33% | -0.58% | 108% | 98% |
| Class A2 | -0.69% | -1.01% | -1.18% | 107% | 98% |
| Class B | -0.51% | -0.59% | -0.71% | 108% | 98% |
| Class C | -0.25% | -0.31% | -0.33% | 108% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.49% | -0.55% | -0.68% | 108% | 99% |
| Class D | -0.22% | -0.35% | -0.58% | 108% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.58% | -0.60% | 109% | 97% |
| Class C | -0.17% | -0.35% | -0.64% | 108% | 102% |
| Class E | -0.19% | -0.38% | -0.23% | 106% | 101% |
| Overall | -0.19% | -0.45% | -0.52% | 108% | 100% |
| Class D | -0.12% | -0.17% | -0.78% | 107% | 101% |

Table1 show the gain for merge offset extension(solution 1).

- Solution 2

Table 2. Experimental results of Merge offset extension(Solution 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.60% | -0.90% | 104% | 97% |
| Class A2 | -1.15% | -1.58% | -1.74% | 104% | 97% |
| Class B | -0.79% | -0.93% | -1.04% | 106% | 98% |
| Class C | -0.39% | -0.51% | -0.69% | 108% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -0.88% | -1.06% | 106% | 98% |
| Class D | -0.31% | -0.73% | -0.86% | 109% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.46% | -0.63% | 106% | 96% |
| Class C | -0.15% | -0.43% | -0.34% | 107% | 102% |
| Class E | -0.04% | -0.13% | -0.53% | 106% | 100% |
| Overall | -0.17% | -0.37% | -0.51% | 107% | 99% |
| Class D | -0.11% | 0.25% | -0.96% | 108% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.29% | -0.11% | 110% | 98% |
| Class C | -0.15% | -0.18% | -0.45% | 111% | 103% |
| Class E | 0.03% | 0.12% | 0.33% | 111% | 100% |
| Overall | -0.13% | -0.15% | -0.11% | 110% | 100% |
| Class D | -0.05% | -0.14% | -0.98% | 112% | 104% |

Table2 show the gain for merge offset extension (solution 2).

Reference

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM.

- F. Bossen,J. Boyce, X. Li, V. Seregin,K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010,Ljubljana, SI, 10–18July 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).