| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0184 |
| --- | --- |

| Title: | CE1-related: Flexible Luma and Chroma Block Partitioning Trees Separation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An Yu-Chen Sun Jian Lou | Tel: Email: | +86-13439745728 jicheng.ajc@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes a flexible luma and chroma block partitioning trees separation method. The luma and chroma block partitioning trees can be separated under specific block size for both intra and inter slices. Compared to the current structure, the proposed method has the advantages including firstly, the minimum CB size for luma and chroma can be set independently also for inter slice; secondly, the intra and inter slices can share the same block partitioning structure with only high-level parameter adjustable; thirdly, the proposed method is more flexible, the current VTM is a special configuration of the proposed method. For inter slice, a cross component motion information prediction is proposed to reduce the bit cost of the chroma motion information signaling due to the trees separation. Currently, the implementation is finished for intra slice.

Introduction

In current VVC, the luma chroma separate block partitioning trees are used for intra (I) slice under 64x64 luma block and 32x32 chroma block, and luma chroma shared block partitioning tree is used for inter (P and B) slice. For the inter slice, the minimum coding block for luma and chroma cannot be set independently, currently the minimum luma CB is 4x4 luma samples which implies the minimum chroma CB is 2x2 chroma samples. For example, if we want to set the minimum luma CB and chroma CB size both equal to 4x4 luma and chroma samples respectively, the current structure cannot achieve it.

Proposed flexible luma and chroma block partitioning trees separation

This contribution proposes a flexible luma and chroma block partitioning trees separation method. In the proposed method, the luma and chroma block partitioning trees are separated under the separation node.

If the following two conditions are both met for one node, that node is defined as separation node:

- both its luma width and height are not smaller than separation size (SepSize) luma samples;

- at least one of its child nodes has luma width or height smaller than the SepSize luma samples.

The SepSize is a high-level parameter which can be set at slice or higher level.

Fig. 1 shows an example of the proposed block partitioning tree separation process. The root node size is 32x32, which means 32x32 luma samples and corresponding chroma samples (two 16x16 chroma sample blocks for 4:2:0 format). The SepSize is set to 8. The minimum luma CB side length is set to 4 luma samples, and the minimum chroma CB side length is set to 4 chroma samples. The CTU is split by the QTBTT block partitioning structure. There are 3 separation nodes in this example, i.e., the node A, B, and C as shown in Fig. 1. The black line indicates the shared tree partitioning for both luma and chroma, the red line illustrates the separate luma tree partitioning, and the blue line illustrates the separate chroma tree partitioning. It can be seen that the partitioning tree is separated when it goes deeper under the separation nodes.

For node A with 8x16 luma samples, both its luma width 8 and height 16 are not smaller than the SepSize 8, and its two child nodes 4x16 have luma width 4 smaller than SepSize, so the node A is a separation node. As shown in Fig. 4, its luma component is first split by vertical BT into two 4x16 luma block, and then the second 4x16 luma block is further split by horizontal TT; the chroma component of node A is split by horizontal BT into two 4x4 chroma blocks.

For node B, its luma width equal to luma height equal to the SepSize, and its child nodes have luma width 4 smaller than SepSize, so the node B is a separation node. Its luma component is further split by vertical BT into two 4x8 luma CBs. The chroma component of node B with 4x4 chroma samples is not further split since it already reaches the minimum chroma CB size.

For node C, its luma width and height are both larger than SepSize, and two of its child have luma width 4 smaller than the SepSize, so the node C is a separation node. Its luma component is split by vertical TT into 3 luma CBs, and its chroma component is split by QT into 4 chroma CBs.

Fig. 1. An example of the proposed tree separation process

It can be seen that, if the SepSize is set equal to the CTU side length, then the block partitioning trees are totally separated between luma and chroma from the starting point CTU level. If the SepSize is set equal to or smaller than the minimum luma CB side length, then the block partitioning tree is totally shared between luma and chroma.

Currently in the VTM2.0, for I slice the CTU equal to 128x128 luma block and 64x64 chroma blocks is first implicitly split into four 64x64 luma blocks and 32x32 chroma blocks, and then the separate trees are used under 64x64 luma block and 32x32 chroma block. For the proposed method, the implicit quadtree split (from 128x128 luma/64x64 chroma to 64x64 luma/32x32 chroma) is not changed, if the SepSize is set to 64, then the results should be the same as the current VTM2.0. if the SepSize is set to 4, then the results should be the same as the current VTM2.0 with shared tree configuration.

Compared to the current structure, the proposed method has the following advantages:

- The minimum CB size for luma and chroma can be set independently also for inter slice;

- The intra and inter slices share the same block partitioning structure with only high-level parameter SepSize adjustable.

- The proposed method is more flexible, the current VTM is a special configuration of the proposed method.

Cross-component motion information prediction

When the separate block partitioning trees are used for P and B slices, the intra or inter prediction mode, intra prediction direction, motion information (including inter prediction direction, reference index, and motion vectors, etc.,), and all the CU level information are all signaled respectively for luma CB and chroma CB.

Since the luma and chroma samples sharing the same region usually have similar motion information, it is not efficient to code the chroma motion information independently from its collocated luma motion information. In this contribution, it is proposed to predict the chroma motion information from the collocated luma motion information.

It is proposed to divide the current chroma CB into multiple sub CBs, and then for each chroma sub CB, the motion information is directly copied from that of collocated luma as shown in Fig. 2. This method is signaled as the first merge candidate at chroma CB level, i.e., the merge index 0.

Fig. 2. Motion information prediction from luma to chroma at sub-CB granularity

Experimental results

The proposed method has been implemented based on VTM2.0.1, currently only I slice is finished. Table 1 and Table 2 show the results of setting the SepSize equal to 64 and 4 respectively, as explained in section 2, the Table 1 should have almost the same performance as the current VTM2.0.1 CTC, and the Table 2 should have almost the same performance as the current VTM2.0.1 with shared tree configuration. The results of setting SepSize equal to 32, 16, and 8 are all provided in the attachment, as expected those results are in between the performance of SepSize equal to 64 and 4.

Table 1. Performance of SepSize equal to 64 compared to VTM2.0.1 CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.33% | 0.16% | 104% | 103% |
| Class A2 | -0.02% | 0.03% | -0.08% | 101% | 102% |
| Class B | -0.01% | -0.01% | -0.04% | 99% | 100% |
| Class C | -0.03% | -0.27% | -0.14% | 101% | 103% |
| Class E | -0.06% | -0.27% | -0.25% | 101% | 103% |
| Overall | -0.02% | -0.05% | -0.07% | 101% | 102% |
| Class D | -0.02% | -0.06% | -0.36% | 101% | 101% |
| Class F (optional) | -0.03% | -0.33% | -0.33% | 101% | 103% |

Table 2. Performance of SepSize equal to 64 compared to VTM2.0.1 with shared tree cfg

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1_sharedT_cqp1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.06% | 100% | 101% |
| Class A2 | 0.00% | -0.04% | 0.00% | 100% | 102% |
| Class B | 0.00% | -0.03% | 0.01% | 100% | 102% |
| Class C | 0.00% | -0.02% | -0.07% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.01% | 101% | 104% |
| Overall | 0.00% | -0.03% | -0.02% | 100% | 102% |
| Class D | 0.00% | 0.04% | -0.01% | 102% | 103% |
| Class F (optional) | 0.00% | 0.01% | 0.00% | 101% | 104% |

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).