| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0189_v2 |
| --- | --- |

| Title: | CE9-related: Improved Unidirectional Template based DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Shurui Ye Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents an improved unidirectional template based DMVR. To avoid latency and reuse refined MV within one CTU, new candidates based on searching pattern are generated. For bidirectional candidates, a better searching method is also developed. It is reported that the proposed method can provide 2.43%, X.XX%, X.XX% gain for RA, LB, LP respectively compared to VTM-2.0 with around X% encoding time and X% decoding time increase.

Introduction

In [1], we propose a unidirectional template based DMVR. As shown in Figure 1, the template is compose of two parts, i.e., the upper sub-template (TM1) and the left sub-template (TM2). The same strategy is adopted for the generation of two sub-templates. For TM1, it is obtained with the motion information of first upper PU, whose horizontal position is the same as that of current block. The width of TM1 is the same as that of the first upper PU, and its height is set as a fixed value (1 for current version).With basic motion information, i.e., MV, reference index, and inter direction, the TM1 is obtained with simple interpolation using these information. If the first upper PU is bidirectional, then the TM1 is the average of two direction. Moreover, if the first upper PU adopts intra predication mode, TM1 will be invalid and unnecessary to be constructed. Similarly, the TM2 is also acquired with the motion information of first left PU, whose vertical position is the same as that of current block. And the height of TM2 is the same as that of the first left PU, and its width is also fixed (1 for current version). Then, if it is intra coded, TM2 would be also unavailable. As a result, when both of the first PU and the first left PU are unavailable (intra coded or non-existent), the proposed method would be invalid, and the related mode checking will be also skipped.

To avoid latency, only the refined MV of block in the above CTU will utilized for spatial motion prediction. Otherwise, the non-refined MV will be used for spatial motion prediction. However, the constraint brings significant loss of coding performance.

Fig. 1 The template generation for current block

Proposed improvements

First, new candidates based on searching pattern are generated to obtain MVs similar to refined MVs. In [1], if the neighbouring PU utilizes MVR mode, then only its non-refined MV can be used as predictor. Since the difference between refined MV and non-refined MV has fixed pattern, we can try to generate some MVs similar to refined MVs, and add them into candidate list. That is, new candidates can be generated by adding the fixed offset (-1,-1)，(+1,+1)，(-2,-2)，(+2,+2) with the first candidate if it has refined MV. Moreover, the refined MVs of the nearest row in the above CTU are also inserted into the candidate list.

Second, for bidirectional candidates, the searching process is only conducted on list0, and the refined MV of list1 is directly derived without repeating the searching process. And the MV differences of list0 and list1 are simply mirrored.

Experimental Results

To evaluate the coding performance of the proposed method, the BMS-2.1 [2] with VTM configuration are served as the experimental platform. Three coding configurations, namely, Random Access (RA) Main 10 Profile, Low Delay B (LDB) Main 10 Profile, and Low Delay P (LDP) Main 10 Profile specified in [3], are adopted, and all the sequences are tested across the configurations with the common test conditions. BD-rate is used to evaluate the final coding performance.

The results of the improved unidirectional template based DMVR are given in Table I. It is reported that the proposed method achieves 0.73% gains with 10% encoding time increase and 6% decoding time increase for RA configuration. The results of the combination of the proposed method and the modified DMVR in [1] are also given in Table II. It is reported that the proposed method achieves 2.43% gains with 19% encoding time increase and 24% decoding time increase for RA configuration.

Table I Results of the improved unidirectional template based DMVR

|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over BMS 2.1 with VTM configuration |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  |  |  | U |  | V | EncT | DecT |
| Class A1 |  | -0.60% |  |  |  |  | -0.67% |  | -0.54% | 108% | 103% |
| Class A2 |  | -0.89% |  |  |  |  | -0.76% |  | -0.89% | 109% | 106% |
| Class B |  | -0.79% |  |  |  |  | -0.77% |  | -0.77% | 110% | 106% |
| Class C |  | -0.62% |  |  |  |  | -0.65% |  | -0.58% | 113% | 108% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.73% |  |  |  |  | -0.72% |  | -0.70% | 110% | 106% |
| Class D |  | -0.65% |  |  |  |  | -0.45% |  | -0.75% | 114% | 110% |

Table II Results of combination of the proposed method and DMVR

Random Access Main 10. Over BMS 2.1 with VTM configuration. Y; U; V; EncT; DecT. Class A1; -2.64%; -2.23%; -2.70%; 115%; 117%. Class A2; -3.34%; -3.21%; -3.34%; 116%; 119%. Class B; -2.10%; -2.18%; -2.23%; 119%; 125%. Class C; -2.02%; -2.21%; -2.26%; 123%; 131%. Class E. Overall; -2.43%; -2.41%; -2.55%; 119%; 124%. Class D; -2.28%; -2.21%; -2.51%; 126%; 132%. Low delay B Main10. Over BMS 2.1 with VTM configuration. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class C; -0.22%; -0.43%; 0.03%; 120%; 113%. Class E; 0.02%; 0.30%; 0.61%; 115%; 110%. Overall; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class D; -0.13%; 0.00%; -0.49%; 120%; 108%. Low delay P Main10. Over BMS 2.1 with VTM configuration. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class C; -0.88%; -0.80%; -0.68%; 122%; 114%. Class E; -0.39%; 0.43%; 0.00%; 118%; 107%. Overall; #VALUE!; #VALUE!; #VALUE!; #NUM!; #NUM!. Class D; -1.14%; -0.97%; -0.99%; 123%; 111%.

Conclusion

In this proposal, we propose a template based motion vector refinement method. It has low no latency because of the reuse of neighboring motion, and it brings low decoder complexity because of the limited size of template, and significant coding gains are achieved. Based on the above results, it is suggested to study it in the related CE.

Reference

- F. Chen, L. Wang, “CE9: Unidirectional Template based DMVR and its Combination with Simplified Bidirectional DMVR (Test 9.2.10 and Test 9.2.11),” JVET-L0188, Macao, CN, 3–12 Oct. 2018.

- BMS-2.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE9/VVCSoftware_BMS.

- F. Bossen,J. Boyce, X. Li, V. Seregin,K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010,Ljubljana, SI, 10–18July 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).