| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0197-v2 |
| --- | --- |

| Title: | CE4-Related: Generalized bi-prediction improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chi Su, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yu-chi.su, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents four methods to improve BMS2.1 generalized bi-prediction (GBi), including mvd_l1_zero_flag dependent dual weight sets, coding unit (CU) size constraint for GBi, early termination of affine inter mode based on best mode of equal weight pair, and the pre-selection of GBi weight pairs based on picture order count (POC) differences. Compared to BMS2.1 with VTM configuration (GBi off), simulation results show that GBi with the four proposed methods achieves -0.54% and -0.46% luma BD-rates with 9% and 9% encoding time increases and 1% and 0% decoding time increases for RA and LB, respectively.

# Introduction

GBi was proposed in JVET-C0047 [1]. JVET-K0248 [2] improved the gain-complexity trade-off for GBi and was adopted into BMS2.1. The BMS2.1 GBi applies unequal weights to predictors from L0 and L1 in bi-prediction mode. In inter prediction mode, multiple weight pairs including the equal weight pair (1/2, 1/2) are evaluated based on rate-distortion optimization (RDO), and the GBi index of the selected weight pair is signaled to the decoder. In merge mode, the GBi index is inherited from a neighboring CU. In BMS2.1 GBi, the predictor generation in bi-prediction mode is shown in Equation (1). 

          PGBi = (w0 * PL0 + w1 * PL1 + RoundingOffsetGBi) >> shiftNumGBi,             (1)

where PGBi is the final predictor of GBi. w0 and w1 are the selected GBi weight pair and applied to the predictors of list 0 (L0) and list 1 (L1), respectively. RoundingOffsetGBi and shiftNumGBi are used to normalize the final predictor in GBi. The supported w1 weight set is {-1/4, 3/8, 1/2, 5/8, 5/4}, in which the five weights correspond to one equal weight pair and four unequal weight pairs. The blending gain, i.e., sum of w1 and w0, is fixed to 1.0. Therefore, the corresponding w0 weight set is {5/4, 5/8, 1/2, 3/8, -1/4}. The weight pair selection is at CU-level.

For non-low delay pictures, the weight set size is reduced from five to three, where the w1 weight set is {3/8, 1/2, 5/8} and the w0 weight set is {5/8, 1/2, 3/8}. The weight set size reduction for non-low delay pictures is applied to the BMS2.1 GBi and all the GBi tests in this contribution.

# Proposed methods

## mvd_l1_zero_flag dependent dual weight sets

In this method, two weight sets are provided and switched depending on mvd_l1_zero_flag. If mvd_l1_zero_flag is equal to zero, the w1 weight set is {-1/4, 3/8, 1/2, 5/8, 5/4}; otherwise, the w1 weight set is {11/8, 3/4, 1/2, 5/8, 5/4}. The number of loops for GBi weight search at encoder is kept the same as that in BMS2.1 GBi, so the encoding time change by this method is minor.

## CU size constraint for GBi

In this method, GBi is disabled for small CUs. In inter prediction mode, if bi-prediction is used and the CU size is smaller than 256 luma samples, GBi is disabled without any signaling.

## Early termination of affine inter mode based on best mode of equal weight pair

In BMS2.1 GBi, the equal weight pair is evaluated first, and unequal weight pairs are evaluated subsequently at the encoder. In this method, if the best mode of the equal weight pair in advanced motion vector prediction (AMVP) mode test is not an affine inter mode, affine inter modes during evaluation of unequal weight pairs are skipped.

## Pre-selection of weight pairs based on POC differences

In BMS2.1 GBi, the equal weight pair is evaluated first. In this method, in low delay pictures, if the POC difference between the L0 reference picture selected in the equal weight test and the current picture is larger than the POC difference between the L1 reference picture selected in the equal weight test and the current picture, evaluation of w0 = 5/4 and w1 = -1/4 is skipped at the encoder; otherwise, evaluation of w0 = -1/4 and w1 = 5/4 is skipped at the encoder.

# Experimental results

The proposed methods are implemented on top of BMS2.1 [3]. Results of GBi with the four proposed methods on BMS2.1 with VTM configuration (GBi off) are shown in Table 1.

Table1. Results of the proposed GBi on BMS2.1 with VTM configuration, where anchor disables GBi

# Conclusions

This contribution presents several methods to improve BMS2.1 GBi. The proposed GBi reportedly achieves -0.54% and -0.46% luma BD-rates with 9% and 9% encoding time increases and 1% and 0% decoding time increases for RA and LB, respectively, when compared against BMS2.1 with VTM configuration, where the anchor disables GBi.

# Patent rights declaration

MediaTek, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- [1] C.-C. Chen, X. Xiu, Y. He and Y. Ye, “Generalized bi-prediction for inter coding,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-C0047, May 2016.

[2] Y.-C, Su, T.-D. Chung, C-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE4.4: Generalized bi-prediction for inter coding,” Document of Joint Video Experts Team, JVET-K0248.

[3] BMS-2.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.1