| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0202 |
| --- | --- |

| Title: | HLS for spatial relation between independent VVC sub bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexandre Gabriel, Emmanuel Thomas | Tel: Email: | 0031 611 843 111 {alexandre.dasilvapratasgabriel , emmanuel.thomas}@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution elaborate on JCTVC-AB0032 [1] and proposes a method for defining independent sub video bit streams depicting spatial regions of a video which constitute together a single bitstream. In the context of this document, these independent sub video bit streams are called Ttiles but they are orthogonal to the current HEVC tile features. A new parameter set called Tile-positioning Parameter Set (TPS) is proposed which is pointed by a the Sequence Parameter Set (SPS) and links to a possible Video Parameter Set (VPS). A TPS permits describing the relation between different tiles by using the concept of hooks. Hooks permit describing the relative position, hence not absolute, between tiles without a coordinate system. A pair of matching hook identifiers express that two tiles are neighbours along a certain border. No matching hook identifiers express the absence of spatial relationship between two tiles.

# Preliminary observation

The goal of this proposal is NOT to replace current HEVC tiles concept. As a matter of fact, this proposal and the current HEVC tile concept are orthogonal and can coexist in a bitstream.

This proposal addresses the concept of spatial random access at a higher level of the syntax.

# Problem Statement

## Background

Figure 1 – Panorama with a particular Region of Interest

There are a several applications where the spatial subdivision of a frame can be useful. Examples include applications when the user may want to only watch one area of the full video and/or pan. In this case, streaming and/or decoding the whole frame only to display a sub part of it is inneficient. It is desirable to be able to access only a region of the video. To this end, HEVC sequences image can be divided into motion constrained tile sets so that the decoder may only decode the necessary tiles to render the viewpoint the user desires.

Figure 1 depicts a situation where a user zooms in to see the first cyclists in the group. In this case, decoding the whole frame would not be an efficient use of computing power.

Another application of spatially segmented video is the mixing of bitstreams to produce a new video such as in teleconference, see Figure 2.

Figure 2 – Multiparty conference call

Lastly, VR 360 applications do not render the whole video but only a small part of it corresponding to the user's viewport. Figure 3 shows an example of how a tiling system can enable bandwidth savings when streaming: if a user is looking down then only tiles numbered 3 and 4 are needed in order for the application to render the user's viewport. In this case, this would allow for approximately 50% saving in transmitted bitrate compared to sending the whole bitstream. Note that this tilling of ERP is merely an example and any other tiling scheme could be used to describe the VR application case. Later in the document, an improved tiling grid is presented.

Figure 3 – 360 video partitioned into 4 tiles (red) and the user's viewport (yellow)

## Problems with HEVC tiles

Although HEVC tiles gained momentum for enabling spatial random access over a sequence of frames,  the original design of HEVC tiles did not anticipate these use cases and others that would eventually be implemented. As a result, there are several shortcomings and inefficiency of the HEVC tile definition for these uses cases. Essentially, these issues are related to the fact that HEVC tiles were designed for in-frame encoding/decoding parallelisation but not for true spatial random access, especially over several frames.

The main drawbacks of the HEVC tile concept are:

- grid structure

- predetermined absolute position of tiles

- absence of high-level syntax element for easy parsing

### Rigid Grid Structure

One of the impediments results in HEVC tiles always having to be in a grid format. Figure 4 shows a frame partition that it is not possible to have with HEVC tiles, i.e. one tile above another with different widths (region 2 and 4), or tiles side by side with different heights (region 1 and 2). This scenario was also proposed in JVET-K0260 [1].

Figure 4 – Example of heterogeneous tile size that is not currently possible

Looking at the ERP video example in Figure 3, a different tile distribution might be more suitable with the pole regions being composed of one tile each and the equatorial area being covered in tiles – see Figure 5 as described in ISO/IEC 23090-2 OMAF Annex D [2].

Figure 5 - Improved tiling grid for an 360 ERP video

### Predetermined Absolute Tile Position

The HEVC syntax specifies the spatial relations between HEVC tiles at a low level in the bitstream. This means that if an alteration of the tile (e.g. position, replacement) has to be made, for example on-the-fly by a network element, almost the entire bitstream syntax has to be parsed to find the point where the tile position and relations are described which is a cumbersome and time-consuming task. This is increasingly relevant in the streaming of 360 videos.

While a possible workaround is to constrain one HEVC tile per slice. However, the number of slices going up causes a higher overhead due to slice headers in the frame.

### Others

Furthermore, the encoding of the HEVC content for such applications using HEVC tile-based access requires careful configuration of the encoders. From the literature in the domain [4] , there are well-known specific configuration to be applied :

- in-loop filtering needs to be disabled

- motion vectors needs to be constrained within the same HEVC tiles over a desired number of frames

- the tile grid needs to be invariant over successive frames

- the invariance of the tile grid needs to be signalled to the decoder

- HEVC tiles position and address may have to be updated if a HEVC tile changes position in the frame

- HEVC tiles are not a bitstream partition which makes them invisible to the high level syntax of the HEVC bitstream

All of these problems are being addressed in ongoing standardisation work. However, the amount and the complexity of these measures to be taken advocate a more fundamental design of the bitstream to enable spatial random access by design which HEVC lacks.

# Proposed solution – Tiles and Hooks

## Functionality of Tiles and Hooks

The proposed solution uses the concept of Tiles and hooks to describe the spatial relationship between spatial regions. Hooks serve as descriptors of points of contact between different Tiles. A Tile has 4 hooks, top, bottom, left and right. If the identifier of two hooks, either north/south or east/west, from different Tiles match then the Tiles are adjacent to each other at this common edge. Figure 6 shows an example of matching and non matching hooks.

Figure 6 – Example of hooks and exemplative identification numbers

One technical benefit of hooks is that they allow for variable sizes of Tiles. The way a decoder can reassemble the Tiles is described below with examples from Figure 6.

- Tiles vertically adjacent – region 1 and 2 (bottom hook of 2 and top hook of 1 have same ID – 76 in the example)

  - The decoder will place region 1’s top left pixel below region 2’s bottom left pixel

- Tiles horizontally adjacent – region 3 and 1 (right hook of 3 and left hook of 1 have same ID – 1 in the example)

  - The decoder will place region 1’s top left pixel to the right of region 3’s top right pixel

Figure 7 - Hooks dictate how Tiles are reassembled

There is also the possibility of having multiple hook configurations for a layout of Tiles as can be seen in Figure 8, which shows the case of multiple tile sizes as well as different hook matching configurations. The decoder would be able to deduce the image on the left by looking at the pixel resolutions of each of the Tiles and following the logic explained previously.

Figure 8 – Multiple hook configuration for the same tile layout

## Proposed syntax

### New Parameter Set

The parameter sets from HEVC have a certain relation that can be seen in Figure 9.

Figure 9 – HEVC parameter sets

Table 1 gives a summary of the information these HEVC parameter sets contain.

Table 1 - Some of the purposes of HEVC parameter sets

| Parameter Set | Type of information |
| --- | --- |
| Video Parameter Set (VPS) | Information regarding layers in the bitstream etc. |
| Sequence Parameter Set (SPS) | Information about all slices of a sequence; e.g., profile, picture size, etc. |
| Picture Parameter Set (PPS) | Information that can change from picture to picture, e.g. tile grid, loop filters, etc. |

The solution is located at a high-level in the syntax such that it can be easily read by a bitstream parser which can be a on-the-fly bitstream processer in the network or a software program on devices. This contribution suggests the creation of a new parameter set – Tile-positioning Parameter Set (TPS) – that is inserted between the Video Parameter Set (VPS) and the Sequence Parameter Set (SPS) as they are currently defined in HEVC. The new relation between parameter sets can be seen in Figure 10.

Figure 10 – Proposed relation between parameter sets

The proposed Tile-positioning Parameter Set syntax and semantics are below.

### Syntax and Semantics

Syntax:

| tile_positioning_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| tps_video_parameter_set_id | u(4) |
| tps_tile_positioning_parameter_set_id | ue(v) |
| boundary_identifier_north | u(8) |
| boundary_identifier_east | u(8) |
| boundary_identifier_south | u(8) |
| boundary_identifier_west | u(8) |
| byte_alignment() |  |
| } |  |

Semantics:

| Syntax element | Meaning |
| --- | --- |
| tps_video_parameter_set_id | VPS NAL unit id that the TPS depends on; |
| tps_tile_positioning_parameter_set_id | Id of this Tile-positioning Parameter Set; |
| boundary_identifier_north | Identifier at the north boundary of the tile; |
| boundary_identifier_east | Identifier at the east boundary of the tile; |
| boundary_identifier_south | Identifier at the south boundary of the tile; |
| boundary_identifier_west | Identifier at the west boundary of the tile. |

In addition, a nuh_tile_id is added to the NAL header.

Syntax:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_bits | f(3) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_tile_id | u(6) |
| } |  |

Semantics:

| Syntax element | Meaning |
| --- | --- |
| nuh_tile_id | nuh_tile_id ≠ 0 tps_tile_positioning_parameter_set_id to which the NAL refers to. nuh_tile_id = 0 NAL unit that has no relation with tiles and is invariant to merging and extraction operations of tile bitstreams |

The advantages of having this new parameter set, essentially linked to efficient bitstream parsing, are the following:

- Tile position reshuffling in the same bitstream is fast since it only requires the parsing of the high-level syntax.

- It is also possible to merge two or more bitstreams that have compatible characteristics into a single bitstream by introducing the relevant TPS and ids in the NAL unit headers without complex rewriting deep in the bitstream.

## Insight on coding efficiency impact of proposed Tiles

### BD-rate comparison of HEVC Motion Constrained tiles vs. HEVC normal tiles

The proposed solution would mean that when encoding one sequence with many different Tiles there would be the equivalent of having HEVC motion constrained tiles. An experiment was conducted to see the cost in coding efficiency by using normal tiles as the anchor and MCTS as the comparison point.

Table 2 shows the BD rate difference between nine different 4K sequences with three different tiling schemes. The tiling schemes are the same used in a previous contribution submitted to MPEG OMAF group [5] . These results were generated using an HEVC implementation that supports motion constrained tiles – Kvazaar. The only difference between anchor and the other bitstream was the inclusion of the flag ‘--mv-constraint frametilemargin’. The QPs used were 22, 27, 32 and 37.

The main purpose of the test is to evaluate the cost of motion constrained tiles compared to regular tiles in terms of coding efficiency in HEVC. On average, the loss in BD rate is 1,96%, 4,08% and 8.63% for respectively 3x2, 6x4 and 16x8 tiling schemes. For three particular sequences, the effect is more noticeable (CatRobot, DaylightRoad2 and Drums).

Table 2 - BD rates for tiles with no MCTS as an anchor against MCTS

|  | 3x2 | 6x4 | 16x8 |
| --- | --- | --- | --- |
| BuildingHall2_3840x2160 | 0,7% | 1,4% | 3,2% |
| Campfire_3840x2160 | 0,8% | 1,6% | 3,4% |
| CatRobot_3840x2160 | 3,3% | 8,6% | 19,1% |
| DaylightRoad2_3840x2160 | 3,4% | 6,8% | 15,2% |
| Drums_3840x2160 | 3,8% | 7,5% | 15,4% |
| RollerCoaster_4096x2160 | 2,2% | 3,2% | 5,5% |
| Tango_4096x2160 | 1,6% | 3,4% | 6,9% |
| ToddlerFountain_4096x2160 | 0,6% | 1,1% | 2,2% |
| TrafficFlow_3840x2160 | 1,2% | 3,1% | 6,8% |
| AVERAGE | 1,96% | 4,08% | 8,63% |

The results in term of compression efficiency loss for the current proposal for VVC are deemed to be comparable with these costs since the proposed Tile concept for VVC is similar to HEVC MCTS tiles regarding coding constraints.

# Conclusion

The current tile feature in HEVC was originally designed for in-frame parallelisation and not for spatial random access as it is currently being used in many applications. This contribution aims at enabling a true spatial random access for VVC video bitstreams to better satisfy the needs of applications with ultra high resolution (panoramic video, 360 VR video, etc..) as well as scenarios where multiple streams are mixed together such as video conferencing, video surveillance, etc… By bringing the necessary syntax higher up in the bitstream, on the fly alterations can be made quicker and more efficiently. By introducing the concept of hooks and Tiles, it is possible to have heterogenous spatial regions and the positions in the arrangement of the regions is no longer a parameter depending on the block partitioning of the frame.

It is recommended to adopt the current technical proposal for enabling by design in VVC a true spatial random access over several frames.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Alexandre Gabriel, Emmanuel Thomas, "Proposed design of high-level syntax for spatial relation between independent HEVC sub bitstreams", JCTVC-AB0032, Torino, IT, 15–21 July 2017

- Rickard Sjöberg, Mitra Damghanian, Martin Pettersson, Jack Enhorn, "Flexible Tiles", JVET-K0260-v2, Ljubljana, SI, 10–18 July 2018

- ISO/IEC 23090-2, Information technology -- Coded representation of immersive media -- Part 2: Omnidirectional media format

- Y. Sanchez, R. Globisch, T. Schierl and T. Wiegand, "Low Complexity Cloud-video-Mixing Using HEVC".

- ISO/IEC JTC1/SC29/WG11 MPEG2018/M43412, Maryam Homayouni, Miska M. Hannuksela, Alireza Aminlou, Alireza Zare, Kashyap Kammachi-Sreedhar, "[OMAF] Rate-distortion comparison of late binding versus viewport-independent encodings", July 2018, Ljubljana, Slovenia

https://github.com/ultravideo/kvazaar