| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0207-v1 |
| --- | --- |

| Title: | CE4 related: simplified non-sub-block STMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, T. Poirier, F. Galpin 975 Avenue des Champs Blancs, 35576 Cesson-Sévigné | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com tangi.poirier@technicolor.com franck.galpin@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a modified version of the spatial-temporal merge mode, also called non-sub-block STMVP, introduced in JVET-K0532 and studied in test CE4.4.1.

In the proposed method, a reduced number of spatial positions are checked compared to JVET-K0532, to construct the spatial-temporal merge candidate. A single spatial position on the top of the CU and a single position on the left are checked.

This reduced number of checks for spatial neighboring motion vectors is coupled with a slightly modified way to retrieve the co-located temporal motion vector predictor (TMVP) and a modified condition to compute the availability of the STMVP merge candidate.

It is reported that the BD-rate gain obtained over the VTM-2 is 0.44% in RA and 0.1% in LDB. The encoder and decoder runtimes are respectively 100% and 102% in RA.

In some additional data, the proposed STMVP merge mode is combined with some other merge candidate lists studied in CE4. When combined with the HMVP (History-Based Motion Vector Prediction) tool of CE 4.4.7, it reportedly brings 0.97% and 0.83% BD-rate gain in RA, respectively in cases where HMVP is applied to both AMVP + Merge and to Merge mode only.

# Introduction

The goal of this contribution is to design a spatial-temporal MVP merge tool, with a complexity level as small as possible. Therefore, the checks for spatial and temporal candidates has been reduced as much as possible, while preserving the gain of the non-sub-block STMVP of CE4.4.1.

The remaining of this document describes the proposed technique in detail. Then results are provided. Finally, the non-overlapping between the proposed STMVP mode and other merge lists arrangements proposed in CE4 is shown. In particular, the proposed STMVP candidate is introduced into the history-based motion vector prediction of test CE4.4.7, leading to around 1% BD-rate gain over the VTM 2.0.1.

# Proposed technique

In JVET-K0532 and in the CE4.4.1, up to four spatial positions are checked in order to get some neighboring motion vectors to compute the STMVP merge candidate.

In the proposed method, only one top and one left neighboring MV positions are checked to obtain spatial predictors. They correspond to the top-middle and left-middle positions, see Figure 1.

Figure 1: spatial position checked to construct the STMVP merge candidate

In addition, three slight modifications are brought in the STMVP candidate computation.

- The retrieval of the co-located TMVP candidate is slightly modified. The temporal candidate is set as always available, given that the Right-Bottom position of the current block is inside the picture. The y-position of the temporal candidate is clipped to ensure it lies inside the current CTU row. This is illustrated on Figure 2.

Figure 2:temporal predictor used to construct the STMVP candidate. If the bottom-right neighboring 4x4 block of current CU is below the current CTU row, then the 4x4 sub-block on the right of the bottom-right corner of current CU is used.

- The STMVP candidate is considered as available if at least 2 spatial-temporal MV predictors are found. If only one neighboring MV is retrieved, no STVMP candidate is included into the merge list.

- The merge list is ordered a bit differently. The ATMVP and STMVP candidates are placed right after the two first spatial candidates (left and above).

# Experimental results of proposed STMVP candidate

Table 1 provides the summary of BD-rate performances obtained with the proposed STMVP merge mode, in Random Access and Low-Delay. A bitrate saving of 0.44% is obtained in Luma over VTM-2.0.1 in random access, and 0.1% gain is obtained in low delay. This shows the same BD-rate result as test CE4.4.1 (0.44%) can be obtained with fewer tests for spatial MV predictors derivation. In low-delay, a slightly better coding efficiency than test CE4.4.1 is obtained.

Table 1: performances of the proposed STMVP merge mode over VTM-2.0.1

# Complexity analysis

# Combinations with longer merge lists proposed in CE4

## Combination with HMVP (test CE-4.4.7)

Table 2 and Table 3 respectively show the performances obtained when combining the proposed STMVP mode and the HMVP (History-Based Motion Vector Prediction) tool proposed in CE4.4.7.  As can be seen, a gain of 0.83% is obtained when HMVP is applied only in merge mode. The gain is 0.97% if HMVP is applied both in merge and AMVP. This is to be compared with the pure HMVP gains, which are respectively equal to 0.43% and 0.57% when applied to merge mode only and to both merge + AMVP.

These results show the proposed STMVP tool can be combined with some enriched merge list construction methods that are being investigated in CE4. A cumulated BD-rate gain can be easily achieved with such combination.

Table 2: performances over VTM-2.0.1 of the proposed STMVP, combined with HMVP (CE4.4.7)

Table 3: performances over VTM-2.0.1 of the proposed STMVP, combined with HMVP (CE4.4.7)

# Conclusion

In this contribution, a further simplified version of STMVP compared to JVET-K0532 is proposed. The proposed STMVP merge mode reportedly brings 0.44% BD-rate gain in luma in Random Access and 0.1% gain in low-delay. Therefore, equivalent performance to that of JVET-K0532 can be obtained with this simplified version.

Finally, it has been shown that the proposed STMVP mode can be included in some enriched merge candidate lists proposed in CE4, and that further BD-rate gain can be obtained from such combination.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, July 2018.

[2] T. Zhou, T. Ikai (Sharp), “Spatial-temporal merge mode (non subblock STMVP)”, JVET-K0532, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).