| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0207-v1 |
| --- | --- |

| Title: | CE4 related: simplified non-sub-block STMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, T. Poirier, F. Galpin 975 Avenue des Champs Blancs, 35576 Cesson-Sévigné | Tel: Email: | +33(0)299273198 fabrice.leleannec@technicolor.com tangi.poirier@technicolor.com franck.galpin@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a modified version of the spatial-temporal merge mode, also called non-sub-block STMVP, introduced in JVET-K0532 and studied in test CE4.4.1.

In the proposed method, a reduced number of spatial positions are checked compared to JVET-K0532, to construct the spatial-temporal merge candidate. A single spatial position on the top of the CU and a single position on the left are checked. This reduces the number of checks for spatial neighboring motion vectors.

This is coupled with a slightly modified way to retrieve the co-located temporal motion vector predictor (TMVP) and a modified condition to compute the availability of the STMVP merge candidate.

It is reported that the BD-rate gain obtained over the VTM-2 is 0.44% in RA and 0.1% in LDB. The encoder and decoder runtimes are respectively 100% and 102% in RA. With a merge list size equal to 8, a BD-rate gain of 0.6% over the VTM-2 is obtained, using 2 STMVP potential merge candidates.

In some additional data, the proposed STMVP merge mode is combined with some other merge candidate lists studied in CE4. When combined with the HMVP (History-Based Motion Vector Prediction) tool of CE 4.4.7, it reportedly brings 0.83% and 0.97% gain in RA, respectively in cases where HMVP is applied to merge only and to both AMVP and merge.

Finally, the proposed STMVP merge mode has also been tested in combination with the test CE4.4.2+CE4.4.7 (JVET-L0321). It is reported that with a merge list size of 10, up to 1.34% and 1.46% average BD-rate gain can obtained over the VTM-2, respectively when HMVP is applied only in merge, and in both AMVP and merge.

# Introduction

The goal of this contribution is to design a spatial-temporal MVP merge tool, with a complexity level as small as possible. Therefore, the checks for spatial and temporal candidates has been reduced as much as possible, while preserving the gain of the non-sub-block STMVP of CE4.4.1.

The remaining of this document describes the proposed technique in detail. Then results are provided. Finally, the non-overlapping between the proposed STMVP mode and other merge list arrangements proposed in CE4 is assessed. In particular, the proposed STMVP candidate is introduced into the combined test CE4.4.2+CE4.4.7, leading to around 1.4% BD-rate gain over the VTM 2.0.1.

# Proposed techniques

While investigating various versions of the non-sub-block STMVP merge mode, the following observations have been made.

- Most of the coding gain comes from the use of the averaged spatial motion vector predictor.

- The use of the temporal motion vector predictor in the computation of the averaged STMVP predictor brings about 0.1% gain, compared to using only spatial predictors. However, coding gain can be increased by slightly modifying the derivation of the TMVP predictor as explained later.

- The gain of non-sub-block STMVP is not very sensitive to the position of the spatial MV predictors used to compute the STMVP candidate. Nevertheless, the spatial positions that bring the most coding gain are the left-middle and above-middle ones.

- The use of several spatial positions on the left and above the current block is almost useless in terms of coding efficiency. The overall coding gain is very slightly affected by removing the spatial positions “Afar” and “Lfar” used in JVET-K0532 or CE4.4.1.

The two methods presented in the following to derive the STMVP merge candidate are based on these observations.

## Reducing the spatial candidates

In JVET-K0532 and in test CE4.4.1, up to four spatial positions are checked to get some neighboring motion vectors to compute the STMVP merge candidate. In the proposed first method, only one top and one left neighboring MV positions are checked to obtain spatial predictors. They correspond to the top-middle and left-middle positions, see Figure 1.

Figure 1: spatial position checked to construct the STMVP merge candidate

In addition, three slight modifications are brought to the STMVP candidate computation.

- The retrieval of the co-located TMVP candidate is slightly modified. The temporal candidate is set as always available, given that the Right-Bottom position of the current block is inside the picture. The y-position of the temporal candidate is clipped to ensure it lies inside the current CTU row. This is illustrated on Figure 2.

Figure 2:temporal predictor used to construct the STMVP candidate. If the bottom-right neighboring 4x4 block of current CU is below the current CTU row, then the 4x4 sub-block on the right of the bottom-right corner of current CU is used.

- The STMVP candidate is considered as available if at least 2 spatial-temporal MV predictors are found. If only one neighboring MV is retrieved, no STVMP candidate is included into the merge list.

- The merge list is ordered a bit differently. The ATMVP and STMVP candidates are placed right after the two first spatial candidates (left and above).

- To achieve increased coding efficiency, an additional merge candidate can be used in the case of a merge list size equal to 8 or higher. Indeed, with an increased merge list size, there is room for additional potential merge candidates. Moreover, as stated before, we observed that an STMVP mode using only spatial MV predictors gives gain, without using any temporal predictor. Therefore, the proposed additional STMVP candidate consists in an averaged motion vector between the two spatial predictors used to construct the first STMVP candidate. This additional STMVP candidate is used when 3 motion vectors (thus including the temporal predictor) had been used to generate the first STMVP predictor. It is added to the merge list if different from all candidates already present in the list. Experimental results in next section are shown with and without the use of this additional STMVP candidate.

Experimental results of proposed STMVP candidate (1st method)

Table 1 provides the BD-rate performances obtained with the proposed STMVP merge mode, in Random Access and Low-Delay. A bitrate saving of 0.44% is obtained in Luma over VTM-2.0.1 in random access, and 0.1% gain is obtained in low delay. This shows the same BD-rate result as test CE4.4.1 (0.44%) can be obtained with fewer tests for spatial MV predictors derivation. In low-delay, a slightly better coding efficiency than test CE4.4.1 is obtained.

Table 1: performances of the proposed STMVP merge mode over VTM-2.0.1. Merge list size = 6.

Table 2 shows the BD-rate performances obtained with a merge list size equal to 8: 0.54% in RA, 0.16% gain in LDB. Again, very similar gain over VTM-2.0.1 as CE4.4.1 is obtained.

Table 2: performances of the proposed STMVP merge mode over VTM-2.0.1 (merge list size =8)

Table 3 shows the BD-rate gain obtained over VTM-2.0.1 with a merge list size equal to 8 and two STMVP candidates used. Increased coding efficiency is obtained compared to the previous test with 1 STMVP candidate, and over test CE4.4.1.

Table 3: performances of the proposed STMVP merge mode over VTM-2.0.1 (merge list size =8) with one additional merge candidate corresponding to the average of spatial candidates

## Sharing candidates with other Merge modes (2nd STMVP method)

A further simplified version of STMVP has been tested to further simplify the design. It consists in the two following aspects.

- The spatial MV predictor positions A1 and B1 (see Figure 3) are used instead of Amid and Lmid. This allows using the same predictors as in the usual merge mode, thus further reduces the number of checks to retrieve spatial MV predictors.

- The modified TMVP predictor derivation, described in section 2.1, is also applied in the usual TMVP merge mode. This allows sharing the same temporal predictor in STMVP and TMVP merge modes. Thus, this reduces the number of temporal predictors from 2 to 1 in the worst-case scenario of the decoding process.

Figure 3: spatial MV predictors A1 and B1 commonly used in the further simplified STMVP and as usual merge candidates.

## Experimental results of the second STMVP method

Table 4 shows the BD-rate performances obtained with the further simplified STMVP design when the merge list size is 6. One sees very limited loss compared to the version where middle top and left positions are considered for spatial MVPs.

Table 4: performances of the proposed STMVP merge mode over VTM-2.0.1, when using A1 and B1 spatial candidates, and same temporal candidate as TMVP. Merge list size = 6.

Table 5 shows the results obtained with the simplified STMVP design, when the merge list size is 8. A BD-rate gain of 0.52% is obtained over the VTM-2.

Table 5: performances of the proposed STMVP merge mode over VTM-2.0.1, when using A1 and B1 spatial candidates, and same temporal candidate as TMVP. Merge list size = 8.

Table 6 shows the performances of the simplified STMVP mode when the merge list size is 8 and 2 STMVP potential candidates are used. In this configuration, 0.XX% gain is obtained over VTM-2.

Table 6: performances of the proposed STMVP merge mode over VTM-2.0.1, when using A1 and B1 spatial candidates, and same temporal candidate as TMVP, and 2 STMVP candidates. Merge list size = 8.

# Complexity analysis

Table 7 shows the complexity figures of CE4, evaluated on the 2 presented solutions. Two sets of values are provided.

The first corresponds to the lines labelled “Middle Top and Left”. They provide the performances of the 1st STMVP method described in section 2. As can be seen, in the worst-case scenario of the decoding process, one additional temporal MVP is computed compared to the VTM-2.

The 2nd STMVP method corresponds to the lines labelled “Middle Top and Left”. They provide the performances of the 2nd STMVP mode proposed in section 2.2. As can be seen, no additional temporal MVP is used here. Moreover, the only additional complexity compared to the VTM-2 is an increased number of potential merge candidates, by one or two. This comes with a number of comparisons increased to 21 or 27, depending on the total number of potential candidates.

Table 7: CE4 complexity figures of JVET-L0207 compared to CE4.4.1. “Middle Top and Left”: different STMVP versions described in section 2.1. “Middle Top and Left++”: STMVP mode with two added merge candidates. “TRBL+TMVP-Clip”: STMVP modes described in section 2.2. “TRBL+TMVP-Clip++”: STMVP modes described of section 2.2 with 2 added merge candidates.

# Combinations with longer merge lists proposed in CE4

## Combination with HMVP (test CE-4.4.7)

Table 8 and Table 9 show the performances obtained when combining the proposed STMVP mode and the HMVP (History-Based Motion Vector Prediction) tool proposed in CE4.4.7.  As can be seen, a gain of 0.83% is obtained when HMVP is applied only in merge mode. The gain is 0.97% if HMVP is applied both in merge and AMVP. This is to be compared with the pure HMVP gains, which are respectively equal to 0.43% and 0.58% in these two configurations.

These results show the proposed STMVP tool can be combined with some enriched merge list construction methods currently investigated in CE4. A cumulated BD-rate gain can be easily achieved with such combination.

Table 8: performances over VTM-2.0.1 of the proposed STMVP, combined with HMVP (CE4.4.7) HMVP is applied in merge only. Merge list size is 6.

Table 9: performances over VTM-2.0.1 of the proposed STMVP, combined with HMVP (CE4.4.7). HMVP is applied in AMVP. Merge list size is equal to 6.

## Combination with Long-Distance Merge + HMVP (CE-4.4.2+CE4.4.7 = JVET-L0321)

Table 10, Table 11 and Table 12 show the performances obtained when combining the proposed STMVP mode and the package CE4.4.2+CE4.4.7 proposed in JVET-L0321. The whole combination has been tested with a merge list size equal to 10. The maximum number of search rounds is 2.

As can be seen, when one STMVP candidate is used, 1.27% gain is obtained when HMVP is applied only in merge mode. 1.40% gain is obtained wen HMVP applies to both merge and AMVP. This is to be compared with the pure HMVP gains, which are respectively equal to 0.88% and 1.02 % when applied to merge mode only and to both merge and AMVP.

In the case of 2 STMVP candidates, X.XX% and 1.46% gain is obtained, respectively in these 2 configurations.

These results show the proposed STMVP tool can be advantageously combined with the package proposed in JVET-L0321, since STMVP and JVET-L0321 are not overlapping in terms of gain.

Table 10:RA performances of JVET-L0321+STMVP with Merge list size = 10 and MaxSearchRound=2. HMVP is applied in merge only.

Table 11: RA performances of JVET-L0321+STMVP with MaxSearchRound=2. HMVP is applied in merge and AMVP.

Table 12: RA performances of JVET-L0321+STMVP with MaxSearchRound=2 and 2 STMVP candidates. HMVP is applied in merge only.

Table 13: RA performances of JVET-L0321+STMVP with MaxSearchRound=2 and 2 STMVP candidates. HMVP is applied in merge and AMVP.

Finally, the combination of test CE4.4.2+CE4.4.7 with the second STMVP method proposed in section 2.2 has been tested. The obtained results are reported in Table 14 and Table 15.

Table 14: combination of 2nd proposed STMVP method with CE4.4.2+CE4.4.7. 2 STMVP candidates are used. HMVP used only in merge.

Table 15: combination of 2nd proposed STMVP method with CE4.4.2+CE4.4.7. 2 STMVP candidates are used. HMVP used in AMVP and merge.

# Conclusion

In this contribution, 2 simplified versions of STMVP compared to JVET-K0532 are proposed. The first one gives 0.44% BD-rate gain in Random Access and 0.1% gain in low-delay. Therefore, equivalent performance to that of JVET-K0532 can be obtained with this simplified version.

The second one is further simplified, in the sense that it does not uses any new motion vector predictor candidate compared to the usual merge mode. It achieves 0.42% gain over the VTM-2 in RA.

Finally, it has been shown that the proposed STMVP mode can be included in some enriched merge candidate lists proposed in CE4, and that further BD-rate gain can be obtained from such combination. The combination of the proposed first STMVP mode provides 1.27% or 1.4% gain over VTM-2, when combined with JVET-L0321, respectively when HMVP is active in merge and in AMVP + merge. When 2 STMVP candidates are used, 1.34% and 1.45% gain is obtained in these 2 configurations.

All STMVP-related tests made in this contribution are listed in Table 16. The preferred trade-offs between coding efficiency and the amount of modifications to the VTM decoding process are highlighted in red. They correspond to the second STMVP method proposed, where same MV predictors are used between STMVP and other spatial and temporal merge candidates.

Table 16: overview of test driven in this contribution. The preferred trade-off between coding efficiency and added complexity are highlighted in red.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, July 2018.

[2] T. Zhou, T. Ikai (Sharp), “Spatial-temporal merge mode (non subblock STMVP)”, JVET-K0532, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).