| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0213 |
| --- | --- |

| Title: | CE15-related: Combination of palette mode and intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jicheng An, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This document proposes a method combining palette mode and intra prediction. On top of CE15.1, for Class F sequences, in AI/RA/LDB configuration, the results show -0.2%/-0.2%/-0.1% BD-rate luma gain; for 4:2:0 TGM sequences in CE15, the results show -0.7%/-0.4%/-0.1% BD-rate luma gain.

Introduction

This document proposes a method combining palette mode and intra prediction. In HEVC SCC palette mode, a flag is transmitted for each CU to signal whether the palette mode is used in the current CU. If a block is encoded by the palette mode, it will be decoded independently without using any prediction method as Figure 1.

Figure 1 Palette mode in HEVC SCC

To take benefit from both palette mode and traditional prediction methods, this document proposes a method combining palette mode and intra prediction.

Description of techniques

In the proposed combination, the decoder first derives the prediction block based on the intra prediction methods. Then, the coder decodes a palette and an index map. Using the decoding palette information, the decoder refines the prediction block and derives the reconstructed block.

Figure 2 shows an example. Compared with HEVC palette mode, the proposed method combines the conventional intra prediction methods and the palette method.

Figure 2 Proposed method

Figure 3 shows an example combining palette information and the intra prediction information. First, the decoder generates the prediction block (with pixel values A0~A15). And, the decoder decodes index map in the palette mode. If the decoded indices equal to “0”, the corresponding pixels are marked as “P” in Figure 3. The pixels marked as “P” is reconstructed by pixel values in the intra prediction block. Otherwise, the pixels are reconstructed by palette colors, e.g. C0, C1, as HEVC palette mode.

# Figure 3.

Simulation results

The proposed method is evaluated on top of CE 15.1 according to the CE15 description.

Conclusion

This document proposes a method combining palette mode and intra prediction. On top of CE15.1, for Class F sequences, in AI/RA/LDB configuration, the results show -0.2%/-0.2%/-0.1% BD-rate luma gain; for 4:2:0 TGM sequences in CE15, the results show -0.7%/-0.4%/-0.1% BD-rate luma gain.

Reference

- Y.-C. Sun, Y.-H. Chao and X. Xu, "Description of Core Experiment 15: Palette Mode”, JVET-K1035, 2018.

Patent rights declaration(s)

Alibaba Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).