| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0215 |
| --- | --- |

| Title: | CE9: Report on the results of tests CE9.1.1, CE9.2.1, CE9.2.2 and CE9.2.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik Anand Meher Kotra Biao Wang Han Gao Jianle Chen | Tel: Email: | semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution document reports the results of the CE tests CE9.1.1, CE9.2.1, CE9.2.2 and CE9.2.3. The following aspects are tested in order to reduce the complexity of DMVR in the current BMS2.1.

- CE9.1.1: The refined MVs after application of DMVR are used also in the deblocking process.

- CE9.2.1: Integer samples (without interpolation) are used in DMVR search to eliminate intermediate interpolation filtering

- CE9.2.2: Reference block samples are padded in order to eliminate the additional external memory access.

- CE9.2.3: combination of CE9.2.1 and CE9.2.2

It is reported that compared to BMS2.1rc1 anchor with DMVR on;

- CE9.1.1 shows that usage of refined motion vectors for deblocking process provides -0.03% coding gain with no impact on encoding decoding time.

- CE9.2.2 eliminates the additional external memory buffer access completely with -0.01% coding gain and no impact on encoding and decoding times.

- CE9.2.3 eliminates all additional interpolation filtering and additional external buffer access with 0.24% coding loss and 5% reduction in decoding time.

# Introduction

In current BMS2.1 DMVR, 8-tap DCTIF interpolation filtering is used in order to generate predictions from forward and backward direction for refinement search, and a search range of 2 pixels are used.

Once the MV refinement process is over, the refined motion vectors are stored in the temporal MV buffer for later temporal MV prediction, whereas the non-refined (initial) motion vectors are used in deblocking filtering and later spatial MV prediction.

The contribution presents methods to completely eliminate the additional external memory access as well as the need to apply interpolation filtering before DMVR search. Additionally, results for using refined motion vectors also for deblocking is reported.

# Problem Statement

DMVR in BMS2.1 implements 8-tap DCTIF interpolation filtering to generate the predictions for refinement search. Due to this reason the number of multiplication operations per pixel is increased, although the implementation of interpolation filtering can be very efficient. Moreover the search range of DMVR, which is 2 pixels in up/down and left/right directions increase the external memory access requirement to perform DMVR.

# Proposed Method

The contribution reports results for CE9.2.2 and CE9.2.3 that completely eliminate the above mentioned two problems by using reference sample padding and usage of integer reference samples instead of 8-tap DCTIF interpolation for DMVR search. Additionally the effect of using refined MV in deblocking is reported.

## CE9.1.1

The test CE9.1.1 is designed to test the effect of using refined motion vectors for deblocking process. In the current BMS2.1, the refined motion vectors after DMVR are used only in the temporal motion vector prediction. According to test CE9.1.1, the refined MVs are used also for deblocking process.

In the test CE9.1.1 only the effect of using the refined MV in deblocking, hence this test does not include reference sample padding or integer sample based DMVR search. As part of the first sub-test of CE9, the test CE9.1.1 can be considered separately to the tests in the subtest 2.

## CE9.2.1

In CE test 9.2.1, the 8-tap interpolation filter is eliminated and instead integer reference samples are used for MRSAD comparison process of DMVR. The integer reference samples are obtained by rounding the initial motion vector to the nearest integer sample coordinate before DMVR. This means that no interpolation filtering is necessary until the final refined motion vectors are obtained. Only at this step (after the final refined MV is obtained), 8-tap interpolation filtering is applied to obtain the final motion compensation results. It is noted that the final motion vector can still have fractional sample resolution, since the MV rounding operation is reverted before final motion compensated prediction.

The process is elaborated in the following figure:

Figure 1: Application of rounding before MRSAD search and inversion of the rounding operation in order to obtain refined MV.

## CE9.2.2

In this test padding is applied in order to extend the reference sample block that is pointed by the initial motion vector. If the size of the coding block are given by “w” and “h”, then it is assumes that a block of size w+7 and h+7 is retrieved from the reference picture buffer. The retrieved buffer is then extended by 2 samples in each direction by repetitive sample padding using the nearest sample. Afterwards the extended reference sample block is used to generate the final prediction once the refined motion vector is obtained (which can deviate from the initial motion vector 2 samples in each direction).

It is noted that this modification eliminates the external memory access requirement of DMVR completely without any coding loss. The experimental results are reported in the following section.

## CE9.2.3

The reference sample padding (CE9.2.2) and integer DMVR (CE9.2.1) are applied together.

# Test conditions and anchors

In CE9 the BMS2.1 release candidate 1 is used as anchor software due to the fact that the release of BMS2.1 is delayed. The SIMD setting is AVX2 in both anchor and test.

# Simulation Results

Results for 2 sets of simulations are provided.

Simulation results 1:

- Anchor 1: BMS2.1rc1 VTM encoder configuration

- Test: Anchor + proposal.

Simulation results 2:

- Anchor 2: BMS2.1rc1 VTM encoder configuration + DMVR on

- Test: Anchor + proposal.

## Experimental results for CE9.2.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 – with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.71% | -1.56% | -1.87% | 104% | 109% |
| Class A2 | -2.08% | -2.27% | -2.38% | 103% | 109% |
| Class B | -1.03% | -1.22% | -1.25% | 104% | 114% |
| Class C | -1.06% | -1.33% | -1.31% | 104% | 117% |
| Class E |  |  |  |  |  |
| Overall | -1.39% | -1.53% | -1.61% | 104% | 113% |
| Class D | -1.07% | -1.39% | -1.44% | 105% | 117% |

Table 1: simulation results for CE9.2.2 compared to Anchor 1 (BMS2.1rc1 VTM encoder configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config-DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.01% | 0.05% | 100% | 95% |
| Class A2 | 0.25% | 0.19% | 0.15% | 100% | 94% |
| Class B | 0.29% | 0.26% | 0.29% | 99% | 93% |
| Class C | 0.34% | 0.29% | 0.39% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.25% | 0.20% | 0.24% | 100% | 93% |
| Class D | 0.69% | 0.36% | 0.41% | 99% | 93% |
| Class F (optional) |  |  |  |  |  |

Table 2: simulation results for CE9.2.2 compared to Anchor 2 (BMS2.1rc1 VTM encoder configuration + DMVR on)

## Experimental results for CE9.2.2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 – with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.78% | -1.57% | -1.94% | 105% | 114% |
| Class A2 | -2.32% | -2.42% | -2.52% | 104% | 114% |
| Class B | -1.33% | -1.46% | -1.50% | 106% | 121% |
| Class C | -1.40% | -1.66% | -1.69% | 105% | 123% |
| Class E |  |  |  |  |  |
| Overall | -1.64% | -1.73% | -1.84% | 105% | 118% |
| Class D | -1.73% | -1.78% | -1.83% | 105% | 123% |

Table 1: simulation results for CE9.2.2 compared to Anchor 1 (BMS2.1rc1 VTM encoder configuration)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config-DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | -0.02% | 100% | 99% |
| Class A2 | 0.00% | 0.03% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.02% | 0.04% | 100% | 100% |
| Class C | -0.01% | -0.04% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.03% | 0.02% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2: simulation results for CE9.2.2 compared to Anchor 2 (BMS2.1rc1 VTM encoder configuration + DMVR on)

## Experimental results for CE9.2.3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -1.55% | -1.89% | 103% | 109% |
| Class A2 | -2.08% | -2.30% | -2.34% | 102% | 110% |
| Class B | -1.04% | -1.28% | -1.35% | 104% | 114% |
| Class C | -1.06% | -1.29% | -1.34% | 104% | 116% |
| Class E |  |  |  |  |  |
| Overall | -1.39% | -1.54% | -1.65% | 104% | 113% |
| Class D | -1.08% | -1.48% | -1.43% | 105% | 116% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 3: simulation results for CE9.2.3 compared to Anchor 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.01% | 0.03% | 99% | 96% |
| Class A2 | 0.25% | 0.15% | 0.19% | 99% | 96% |
| Class B | 0.28% | 0.20% | 0.18% | 99% | 94% |
| Class C | 0.34% | 0.33% | 0.35% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.24% | 0.19% | 0.20% | 99% | 95% |
| Class D | 0.68% | 0.27% | 0.42% | 100% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 4: simulation results for CE9.2.3 compared to Anchor 2

## Experimental results for CE9.1.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.81% | -1.54% | -1.90% | 104% | 115% |
| Class A2 | -2.36% | -2.44% | -2.54% | 103% | 115% |
| Class B | -1.33% | -1.48% | -1.52% | 105% | 121% |
| Class C | -1.42% | -1.61% | -1.72% | 104% | 122% |
| Class E |  |  |  |  |  |
| Overall | -1.66% | -1.72% | -1.85% | 104% | 119% |
| Class D | -1.75% | -1.83% | -1.85% | 105% | 123% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 5: simulation results for CE9.1.1 compared to Anchor 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config-DMVR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | -0.03% | 0.01% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.02% | 100% | 100% |
| Class C | -0.03% | 0.00% | -0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.08% | -0.01% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 6: simulation results for CE9.1.1 compared to Anchor 2

# Conclusion

This contribution document reports the results of the CE tests CE9.1.1, CE9.2.1, CE9.2.2 and CE9.2.3. It is reported that significant reduction can be achieved with tests CE9.2.2 and CE9.2.3 with no impact in coding gain and a small reduction in coding gain respectively. Additionally the results of CE9.1.1 show that the usage of refined motion vectors in deblocking process provides a small gain with respect to using the refined MV only as TMVP candidate. It is noted that the modifications proposed in CE9.1.1 and CE9.2.3 are orthogonal to each other and were considered in different subtests. Therefore it is proposed to include both CE9.2.3 and CE9.1.1 in VVC.

# Reference

[1] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, August 2018

[2] “ Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, S. Esenlik, Y.-W. Chen, F. Chen, doc. JVET-K1029, August 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).