| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0218 |
| --- | --- |

| Title: | Non-CE1: Overriding QT/BT/TT Split Constraint Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Jianle Chen Biao Wang Anand Meher Kotra | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-L0382 Simplified Fixed sub-block size and restriction for ATMVP. The simulations were carried out based on the common test condition (CTC). Based on the partial results, the test results are matched.

# Introduction

In the current VVC draft 2 design, the QT/BT/TT split constraint syntax elements are signaled separately in SPS and Slice header, which losses the flexibility controlling of the trade-off between splitting complexity and coding gain. The proposed method provides the overriding function of the QT/BT/TT split constraint syntax elements to address this problem.

# Problem Statement

In current VVC draft 2 [1], there is three QT/BT/TT split constraint syntax elements signaled, namely

- MinQtSizeY to specify the minimum luma size of a leaf block resulting from quadtree splitting of a CTU, which is indicated by log2_min_qt_size_intra_slices_minus2 and log2_min_qt_size_inter_slices_minus2 in the SPS ;

- MaxMttDepth to specify the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf, which is indicated by max_mtt_hierarchy_depth_inter_slices and max_mtt_hierarchy_depth_intra_slices in the SPS;

- MaxBtSizeY to specify maximum luma size (width or height) of a coding block that can be split using a binary split which is indicated by log2_diff_ctu_max_bt_size in the Slice header;

By current design, the QT/BT/TT split constraint syntax elements MinQTSizeY and MaxMttDepth are signalled in the SPS for each slice type (I-Slice and B/P-Slice) and cannot be controlled in individual slices. In the meantime the MaxBtSizeY is signalled only in the slice header and not in the SPS.

It is asserted that the current design has following problems:

- The constraints MinQtSizeY and MaxMttDepth are tree syntax elements that control that have a very high encoding time impact. Depending on the scene characteristics and encoding time limitations it would be desirable to control these syntax elements at a finer granularity than the SPS.

- MaxBtSizeY is signalled in the slice header and the MaxMttDepth is signalled in the SPS. In most scenarios we envision that these two parameters need to be adjusted together, so it seems to us that changing only the MaxBtSizeY in the slice header is an incomplete solution. In fact the MaxMttDepth is the syntax element that has more impact on the encoding time.

# Proposed Solution

In the proposed solution:

- MaxBtSizeY is signalled in the SPS along with MinQtSizeY and MaxMttDepth. It is indicated with two syntax elements sps_log2_diff_ctu_max_bt_size_inter_slices and sps_log2_diff_ctu_max_bt_size_intra_slices controlling the maximum binary split size individually for intra and inter slices.

- One sps_override_split_constraints_enable_flag is proposed to be signaled in SPS, to enable overriding of the parameters MinQtSizeY, MaxMttDepth and MinQtSizeY in the slice header.

- One slice header flag split_constraints_override_flag is proposed to be included to facilitate overriding MinQtSizeY, MaxMttDepthY and MaxBtSizeY in the slice header.

- If the split_constraints_override_flag is true;

Overriding the parameters MinQtSizeY, MaxMttDepthY and MaxBtSizeY in the slice header by signalling the syntax elements: log2_diff_ctu_max_bt_size, log2_min_qt_size_minus2 and max_mtt_hierarchy_depth.

–	Otherwise;

Inferring the split constraint parameters MinQtSizeY, MaxMttDepthY and MaxBtSizeY to be equal to the corresponding values signaled in the SPS based on slice type.

It is noted that the syntax elements that can be overridden in the slice header can be extended to the syntax elements controlling the split constraints for chroma channel in the case of dual tree mode and the MaxTtSizeY, which is noted to be hardcoded according [1].

# Syntax Modifications

Syntax modifications are provided as attachment to this contribution document.

# Conclusion

A mechanism is proposed to enable overriding of the split constraint syntax elements in the slice header. It is asserted that the proposed mechanism provides more flexibility for encoders to adapt to the scene characteristics and to encoding time limitations. It is proposed to be included in next VVC draft text.

# References

[1] “ Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, July 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).