| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0223-v1 |
| --- | --- |

| Title: | CE4.6: Intra and Inter/Intra Boundary Padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jens Brandenburg, Robert Skupin, Heiko Schwarz, Detlev Marpe, Thomas Schierl, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany | Email: | robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In HEVC, reference pictures are extended by a perpendicular padding of the picture boundary samples. CE4.6 investigates further methods for boundary padding which rely either on inter-prediction (JVET- CE4.6.1), intra-prediction (CE4.6.2) or a combination thereof (CE4.6.3).

This document reports results for CE4.6.2 (intra-based padding) and CE4.6.3 (inter/intra based padding) when applied under CTC. Reported experimental results show BD-rate gains in RA and LD configurations.

For CE4.6.3 VTM 2.0.1 RA overall (Class A1 to C), -0,24% luma BD-rate gain at 109% combined Enc/Dec runtime are reported. For small video resolutions such as Class D, luma BD-rate gains of -0.61% at 125% combined Enc/Dec runtime for RA are reported. For CE4.6.2, the gains in RA configuration are considerably smaller.

In addition, this document reports results for padding on motion constrained tile boundaries, for which a 6x4 division into a rectangular tile grid was applied. The anchor in this comparison are motion constrained tiles with traditional perpendicular padding on tile boundaries. For CE4.6.3 VTM 1.0 RA (Class A1 and A2) -0,91% luma BD-rate gain at 121% combined Enc/Dec runtime are reported. Again, for CE4.6.2, the gains in RA configuration are considerably smaller.

# Introduction

In HEVC, reference pictures are extended by a perpendicular padding of the picture boundary samples. CE4.6 in [1] investigates further methods for boundary padding which rely either on inter-prediction (CE4.6.1 intra-prediction (CE4.6.2) or a combination thereof (CE4.6.3).

Inter-prediction based boundary padding in CE4.6.1 as proposed in [2] uses motion compensated prediction to extend the area of the reference picture. The boundary extension area is divided into blocks of 4xM or Mx4 samples. Each block is filled by motion compensation using the motion information of the adjacent reference picture block. For boundary extensions blocks without associated motion information and for boundary extension areas for which motion information points outside of the reference picture, fallback perpendicular padding is carried out.

Intra-prediction based boundary padding in CE4.6.2 as introduced in [3] uses angular intra-prediction to fill the area of a referenced block outside the reference picture. The applied angular intra-prediction mode is chosen in encoder and decoder using a probing approach of decoded picture samples.

The harmonized inter/intra-prediction based boundary padding in CE4.6.3 as introduced in [4] combines per-picture inter-prediction based boundary padding (the particular flavor introduced in JVET-K0363 [5]) with per-reference intra-prediction based boundary padding as in CE4.6.2. After generation of the inter-prediction based boundary padding, for each reference block entailing boundary padding samples, the number of boundary padding samples originating from perpendicular boundary padding is evaluated. If this number exceeds a threshold (50% of boundary padding samples), intra-prediction based boundary padding is used for the reference block instead.

# Simulation Results

In this section, experimental results in terms of Bjøntegaard Delta (BD) bit rate savings [6] and Encoder Decoder runtimes are presented. Simulation runs are based on test sets and configurations specified in the JVET common test conditions [7]. Simulation results with VTM 2.0.1 as reference have been generated for the following boundary padding approaches:

- CE4.6.2: intra-prediction based boundary padding

- CE4.6.3: proposed harmonization of JVET-K0363 [5] and CE4.6.2

## VTM 2.0.1 CTC configuration

### CE4.6.2

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | -0.11% | 102% | 102% |  |  |  |  |  |
| Class A2 | -0.01% | -0.10% | -0.04% | 103% | 101% |  |  |  |  |  |
| Class B | -0.05% | -0.10% | -0.08% | 102% | 105% | -0.04% | -0.04% | 0.01% | 105% | 107% |
| Class C | -0.08% | 0.00% | -0.13% | 105% | 111% | -0.04% | -0.08% | 0.08% | 108% | 108% |
| Class E |  |  |  |  |  | -0.14% | -0.06% | -0.20% | 100% | 100% |
| Overall | -0.05% | -0.06% | -0.09% | 103% | 105% | -0.07% | -0.06% | -0.02% | 105% | 106% |
| Class D | -0.16% | -0.12% | -0.25% | 109% | 112% | -0.50% | -0.39% | -0.98% | 116% | 116% |
| Class F (optional) | -0.03% | -0.01% | 0.04% | 102% | 104% | -0.25% | 0.35% | -0.27% | 103% | 101% |

### CE4.6.3

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.12% | -0.17% | 100% | 103% |  |  |  |  |  |
| Class A2 | -0.16% | -0.13% | -0.16% | 101% | 102% |  |  |  |  |  |
| Class B | -0.26% | -0.18% | -0.26% | 101% | 109% | -0.03% | -0.12% | -0.05% | 105% | 108% |
| Class C | -0.29% | -0.15% | -0.31% | 104% | 121% | -0.07% | -0.04% | -0.24% | 107% | 113% |
| Class E |  |  |  |  |  | -0.29% | -0.64% | -0.64% | 99% | 102% |
| Overall | -0.24% | -0.15% | -0.24% | 102% | 110% | -0.11% | -0.23% | -0.26% | 104% | 108% |
| Class D | -0.61% | -0.42% | -0.52% | 106% | 128% | -0.49% | -1.16% | -1.00% | 115% | 125% |
| Class F (optional) | -0.07% | -0.07% | -0.02% | 102% | 112% | -0.17% | 0.17% | 0.30% | 103% | 107% |

## VTM 1.0 Tiles configuration

Tiles configuration is used for Class A1 and A2 sequences. Each input yuv sequence is split in 6x4 motion constrained tiles each with a size of 640x512 luma samples (5x4 CTU’s). The size of the last line of tiles is set to 640x624 luma samples. Tile boundaries are padded for the motion compensation. The anchor in this comparison uses motion constrained tiles with traditional perpendicular padding on tile boundaries.

### CE4.6.2

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CampfireParty2 | -0.03% | -0.13% | -0.15% | 101% | 102% | -0.11% | -0.41% | -0.21% | 105% | 103% |
| CatRobot1 | -0.28% | -0.32% | -0.29% | 104% | 104% | -0.28% | -0.20% | -0.17% | 110% | 106% |
| DaylightRoad2 | -0.31% | -0.37% | -0.32% | 107% | 105% | -0.18% | 0.65% | -0.21% | 113% | 106% |
| FoodMarket4 | -0.34% | -0.47% | -0.53% | 107% | 104% | -0.61% | -0.37% | -0.22% | 118% | 108% |
| ParkRunning3 | -0.18% | -0.20% | -0.24% | 108% | 105% | -0.18% | -0.17% | -0.08% | 115% | 108% |
| Tango2 | -0.49% | -0.51% | -0.50% | 105% | 108% | -0.49% | -0.23% | -0.24% | 113% | 107% |
| Overall | -0.27% | -0.33% | -0.34% | 105% | 105% | -0.31% | -0.12% | -0.19% | 112% | 106% |

### CE4.6.3

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CampfireParty2 | -0.13% | -0.15% | -0.25% | 104% | 114% | -0.05% | -0.08% | 0.09% | 105% | 111% |
| CatRobot1 | -0.72% | -0.70% | -0.76% | 106% | 122% | -0.41% | -0.37% | -0.22% | 109% | 114% |
| DaylightRoad2 | -1.15% | -1.08% | -1.36% | 106% | 124% | -0.27% | -0.18% | 0.38% | 112% | 117% |
| FoodMarket4 | -1.71% | -1.67% | -1.80% | 106% | 128% | -0.59% | -0.10% | -0.22% | 117% | 118% |
| ParkRunning3 | -0.73% | -0.70% | -0.73% | 108% | 124% | -0.19% | -0.18% | -0.13% | 113% | 113% |
| Tango2 | -1.01% | -0.92% | -1.16% | 108% | 133% | -0.43% | -0.20% | -0.21% | 112% | 116% |
| Overall | -0.91% | -0.87% | -1.01% | 106% | 124% | -0.32% | -0.19% | -0.05% | 111% | 115% |

# References

- H. Yang, S. Liu and K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding“ JVET, doc JVET-K1024, July 2018.

- M. Park, M. W. Park, W. Choi, C. Kim (Samsung), “CE4: Reference Picture Boundary Padding in JVET-J0025 (Test 4.5.3) “, JVET-K0117, July 2018

- M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand and M. Winken, “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI,“ JVET, doc. JVET-J0014, April 2018

- J. Brandenburg, R. Skupin, H. Schwarz, D. Marpe, T. Schierl, T. Wiegand, „CE4-5 related: Inter/Intra Boundary Padding“, JVET, doc. JVET-K0195, July 2018

- Y. Zhang, Y. Han, C.-C. Chen, C.-H. Hung, W.-J. Chien and M. Karczewicz “CE4.5.2: Motion compensated boundary pixel padding“ JVET, doc JVET-K0363, July 2018

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations,” JVET, doc. JVET-K1010, July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).