| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0228-r1 |
| --- | --- |

| Title: | CE13: HEC with 8 samples padding around face row (Test 1.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 1.1.b. The padding is increased from 4 samples to 8 samples around each face row. Results based on end-to-end WS-PSNR show that HEC with 8 samples padding yields {Y, U, V} BD-rate loss of {0.70%, 0.58%, 0.68%} and {1.01%, 0.12%, 0.08%} over PHEC and HEC, respectively, for VTM configuration.

# Introduction

Face discontinuities in the frame packed picture may yield to seam artifacts in reconstructed viewports. One of the goals of CE13 is to study removing subjective artifacts at face boundaries using non-normative pre- and post-processing. Different amount of padding, blending, and post-filtering of seam artifacts are tested. This contribution reports the coding performance of HEC with 8 samples padding around face row, as studied in CE13 test 1.1.b [1].

# Technical description

HEC with 8 samples padding around face row and blending. Please refer to JVET-K1004 [2] for details.

# Simulation results

The software is implemented based on BMS-2.0.1 (with fix BMS ticket #78 and #79) and 360Lib-7.0. The encoding was done under 360° video CTC [3]. The following table only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for HEC with 8 samples padding around face row over PHEC and HEC are reported in Table 1.

Table 1. BD-rate for HEC with 8 samples padding around face row over PHEC and HEC.

|  |  | PHEC |  |  | HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.53% | 0.41% | 0.06% | 0.50% | -0.59% | -1.08% |
|  | ChairLift | 0.63% | 0.65% | 0.91% | 0.74% | 0.41% | 0.62% |
|  | KiteFlite | 0.74% | 0.62% | 0.85% | 1.19% | 0.49% | 0.47% |
|  | Harbor | 0.63% | 0.13% | 0.17% | 0.60% | -1.37% | -1.67% |
|  | Trolley | 0.95% | 0.81% | 0.61% | 1.61% | 0.32% | 0.52% |
|  | GasLamp | 0.81% | 1.03% | 0.90% | 1.29% | 0.66% | 0.43% |
| Class S2 | Balboa | 0.59% | 0.24% | 1.00% | 0.71% | 0.20% | 0.53% |
|  | Broadway | 0.55% | 0.37% | 0.58% | 0.62% | 0.27% | 0.02% |
|  | Landing2 | 0.69% | 0.56% | 0.76% | 1.19% | -0.48% | -0.09% |
|  | BranCastle2 | 0.88% | 0.94% | 1.02% | 1.70% | 1.30% | 1.05% |
| Class S1 |  | 0.71% | 0.61% | 0.58% | 0.99% | -0.01% | -0.12% |
| Class S2 |  | 0.68% | 0.53% | 0.84% | 1.05% | 0.32% | 0.38% |
| Overall |  | 0.70% | 0.58% | 0.68% | 1.01% | 0.12% | 0.08% |
| Enc Time[%] |  | 100% |  |  | 103% |  |  |
| Dec Time[%] |  | 98% |  |  | 98% |  |  |

BD-rate loss over PHEC and HEC are around {0.70%, 0.58%, 0.68%} and {1.01%, 0.12%, 0.08%}, respectively. The complexity of HEC with 8 samples padding around face row is similar to that of PHEC and HEC.

# Conclusion

This contribution reports simulation result of CE13 test 1.1.b, which evaluates HEC with 8 samples padding around face row. The results show BD-rate loss of {0.70%, 0.58%, 0.68%} and {1.01%, 0.12%, 0.08%} over PHEC and HEC, respectively, for VTM configuration.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 7)”, JVET-K1004, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.