| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0231-r2 |
| --- | --- |

| Title: | CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 3.3. For inter prediction, horizontal geometry padding of reference pictures is used over conventional repetitive padding at the left and right boundaries for the padded equirectangular projection (PERP). Results based on end-to-end WS-PSNR show that PERP with horizontal geometry padding of reference pictures provides {Y, U, V} BD-rate savings of {-0.22%, -0.29%, -0.38%} over PERP for VTM configuration at a similar complexity as repetitive padding. More importantly, it is reported that horizontal geometry padding significantly reduces the seam artifact issue for the ChairLift sequence.

r2 version: Syntax has been changed to address the comments made during the JVET plenary on Wednesday, October 10. Additionally, the horizontal clipping function has been modified to avoid defining a new modulo operator for negative numbers and to use the existing modulo operator, which is defined only for non-negative numbers. The existing luma and chroma sample interpolation process clauses have been modified instead of defining new clauses.

# Introduction

The PERP format was shown to effectively reduce the seam artifacts in reconstructed viewports that encompass the left and right boundaries of the ERP picture [1]. However, padding and blending may not be sufficient to completely resolve the seam issue, as a seam artifact is still visible for the ChairLift sequence, as reported in [2]. The seam artifact in ChairLift is particularly visible during video playback, as the video appears to flicker where face seams occur. One of the goals of CE13 is to study removing subjective artifacts at face boundaries using normative decoding tools. The tested coding tools include modifications to intra and inter prediction, as well as in-loop filtering and projection format adaptation. This contribution reports the coding performance of PERP with horizontal geometry padding of reference pictures, as studied in CE13 test 3.3 [3].

# Technical description

The horizontal geometry padding for PERP is the same as for ERP, with the exception that the offset should be based on the unpadded ERP width instead of the picture width to account for the size of the padded regions. Figure 1 depicts the horizontal geometry padding for PERP. If a reference block is outside the left (right) reference picture boundary, it is replaced with the “wrapped-around” reference block shifted to the right (left) by the ERP width. The traditional repetitive padding is still employed in the vertical direction. The blending of the left and right padded regions is kept out of loop, as a post-processing operation.

Figure 1. Horizontal geometry padding for PERP.

# Syntax

An exemplary syntax to enable horizontal geometry padding of reference pictures for the ERP and PERP formats is shown in Table 1.

Table 1. Geometry padding syntax in SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| ref_wraparound_offset | ue(v) |
| … |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is used for inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that this motion compensation method is not applied.

ref_wraparound_offset specifies the offset in luma samples used for computing the horizontal wrap-around position. ref_wraparound_offset shall not be equal to 0, shall not be larger than pic_width_in_luma_samples, and shall be an integer multiple of MinCbSizeY.

# Simulation results

The software is implemented based on BMS-2.0.1 (with fix BMS ticket #78 and #79) and 360Lib-7.0. The encoding was done under 360° video CTC [4]. The following table only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for horizontal geometry padding over conventional repetitive padding are reported in Table 2 for PERP.

Table 2. BD-rate for horizontal geometry padding over conventional repetitive padding for PERP.

|  |  | PERP |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.53% | -1.11% | -1.18% |
|  | ChairLift | -0.73% | -0.95% | -0.85% |
|  | KiteFlite | -0.02% | 0.03% | 0.03% |
|  | Harbor | -0.03% | -0.05% | -0.10% |
|  | Trolley | -0.12% | -0.13% | -0.18% |
|  | GasLamp | -0.01% | 0.02% | 0.01% |
| Class S2 | Balboa | -0.04% | 0.17% | 0.10% |
|  | Broadway | -0.04% | 0.05% | 0.02% |
|  | Landing2 | -0.14% | -0.26% | -0.81% |
|  | BranCastle2 | -0.52% | -0.70% | -0.81% |
| Class S1 |  | -0.24% | -0.37% | -0.38% |
| Class S2 |  | -0.18% | -0.19% | -0.37% |
| Overall |  | -0.22% | -0.29% | -0.38% |
| Enc Time[%] |  | 101% |  |  |
| Dec Time[%] |  | 98% |  |  |

BD-rate savings for horizontal geometry padding over conventional repetitive padding for PERP are around {-0.22%, -0.29%, -0.38%}. The maximum BD-rate savings is observed on sequence ChairLift and reaches {-0.73%, -0.95%, -0.85%}. The complexity of horizontal geometry padding is similar to that of repetitive padding.

Besides providing over 0.7% BD-rate saving for ChairLift, the most important highlight of horizontal geometry padding is that it significantly improves the subjective quality of the viewport video. With repetitive padding, a seam artifact is clearly visible when the viewport encompasses the left and right boundaries of the 360° video. Though less visible on still images, the seam artifact is particularly objectionable during video playback, as the video appears to flicker where face seams occur. The horizontal geometry padding method significantly reduces the seam artifact issue for the ChairLift sequence, and the seam artifact becomes barely noticeable to expert viewers.

# Conclusion

This contribution reports simulation result of CE13 test 3.3, which evaluates PERP with horizontal geometry padding of reference pictures. The results show BD-rate savings of {-0.22%, -0.29%, -0.38%} over PERP for VTM configuration. It is reported that horizontal geometry padding can significantly reduce the seam artifact for cases where PERP alone is not sufficient.

# References

- J. Boyce, Z. Deng, “EE4: Padded ERP (PERP) projection format”, JVET-G0098, July 2017, Torino, IT.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Geometry padding for PERP”, JVET-J0044, Apr. 2018, San Diego, USA.

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Specification

To enable horizontal geometry padding of reference pictures for the ERP and PERP formats, only the luma and chroma sample interpolation processes need to be modified, i.e., clauses 8.3.4.3.2 and 8.3.4.3.3. Changes are highlighted in yellow. In addition to these modifications, one additional mathematical function needs to be defined.

## 5.8 Mathematical functions

…

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise

ClipH( o, W, x ) =   o--x  % o  % o;x < 0( x-W ) % o + W-o;x > P-1x;otherwise

…

Mod( x, F );x < 0( x-P ) % F + P-F;x > P-1x;otherwise

- -

-

-

-

-

-

-

-

-

##### 8.3.4.3.2	Luma sample interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

–	The variable xOffset is set equal to ref_wraparound_offset.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-7.

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows the following applies:

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ Clip3( 0, picW − 1, xIntL ) ][ Clip3( 0, picH − 1, yIntL ) ] << shift3

–	Otherwise, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ ClipH( xOffset, picW, xIntL ) ][ Clip3( 0, picH − 1, yIntL ) ] << shift3

–	Otherwise if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows the following applies:

–	The value of yPosL is derived as follows:

yPosL = Clip3( 0, picH − 1, yIntL )

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = ( fL[ xFracL ][ 0 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 3 ) ][ yPosL ] +
					 fL[ xFracL ][ 1 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 2 ) ][ yPosL ] +
					 fL[ xFracL ][ 2 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 1 ) ][ yPosL ] +
					 fL[ xFracL ][ 3 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL ) ][ yPosL ] +
					 fL[ xFracL ][ 4 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 1 ) ][ yPosL ] +
					 fL[ xFracL ][ 5 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 2 ) ][ yPosL ] +
					 fL[ xFracL ][ 6 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 3 ) ][ yPosL ] +
					 fL[ xFracL ][ 7 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 4 ) ][ yPosL ] ) >> shift1

–	Otherwise, the value of predSampleLXL is derived as follows:

predSampleLXL = ( fL[ xFracL ][ 0 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 3 ) ][ yPosL ] +
					 fL[ xFracL ][ 1 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 2 ) ][ yPosL ] +
					 fL[ xFracL ][ 2 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 1 ) ][ yPosL ] +
					 fL[ xFracL ][ 3 ] * refPicLXL[ ClipH( xOffset, picW, xIntL ) ][ yPosL ] +
					 fL[ xFracL ][ 4 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 1 ) ][ yPosL ] +
					 fL[ xFracL ][ 5 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 2 ) ][ yPosL ] +
					 fL[ xFracL ][ 6 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 3 ) ][ yPosL ] +
					 fL[ xFracL ][ 7 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 4 ) ][ yPosL ] ) >> shift1

–	Otherwise if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of xPosL is derived as follows:

xPosL = Clip3( 0, picW − 1, xIntL )

–	Otherwise, the value of xPosL is derived as follows:

xPosL = ClipH( xOffset, picW, xIntL )

–	The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = ( fL[ yFracL ][ 0 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL − 3 ) ] +
					 fL[ yFracL ][ 1 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL − 2 ) ] +
					 fL[ yFracL ][ 2 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL − 1 ) ] +
					 fL[ yFracL ][ 3 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL ) ] +
					 fL[ yFracL ][ 4 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL + 1 ) ] +
					 fL[ yFracL ][ 5 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL + 2 ) ] +
					 fL[ yFracL ][ 6 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL + 3 ) ] +
					 fL[ yFracL ][ 7 ] * refPicLXL[ xPosL ][ Clip3( 0, picH − 1, yIntL + 4 ) ] ) >> shift1

–	Otherwise if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- If sps_ref_wraparound_enabled_flag is equal to 0, the sample array temp[ n ] with n = 0..7, is derived as follows:

yPosL = Clip3( 0, picH − 1, yIntL + n − 3 )

temp[ n ] = ( fL[ xFracL ][ 0 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 3 ) ][ yPosL ] +
				 fL[ xFracL ][ 1 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 2 ) ][ yPosL ] +
				 fL[ xFracL ][ 2 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL − 1 ) ][ yPosL ] +
				 fL[ xFracL ][ 3 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL ) ][ yPosL ] +
				 fL[ xFracL ][ 4 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 1 ) ][ yPosL ] +
				 fL[ xFracL ][ 5 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 2 ) ][ yPosL ] +
				 fL[ xFracL ][ 6 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 3 ) ][ yPosL ] +
				 fL[ xFracL ][ 7 ] * refPicLXL[ Clip3( 0, picW − 1, xIntL + 4 ) ][ yPosL ] ) >> shift1

- Otherwise, the sample array temp[ n ] with n = 0..7, is derived as follows:

yPosL = Clip3( 0, picH − 1, yIntL + n − 3 )

temp[ n ] = ( fL[ xFracL ][ 0 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 3 ) ][ yPosL ] +
				 fL[ xFracL ][ 1 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 2 ) ][ yPosL ] +
				 fL[ xFracL ][ 2 ] * refPicLXL[ ClipH( xOffset, picW, xIntL − 1 ) ][ yPosL ] +
				 fL[ xFracL ][ 3 ] * refPicLXL[ ClipH( xOffset, picW, xIntL ) ][ yPosL ] +
				 fL[ xFracL ][ 4 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 1 ) ][ yPosL ] +
				 fL[ xFracL ][ 5 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 2 ) ][ yPosL ] +
				 fL[ xFracL ][ 6 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 3 ) ][ yPosL ] +
				 fL[ xFracL ][ 7 ] * refPicLXL[ ClipH( xOffset, picW, xIntL + 4 ) ][ yPosL ] ) >> shift1

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = ( fL[ yFracL ][ 0 ] * temp[ 0 ] +
					fL[ yFracL ][ 1 ] * temp[ 1 ] +
					fL[ yFracL ][ 2 ] * temp[ 2 ] +
					fL[ yFracL ][ 3 ] * temp[ 3 ] +
					fL[ yFracL ][ 4 ] * temp[ 4 ] +
					fL[ yFracL ][ 5 ] * temp[ 5 ] +
					fL[ yFracL ][ 6 ] * temp[ 6 ] +
					fL[ yFracL ][ 7 ] * temp[ 7 ] )  >>  shift2

##### 8.3.4.3.3	Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

–	The variable xOffsetC is set equal to
 wrap_around_offset / SubWidthC.

The luma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-8.

The predicted chroma sample value predSampleLXC is derived as follows:

–	If both xFracC and yFracC are equal to 0, the value of predSampleLXC is derived as follows the following applies:

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of predSampleLXC is derived as follows:

predSampleLXC = refPicLXC[ Clip3( 0, picWC − 1, xIntC ) ][ Clip3( 0, picHC − 1, yIntC ) ] << shift3

–	Otherwise, the value of predSampleLXC is derived as follows:

predSampleLXC = refPicLXC[ ClipH( xOffsetC, picWC, xIntC ) ][ Clip3( 0, picHC − 1, yIntC ) ] << shift3

–	Otherwise if xFracC is not equal to 0 and yFracC is equal to 0, the value of predSampleLXC is derived as follows the following applies:

–	The value of yPosC is derived as follows:

yPosC = Clip3( 0, picHC − 1, yIntC )

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of predSampleLXC is derived as follows:

predSampleLXC = ( fC[ xFracC ][ 0 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC − 1 ) ][ yIntC ] +
					 fC[ xFracC ][ 1 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC ) ][ yIntC ] +
					 fC[ xFracC ][ 2 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC + 1 ) ][ yIntC ] +
					 fC[ xFracC ][ 3 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC + 2 ) ][ yIntC ] ) >> shift1

–	Otherwise, the value of predSampleLXC is derived as follows:

predSampleLXC = ( fC[ xFracC ][ 0 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC − 1 ) ][ yPosC ] +
					 fC[ xFracC ][ 1 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC ) ][ yPosC ] +
					 fC[ xFracC ][ 2 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC + 1 ) ][ yPosC ] +
					 fC[ xFracC ][ 3 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC + 2 ) ][ yPosC ] ) >> shift1

–	Otherwise if xFracC is equal to 0 and yFracC is not equal to 0, the value of predSampleLXC is derived as follows:

–	If sps_ref_wraparound_enabled_flag is equal to 0, the value of xPosC is derived as follows:

xPosC = Clip3( 0, picWC − 1, xIntC )

–	Otherwise, the value of xPosC is derived as follows:

xPosC = ClipH( xOffsetC, picWC, xIntC )

–	The predicted chroma sample value predSampleLXC is derived as follows:

predSampleLXC = ( fC[ yFracC ][ 0 ] * refPicLXC[ xPosC ][ Clip3( 0, picHC − 1, yIntC − 1 ) ] +
					 fC[ yFracC ][ 1 ] * refPicLXC[ xPosC ][ Clip3( 0, picHC − 1, yIntC ) ] +
					 fC[ yFracC ][ 2 ] * refPicLXC[ xPosC ][ Clip3( 0, picHC − 1, yIntC + 1 ) ] +
					 fC[ yFracC ][ 3 ] * refPicLXC[ xPosC ][ Clip3( 0, picHC − 1, yIntC + 2 ) ] ) >> shift1

–	Otherwise if xFracC is not equal to 0 and yFracC is not equal to 0, the value of predSampleLXC is derived as follows:

- If sps_ref_wraparound_enabled_flag is equal to 0, the sample array temp[ n ] with n = 0..3, is derived as follows:

yPosC = Clip3( 0, picHC − 1, yIntC + n − 1 )

temp[ n ] = ( fC[ xFracC ][ 0 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC − 1 ) ][ yPosC ] +
				 fC[ xFracC ][ 1 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC ) ][ yPosC ] +
					 fC[ xFracC ][ 2 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC + 1 ) ][ yPosC ] +
					 fC[ xFracC ][ 3 ] * refPicLXC[ Clip3( 0, picWC − 1, xIntC + 2 ) ][ yPosC ] ) >> shift1

- Otherwise, the sample array temp[ n ] with n = 0..3, is derived as follows:

yPosC = Clip3( 0, picHC − 1, yIntC + n − 1 )

temp[ n ] = ( fC[ xFracC ][ 0 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC − 1 ) ][ yPosC ] +
				 fC[ xFracC ][ 1 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC ) ][ yPosC ] +
					 fC[ xFracC ][ 2 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC + 1 ) ][ yPosC ] +
					 fC[ xFracC ][ 3 ] * refPicLXC[ ClipH( xOffsetC, picWC, xIntC + 2 ) ][ yPosC ] ) >> shift1

- The predicted chroma sample value predSampleLXC is derived as follows:

predSampleLXC = ( fC[ yFracC ][ 0 ] * temp[ 0 ] +
					 fC[ yFracC ][ 1 ] * temp[ 1 ] +
					 fC[ yFracC ][ 2 ] * temp[ 2 ] +
					 fC[ yFracC ][ 3 ] * temp[ 3 ] ) >> shift2