| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0237 |
| --- | --- |

| Title: | CE13-related: Adaptive frame packing using chroma sample location type 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

HEC with adaptive frame packing (AFP) is studied in CE13 test 6.1 and can achieve {Y, U, V} BD-rate savings of {-0.31%, -0.82%, -0.79%} over HEC (JVET-L0233) for VTM configuration. The AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. When the frame packing configuration of the reference picture is different from that of the current picture, the reference picture needs to be first converted into the frame packing configuration of the current picture. This conversion involves the rotation of some of the HEC faces, which may create a misalignment between the luma and chroma sample locations, depending on the chroma sample location type. However, as reported in JVET-L0238, there are some problems in the way the 360Lib is handling chroma samples, especially when a face is rotated. This contribution reports the simulation results of AFP with the modifications proposed in JVET-L0238 for the 360Lib and the modifications proposed in JVET-L0239 for the cross-component linear model (CCLM) downsampling filter. For chroma sample location type 0, the BD-rate performance of AFP over HEC is {-0.30%, 0.34%, 0.07%}. For chroma sample location type 1 and modified CCLM downsampling filter, the BD-rate performance of AFP over HEC is {-0.32%, -0.55%, -0.74%}.

# Introduction

HEC with adaptive frame packing (AFP) is studied in CE13 test 6.1 and can achieve {Y, U, V} BD-rate savings of {-0.31%, -0.82%, -0.79%} over HEC (JVET-L0233) for VTM configuration. The AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. As depicted in Figure 1, when the frame packing configuration of the reference picture is different from that of the current picture, the reference picture is first converted into the frame packing configuration of the current picture. This conversion is straightforward and only involves rotation and/or translation of the faces within the frame packed pictures. An example of the converted reference picture is shown in Figure 1(c).

Figure 1. Inter frame prediction in the case of different frame packing configurations between the reference (a) and current (b) pictures. The converted reference picture used to predict the current picture is shown in (c).

When a coded face is rotated, for example to convert the frame packing of one reference picture, the location of chroma samples may be misaligned after rotation. For chroma sample location type 0, the location of chroma samples is affected by rotation, as depicted in Figure 2, for rotations of 90°, 180°, and 270° counter-clockwise. However, chroma sample location type 1 is invariant under rotations of 90°, 180°, and 270° counter-clockwise, i.e., the location of one chroma sample is always at the center of four luma samples, as depicted in Figure 3.

| (a) | (b) | (c) | (d) |
| --- | --- | --- | --- |

Figure 2. Location of chroma samples in 4:2:0 chroma format with chroma sample location type 0: nominal sample locations (a) and after rotations of 90° (b), 180° (c), and 270° (d) counter-clockwise.

| (a) | (b) | (c) | (d) |
| --- | --- | --- | --- |

Figure 3. Location of chroma samples in 4:2:0 chroma format with chroma sample location type 1: nominal sample locations (a) and after rotations of 90° (b), 180° (c), and 270° (d) counter-clockwise.

This contribution reports the coding performance of HEC with AFP, as studied in CE13 test 6.1 [1], with modifications to 360Lib [2] and VTM [3] to support different chroma sample location types. The 360Lib modifications include mathematically correct mapping, resampling, and metrics calculations for different chroma sample location types [2]. The VTM modifications include different downsampling filters in cross-component linear model (CCLM) mode based on the chroma sample location type of the input video [3].

# Simulation results

The software is implemented based on modified version BMS-2.0.1 [3] and modified version of 360Lib-7.0 [2]. The encoding was done in the HEC format (without padding) under 360° video CTC [4]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for HEC with AFP over HEC are reported in Table 1. The anchor and test use the same chroma sample location type. Simulation results for type-0 and type-1 encoding are reported.

Table 1. BD-rate for HEC with AFP over HEC.

|  |  | type-0 encoding |  |  | type-1 encoding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.42% | 1.09% | 1.59% | -0.49% | -0.07% | 0.34% |
|  | ChairLift | -0.29% | 1.50% | -0.36% | -0.30% | -0.29% | -1.38% |
|  | KiteFlite | -0.17% | 0.32% | 0.58% | -0.16% | -0.68% | -0.59% |
|  | Harbor | -0.48% | -0.21% | -0.58% | -0.49% | -1.56% | -2.15% |
|  | Trolley | -0.62% | -0.95% | -1.30% | -0.64% | -1.56% | -1.77% |
|  | GasLamp | -0.65% | -0.44% | -0.11% | -0.64% | -1.72% | -1.15% |
| Class S2 | Balboa | -0.26% | 0.77% | 0.21% | -0.24% | -0.10% | -0.25% |
|  | Broadway | 0.02% | 0.86% | 0.82% | 0.00% | 0.82% | 0.12% |
|  | Landing2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | BranCastle2 | -0.16% | 0.42% | -0.11% | -0.19% | -0.35% | -0.53% |
| Class S1 |  | -0.44% | 0.22% | -0.03% | -0.45% | -0.98% | -1.12% |
| Class S2 |  | -0.10% | 0.51% | 0.23% | -0.11% | 0.09% | -0.17% |
| Overall |  | -0.30% | 0.34% | 0.07% | -0.32% | -0.55% | -0.74% |
| Enc Time[%] |  | 100% |  |  | 99% |  |  |
| Dec Time[%] |  | 99% |  |  | 99% |  |  |

Simulation results show that the coding performance on the luma component is not influenced by the chroma sample location type. However, for the chroma components, simulation results show an average coding loss for type-0 encoding, but an average coding gain for type-1 encoding. Overall, the average BD-rate performance of HEC with AFP over HEC is {-0.30%, 0.34%, 0.07%} and {-0.32%, -0.55%, -0.74%} for type-0 and type-1 encoding, respectively.

# Conclusion

This contribution reports the simulation results of AFP for different chroma sample location types. The simulation results show average BD-rate of {-0.30%, 0.34%, 0.07%} and {-0.32%, -0.55%, -0.74%} for type-0 and type-1 encoding, respectively. It is suggested to use chroma sample location type 1 for AFP.

# References

- P. Hanhart, Y. He, Y. Ye, “CE13: HEC with adaptive frame packing (Test 6.1)”, JVET-L0233, October 2018, Macau, CN.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Chroma sample location type support for 360Lib”, JVET-L0238, October 2018, Macau, CN.

- P. Hanhart, Y. He, Y. Ye, “CE3-related: Enabling different chroma sample location types in CCLM”, JVET-L0239, October 2018, Macau, CN.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).