| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0245-v2 |
| --- | --- |

| Title: | CE12-2: HDR In-loop Reshaping |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu, fpu}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This proposal reports CE12-2 subtests on HDR in-loop reshaping based on JVET-K0308, which comprises in-loop luma mapping, in-loop chroma residue scaling and configurable domain for loop filter application. CE12-2.1 evaluated different luma curves based on in-loop reshaping architecture described in JVET-K0308 (CE 12-2.1a) and JVET-K0298(CE12-2.1b). CE12-2.2 evaluated QP harmonization from JVET-K0298 with in-loop reshaping. CE12-2.3 evaluated in-loop cross-component chroma and luma refinement from JVET-K0298 with in-loop reshaping.

Summary performance gains of each CE12-2 subtest for in-loop reshaping of HDR PQ content under HDR fixed QP CTC compared to VTM2.0.1 HDR Anchor are:

- CE12-2.1a, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.1%, -2.3%, 2.5%}, AI {-2.0%, -2.4%, 1.5%}

- CE12-2.1b, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.2%, -3.6%, 3.2%}, AI {-1.7%, -3.6%, 2.8%}.

- CE12-2.2, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.1%, -2.3%, 2.4%}, AI {-2.0%, -2.4%, 1.2%}

- CE12-2.3, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.2%, -2.3%, 0.8%}, AI {-2.3%, -2.5%, -0.6%}

Additional results and comparisons of HDR coding technologies with other CE12 tests are presented and discussed with detailed performance analysis.

- In-loop reshaping provides significant performance gain compared to HDR Anchor.

- QP harmonization is an essential technology in out-of-loop design of CE12-1; but adds unnecessary complexity and marginal performance gain for in-loop design of CE12-2.

- Cross-component chroma refinement provides some chroma gain in both in-loop and out-of-loop designs.

# Introduction

In-loop reshaping is proposed in JVET-J0015 [1] and was tested in JVET-K0308 with VTM1.0 code base. In current CE12-2 [6], three subtests are designed to evaluate the in-loop reshaping architecture with VTM2.0.1 code base with some additional technical aspects: CE12-2.1 is designed to evaluate the in-loop reshaping performance with latest VTM2.0.1 codebase in which the luma mapping curve was derived using either the DLB method (CE12-2.1a) in JVET-K0308[2] or the DRA method (CE12-2.1b) in JVET-K0298[4]. CE12-2 is designed to evaluate the performance of QP harmonization with in-loop reshaping. CE12-3 is designed to evaluate the performance of in-loop cross-component chroma refinement with in-loop reshaping.

Summary performance gains of each CE12-2 subtests for in-loop reshaping of HDR PQ Content under HDR fixed QP CTC compared to VTM2.0.1 HDR Anchor are:

- CE12-2.1a, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.1%, -2.3%, 2.5%}, AI {-2.0%, -2.4%, 1.5%}

- CE12-2.1b, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.2%, -3.6%, 3.2%}, AI {-1.7%, -3.6%, 2.8%}.

- CE12-2.2, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.1%, -2.3%, 2.4%}, AI {-2.0%, -2.4%, 1.2%}

- CE12-2.3, average BDRate for wPSNRY/PSNR100/DE100: RA {-2.2%, -2.3%, 0.8%}, AI {-2.3%, -2.5%, -0.6%}

Additional results and comparisons with other CE12 tests are presented and discussed with detailed performance analysis in section 4. Complexity analysis is presented in section 5.

# CE12-2 Algorithm Description

## CE12-2.1: in-loop luma reshaping

### In-loop reshaping architecture and components

The complete in-loop reshaping architecture comprises in-loop luma mapping, in-loop chroma residue scaling, and configurable domain for loop filter application.

#### In-loop luma mapping

The in-loop luma reshaper is implemented and configured as a pair of one dimensional, 10-bit, 1024-entry mapping tables (1D-LUT). The first LUT is referred to as FwdLUT, which is used to map an input luma code value Yi to an altered value Yr: Yr=FwdLUT[Yi]. The second LUT is referred to as InvLUT, which is used to map altered code value Yr for “the reconstruction of Yi” as Yi: Yi=InvLUT[Yr]. Only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT.

The in-loop reshaper maps the luma code values based on the slice type received by the decoder.

For an intra slice, reshaping is performed in an explicit way. The HDR intra reconstruction process is performed in the same manner as in SDR decoding until the reconstructed samples are about to be stored into the DPB buffer. The reconstructed Yr samples are modified by the inverse reshaping process before the final intra reconstructed samples Yi are stored into the DPB buffer by Yi=InvLUT[Yr]. The simplified HDR intra slice decoding workflow is shown in Figure 1.

Figure 1. HDR Intra slice reconstruction with in-loop luma reshaper

For an inter slice, reshaping is implicitly performed. The decoding reconstruction process (e.g., Yr= Yres+Ypred) is modified so that reconstructed sample values are produced with the influence of the reshaper. The reconstructed samples are generated by Yi=InvLUT[Yres+FwdLUT[Ypred]]. All the other inter decoding tools are used in the same way as they are used in the SDR decoding process, such as entropy decoding, motion compensation, reference picture handling, etc. The simplified HDR inter slice decoding workflow is shown in Figure 2.

Figure 2. HDR Inter slice reconstruction with in-loop luma reshaper

#### In-loop chroma residue scaling

To compensate for the luma signal interaction with the chroma signal when luma reshaping is performed, the chroma components are adjusted with chroma residue scaling. This is an alternative implementation of the luma-based chroma QP Offset (CQPO) implementation in JVET-J0015[1] and JVET-K0308[2] to avoid the decoder-pipeline dependency issue in hardware implementation. At the encoder side, for the residue (CxRes = CxOrg - CxPred) of chroma component Cx of each CU,

CxResScaled = CxRes * cScaleYCU

CxResScaled is the scaled Cb/Cr residue signal of the CU to be transformed and quantized. At the decoder side, CxResScaled is the scaled chroma residue signal after inverse Q/T, and

CxRes = CxResScale / cScaleYCU

The final reconstruction of chroma component is CxRec = CxPred + CxRes. In this way, the decoder can start inverse quantization and transform immediately for chroma decoding after syntax parsing. The cScale being used for a CU is shared by the Cb and Cr components, and is derived as:

Equivalent cQPO[YCU]=-6*log2(FwdLUT'YCU)                (1)

cScaleYCU=FwdLUT'YCU=2-cQPO[YCU]6                       (2)

where YCU is the average of the predicted luma value in the current CU in inter slices (where dual tree coding is not used and therefore reconstructed luma is not available), and YCU is the average of the reconstructed luma value for the current CU in intra slices (where dual tree coding is used).

For HDR default reshaping, once the static default reshaper is constructed, the mapping between luma value and cScale can be calculated and stored in a static LUT cScale[Y] for use in coding the chroma components of the whole sequence. In CE12-2 software, the scales are calculated and stored with 16bit fixed point integers and the scaling operations at both encoder and decoder side are implemented with fixed point integer arithmetic. A fixed-point version of pow2 calculation is implemented to support all power of two calculations in fixed point.

#### Loop filter application

The in-loop reshaping architecture allows the loop filters (deblocking, SAO, and ALF) to be performed either in reshaped domain or in original domain.

For intra slices, if inverse reshaping is performed before loop filter, then the loop filtering is applied in original domain. At encoder side, loop filter (e.g., Deblocking Filter, SAO and ALF) parameters are estimated using pictures in original domain. If inverse reshaping is performed after loop filtering and before DPB storage, then the loop filtering is applied in reshaped domain. At encoder side, loop filter parameters are estimated using pictures in reshaped domain. Since the operations are the same for the two configurations, the only difference is the order of inverse reshaping and loop filtering, the complexity is the same for the two configurations.

For inter slices, pictures are in original domain after residue reconstruction. If loop filter is to be applied in original domain, the conventional loop filtering process is called and there is no additional step. If loop filter is to be applied in reshaped domain, an extra pair of forward and inverse reshaping is needed to reshape the reconstructed picture into reshaped domain, then apply the loop filter, then inverse reshape the filtered picture and store it in DPB. This introduces higher complexity to enable this option. In practice, to achieve a good trade-off between complexity and performance, it is not necessary to apply the loop filter in reshaped domain for all inter pictures. It is proposed that the option be enabled for pictures with TemporalId<= TemporalId_Thr. In CE12-2, loop filter is applied in reshaped domain for both intra slices and inter slices with TemporalId_Thr <= 2. Additional results are also provided for loop filter applied in original domain for both intra and inter slices for lowest complexity [8].

### In-loop reshaping luma mapping LUT

In CE12-2.1, two luma mapping curves are tested in this sub-CE for in-loop reshaping architecture. The only difference between CE12-2.1a and CE12-2.1b are the pair of luma mapping LUTs. All other coding tools remain the same (enable chroma residue scaling, loop filter optimization in reshaped domain).

- CE12-2.1a: the luma mapping curve is described in JVET-K0308, signaled using anchor dQP model and built from anchor dQP table, normalized to [0,1023], and mapped to full range values. The syntax of the K0308 luma mapping curve requires 36 bits for signaling.

- CE12-2.1b: the luma mapping curve is described in JVET-K0298, built from a piece-wise linear (PWL) model based on anchor dQP table, normalized to [0,1023], and mapped to full range values. The syntax of the K0298 luma mapping curve requires 296 bits for signaling.

Figure 3 shows the two luma mapping curves tested under CE12-2.1.

Figure 3. Luma mapping curves in CE12-2.1a (K0308) and CE12-2.1b (K0298).

## CE12-2.2: chroma residue scaling with QP harmonization

In JVET-K0298, it was pointed out that the luma-to-chroma mapping table f_QPi2QPc (HEVC spec. Table 8-10) introduces nonlinear mapping between QPi and final QPc. Because of this nonlinearity, the actual chroma coding QP (CxQP) is not always equal to luma coding QP plus chroma QP offset.

For HDR Anchor, CxQP of chroma component Cx (= Cb or Cr),

CxQP = f_QPi2QPc[QPY + CxQPOffset]

where QPY is the CU luma coding QP, CxQPOffset is the HDR Anchor chroma QP offset derived from input QP and content color gamut (709, P3, BT.2020) and signaled in PPS.

For in-loop luma reshaping with luma-based chroma QP offset implementation, no chroma residue scaling is performed but chroma QP is adjusted by a cQPO, which is luma based chroma QP offset derived from luma reshaping LUT (eq. 1). And the chroma coding QP should be:

CxQP = f_QPi2QPc[QPY + CxQPOffset + cQPO]

Alternatively, for in-loop luma reshaping with chroma residue scaling implementation, the chroma coding QP is the same as for the anchor but the chroma residues are scaled. The nonlinearity is not automatically addressed with the equivalent scale derived from cQPO. Therefore, the cQPO can be refined (harmonized) as:

CxQPO_harm = f_QPi2QPc[QPY + CxQPOffset + cQPO] - f_QPi2QPc[QPY + CxQPOffset]

CxScale = 2-CxQPO_harm6

It is worth mentioning that once QP harmonization is used, the cScale is no longer a static value shared by Cb/Cr and solely determined by the CU average luma. CxScale will change with QPY and CxQPOffset. CxScale either needs to be derived on the fly, or it is required to use two different LUTs to store the Cb and Cr scales respectively. The complexity of using QP harmonization is therefore significantly greater than non using it.

- Without QP harmonization: cScale = func (YCU)

- With QP harmonization:   cScale = func (YCU, QPY, CxQPOffset)

CE12-2.2 is designed to test in-loop reshaping with chroma residue scaling and QP harmonization.

## CE12-2.3: in-loop reshaping with in-loop cross-component refinement

Out-of-loop post cross-component refinement for both luma and chroma component was proposed for HDR video in JVET-J0022 and JVET-K0298 to compensate for the distortion resulting from the compression that may impact the inverse mapping process. In JVET-K0468[7], similar technology is evaluated for chroma components when applied in-loop and CE12-3 was designed to test this method separately with and without luma mapping. It is straightforward to combine this in-loop chroma refinement with in-loop reshaping to understand better the difference between the in-loop reshaping architecture and the out-of-loop architecture consisting of different modules. The software of CE12-3 was merged into CE12-2 to perform CE12-2.3 test.

# Simulation Results

The CE tests are implemented in BMS2.0.1 software with VTM tool settings (JEM_TOOLS = 0) and with JEM-comparable HDR implementation (WCG_EXT=1). The decoding process is implemented using fixed point integer arithmetic. The test is done by disabling BMS tools and used VTM CTC[5] with AI and RA. The test was performed for HDR PQ clips only because HDR HLG anchor does not use luma QP adaptation technology. SIMD optimization (AVX2) is automatically enabled at run time. The encoding time is measured using cluster. The decoding time is measured with YUV output enabled, using a PC station specified by Table 1.

Table 1. Technical specification of the PC workstation for decoding

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

A summary of CE12-2 subsets is presented in Table 2 and detailed results of each CE is presented in Section 3.1 to section 3.3.

Table 2. CE12-2.2 results summary

|  | AI BD-rate |  |  | RA BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| CE12-2.1a | -2.0% | -2.4% | 1.5% | -2.1% | -2.3% | 2.5% |
| CE12-2.1b | -1.7% | -3.6% | 2.8% | -2.2% | -3.6% | 3.2% |
| CE12-2.2 | -2.0% | -2.4% | 1.2% | -2.1% | -2.3% | 2.4% |
| CE12-2.3 | -2.3% | -2.5% | -0.6% | -2.2% | -2.3% | 0.8% |

## CE12-2.1: test of luma mapping curves

Table 3 and Table 4 show the BDrate results in terms of DE100, PSNRL100 and wPSNRY of CE12-2.1a and CE12-2.1b, respectively. Results suggest that under the in-loop reshaping architecture for RA and AI test conditions:

- DLB curve provides more consistent BDrate reduction than the DRA curve in terms of wPSNRY

- DLB curve results in less DE100 distortion than the DRA curve

- DRA curve results in less L100 distortion than the DLB curve

Table 3. CE12-2.1a results: in-loop reshaping with DLB mapping curve

|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI BD-rate |  |  | RA BD-rate |  |  |
| CE12-2.1a | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Market | -2.6% | -3.2% | 1.2% | -2.1% | -2.5% | 2.1% |
| SunRise | -1.4% | -1.9% | 4.1% | -0.4% | -0.5% | 6.2% |
| ShowGirl2 | -1.9% | -2.1% | -0.3% | -1.9% | -2.0% | -0.5% |
| BalloonFestival | -2.8% | -3.4% | 2.7% | -3.0% | -3.2% | 3.2% |
| Hurdles | -1.2% | -1.6% | 0.9% | -2.9% | -3.6% | 2.0% |
| Starting | -2.5% | -2.9% | 0.1% | -2.8% | -3.0% | 1.9% |
| Cosmos1 | -1.6% | -1.7% | 1.6% | -1.9% | -1.6% | 2.3% |
| Average HDR-B | -2.0% | -2.4% | 1.5% | -2.1% | -2.3% | 2.5% |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

Table 4. CE12-2.1b results: in-loop reshaping with DRA mapping curve

|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI BD-rate |  |  | RA BD-rate |  |  |
| CE12-2.1b | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Market | -2.1% | -3.7% | 4.3% | -2.2% | -3.9% | 4.0% |
| SunRise | -1.0% | -4.3% | 4.9% | 0.1% | -2.9% | 7.3% |
| ShowGirl2 | -1.3% | -2.2% | 0.9% | -1.6% | -2.2% | 0.2% |
| BalloonFestival | -2.5% | -4.5% | 3.1% | -2.9% | -4.0% | 3.9% |
| Hurdles | -1.3% | -2.3% | 5.3% | -4.0% | -5.9% | 2.9% |
| Starting | -2.4% | -4.3% | 1.3% | -2.6% | -3.9% | 3.1% |
| Cosmos1 | -1.4% | -3.6% | -0.1% | -2.4% | -2.6% | 0.8% |
| Average HDR-B | -1.7% | -3.6% | 2.8% | -2.2% | -3.6% | 3.2% |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

## CE12-2.2: test of QP harmonization

CE12-2.2 is designed to test the performance of the QP harmonization technology with in-loop reshaping. Table 5 shows the results of CE12-2.2 over HDR Anchor. DLB mapping curve is used so results are expected to be compared with CE12-2.1a (Table 3) using the same curve. Comparing Table 3 with Table 5, it is observed that QP Harmonization:

- does not change the results for wPSNRY and L100

- the change for DE100 is -0.3% (=1.2%-1.5%) for AI and -0.1% (=2.4-2.5%) for RA

Table 5. CE12-2.2: in-loop reshaping with DLB mapping curve + QP harmonization

|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI BD-rate |  |  | RA BD-rate |  |  |
| CE12-2.2 | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Market | -2.6% | -3.2% | 0.8% | -2.1% | -2.5% | 2.0% |
| SunRise | -1.3% | -1.9% | 3.7% | -0.4% | -0.5% | 6.3% |
| ShowGirl2 | -1.9% | -2.0% | -0.6% | -1.9% | -2.0% | -1.1% |
| BalloonFestival | -2.7% | -3.3% | 2.6% | -3.0% | -3.1% | 3.2% |
| Hurdles | -1.2% | -1.6% | 1.0% | -2.9% | -3.6% | 1.9% |
| Starting | -2.4% | -2.9% | -0.1% | -2.8% | -2.9% | 1.9% |
| Cosmos1 | -1.5% | -1.6% | 1.3% | -1.8% | -1.4% | 2.2% |
| Average HDR-B | -2.0% | -2.4% | 1.2% | -2.1% | -2.3% | 2.4% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

## CE12-2.3: test of in-loop chroma refinement with in-loop reshaping

CE12-2.3 is designed to test the performance of the in-loop cross-component chroma and luma refinement in CE12-3 with in-loop reshaping. Table 6.a shows the results of CE12-2.3 with chroma refinement over HDR Anchor. Table 6.b shows the results of CE12-2.3 with chroma and luma refinement over HDR Anchor. DLB mapping curve is used so results are expected to be compared with CE12-2.1a (Table 3) using same curve. Comparing Table 3 with Table 6.a, it is observed that in-loop cross-component chroma refinement:

- does not change the results for wPSNRY and L100

- the change of DE100 is -1.9%(= -0.4%-1.5%) for AI and -1.7% (=0.8%-2.5%) for RA

Comparing Table 3 with Table 6.b, it is observed that in-loop cross-component chroma and luma refinement:

- change the results for wPSNRY: -0.3% (=-2.3%-(-2.0%)) for AI and -0.1% (=-2.2%-(-2.1%)) for RA

- the change of DE100 is -2.1%(= -0.6%-1.5%) for AI and -1.7% (=0.8%-2.5%) for RA

Table 6.a. CE12-2.3: in-loop reshaping with DLB mapping curve + in-loop chroma refinement

|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI BD-rate |  |  | RA BD-rate |  |  |
| CE12-2.3 | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Market | -2.5% | -3.2% | 1.1% | -2.0% | -2.5% | 1.8% |
| SunRise | -1.4% | -1.9% | 3.9% | -0.3% | -0.5% | 5.9% |
| ShowGirl2 | -1.9% | -2.0% | -0.6% | -1.8% | -2.0% | -0.9% |
| BalloonFestival | -2.8% | -3.4% | 2.3% | -3.0% | -3.1% | 3.2% |
| Hurdles | -1.1% | -1.6% | -8.3% | -2.9% | -3.5% | -7.5% |
| Starting | -2.4% | -2.9% | 0.3% | -2.7% | -2.9% | 1.8% |
| Cosmos1 | -1.6% | -1.8% | -1.7% | -1.9% | -1.6% | 1.4% |
| Average HDR-B | -2.0% | -2.4% | -0.4% | -2.1% | -2.3% | 0.8% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |

Table 6.b. CE12-2.3: in-loop reshaping with DLB mapping curve + in-loop chroma and luma refinement

|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI BD-rate |  |  | RA BD-rate |  |  |
| CE12-2.3 CY | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Market | -2.6% | -3.2% | 1.2% | -2.1% | -2.5% | 1.9% |
| SunRise | -1.4% | -1.8% | 4.0% | -0.3% | -0.4% | 6.0% |
| ShowGirl2 | -2.0% | -2.1% | -0.5% | -1.9% | -2.0% | -0.9% |
| BalloonFestival | -2.8% | -3.4% | 2.3% | -3.0% | -3.1% | 3.2% |
| Hurdles | -1.1% | -1.7% | -8.2% | -2.9% | -3.6% | -7.5% |
| Starting | -2.4% | -2.9% | 0.3% | -2.7% | -2.9% | 1.9% |
| Cosmos1 | -4.2% | -2.8% | -3.1% | -2.3% | -1.6% | 1.1% |
| Average HDR-B | -2.3% | -2.5% | -0.6% | -2.2% | -2.3% | 0.8% |
| Enc Time[%] | 103% |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Additional Results and Performance Analysis

## Performance Comparison with other CEs

Since both of CE12-1 (out-of-loop HDR mapping) and CE12-2 (in-loop HDR mapping) are delivered as a whole package consisting of multiple aspects, it is meaningful to study each aspect and understand the performance contribution from each aspect if possible. Table 7 overviews the technologies for HDR coding presented in CE12.

For HDR Anchor, no normative coding tools were used but two encoder optimization methods, CU-based luma deltaQP and PPS chroma QP offset are enabled.

In CE12-1 (out-of-loop HDR mapping), both of the encoder optimizations are disabled and are replaced by luma mapping and chroma scaling. The chroma QP offsets were translated to equivalent chroma DRA scales and are incorporated into the total chroma scaling factors. CE12-1 also enables the QP harmonization and the cross-component refinement by default. Refinement applies for both luma and chroma components.

In CE12-2 (in-loop HDR mapping), only the luma dQP encoder optimization is replaced by the normative luma mapping. The chroma QP offset is kept same as Anchor. Chroma residue scaling is performed only aiming at compensating the luma-chroma interaction caused by luma mapping. QP harmonization and the cross-component refinement are not enabled by default. CE12-2.2 and CE12-2.3 were designed to test these additional aspects.

Table 7. Overview of technologies for HDR coding in CE12-1 and CE12-2

| Official CE12 Test cases | HDR Anchor encoder opt. |  | Normative luma mapping |  | Normative chroma scaling |  |  | Domain for LF Application | Normative Cross-component refinement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CU Luma dQP | PPS chroma QP offset | DLB curve | DRA curve | from eqv. chroma QP offset | from luma mapping | QPHarm |  | Luma | Chroma |
| HDR Anchor | Yes | Yes | No |  | No |  |  | original | No | No |
| CE12-1.1 | No | No | No | Yes, outloop | Yes, Chroma DRA | Yes, pixel based scaling of chroma component | Yes | mapped | Yes, outloop | Yes, outloop |
| CE12-1.2 | No | No | Yes, outloop | No | Yes, Chroma DRA | Yes, pixel based scaling of chroma component | Yes | mapped | Yes, outloop | Yes, outloop |
| CE12-2.1a | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | No | mapped (configurable) | No | No |
| CE12-2.1b | No | Yes | No | Yes, inloop | No | Yes, CU based scaling of chroma residue | No | mapped (configurable) | No | No |
| CE12-2.2 | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | Yes | mapped (configurable) | No | No |
| CE12-2.3 | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | No | mapped (configurable) | Yes, inloop | Yes, inloop |

Given these differences, several extra tests were performed for CE12-1 and CE12-2 in addition to the official CE experiments to study the performance of each aspect as thoroughly as possible. For fair comparison, all tests are performed with the same luma mapping curve (e.g. the DLB luma mapping curve based on CE12-1.2 and CE12-2.1a). The tests are categorized into three groups as follows:

### Group1: luma mapping + chroma scaling only, without QP harmonization and cross-component refinement:

| Test cases | HDR Anchor encoder opt. |  | Normative luma mapping |  | Normative chroma scaling |  |  | Domain for LF Application | Normative Cross-component refinement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CU Luma dQP | PPS chroma QP offset | DLB curve | DRA curve | from eqv. chroma QP offset | from luma mapping | QPHarm |  | Luma | Chroma |
| CE12-1.2 +Refine0 +QPHarm0 | No | No | Yes, outloop | No | Yes | Yes, pixel based scaling of chroma component | No | mapped | No | No |
| CE12-2.1a | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | No | mapped | No | No |

CE12-1.2+Refine0+QPHarm0 is tested by disabling QP harmonization and post refinement in CE12-1.2 software. It is not supported to separate the chroma DRA and CC-DRA in CE12-1 software and enable PPS chroma QP offset, therefore CE12-1.2+Refine0+QPHarm0 is the closest possible setting for comparison with CE12-2.1a. The summary of BDrate performance of this group is shown in Table 8.

The data in Table 8 indicate that luma performance (wPSNRY/PSNR L100) without QP harmonization and refinement in out-of-loop reshaping architecture is significantly impaired compared to the performance of in-loop reshaping architecture.

Table 8. Results of out-of-loop vs in-loop (luma mapping + chroma scaling only)

|  | AI BD-rate |  |  | RA BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | L100 | DE100 | wPsnrY | L100 | DE100 |
| Outloop: CE12-1.2+Refine0 +QPHarm0 | -0.4% | -1.1% | -10.3% | -0.6% | -1.1% | -12.9% |
| Inloop: CE12-2.1a | -2.0% | -2.4% | 1.5% | -2.1% | -2.3% | 2.5% |

### Group2: group1 + QP harmonization:

| Test cases | HDR Anchor encoder opt. |  | Normative luma mapping |  | Normative chroma scaling |  |  | Domain for LF Application | Normative Cross-component refinement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CU Luma dQP | PPS chroma QP offset | DLB curve | DRA curve | from eqv. chroma QP offset | from luma mapping | QPHarm |  | Luma | Chroma |
| CE12-1.2 +Refine0 | No | No | Yes, outloop | No | Yes | Yes, pixel based scaling of chroma component | Yes | mapped | No | No |
| CE12-2.2 | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | Yes | mapped | No | No |

CE12-1.2+Refine0 is tested by enabling QP harmonization and disabling post refinement in CE12-1.2 software. When comparing the data results in Table 9 with those in Group1 (Table 8), it is clearly seen that QP harmonization is responsible for a significant amount of the luma/chroma coding performance in the out-of-loop architecture, while it only has trivial chroma improvement in the in-loop architecture.

The reason for such difference is that the CE12-1 out-of-loop design absorbs the anchor chroma QP offset into the chroma scales instead of applying the chroma QP Offset. If the Offset is applied, chroma coding QP will be automatically “harmonized” by the QPi2QPc table mapping. If the equivalent scale is applied, it will not be automatically harmonized, therefore scales need to be adjusted explicitly. This also explains why QP harmonization only provides trivial gain in the CE12-2 in-loop design, as the chroma QP offsets are kept the same as in anchor and is thus harmonized automatically.

Figure 4 shows two examples of bitrate breakdown, showing that without QP harmonization, the out-of-loop design of CE12-1 has huge chroma bitrate increase at high QP values. For in-loop design in CE12-2, the luma-chroma rate allocation follows HDR anchor closely independent from use or non-use of QP harmonization (used in CE12-2.2 and not used in CE12-2.1a).

Table 9. Results of out-of-loop vs in-loop (luma mapping + chroma scaling + QP Harmonization)

|  | AI BD-rate |  |  | RA BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | L100 | DE100 | wPsnrY | L100 | DE100 |
| Outloop: CE12-1.2+Refine0 | -2.2% | -2.8% | -4.4% | -2.4% | -2.9% | -6.4% |
| Inloop: CE12-2.2 | -2.0% | -2.4% | 1.2% | -2.1% | -2.3% | 2.4% |

Figure 4. Bitrate break down examples. Statistics are taken from decoding logs produced by enabling DecoderAnalyser, “Break down by component/Channel type”. total Ybits = “Y”+“Luma”, total Cbits = “Cb”+“Cr”+ “Chroma”, MiscBits = “-” )

### Group3: group2 + refinement:

| Test cases | HDR Anchor encoder opt. |  | Normative luma mapping |  | Normative chroma scaling |  |  | Domain for LF Application | Normative Cross-component refinement |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CU Luma dQP | PPS chroma QP offset | DLB curve | DRA curve | from eqv. chroma QP offset | from luma mapping | QPHarm |  | Luma | Chroma |
| CE12-1.2 | No | No | Yes, outloop | No | Yes | Yes, pixel based scaling of chroma component | Yes | mapped | Yes, outloop | Yes, outloop |
| CE12-2.2 +CE12-2.3,chroma only | No | Yes | Yes, inloop | No | No | Yes, CU based scaling of chroma residue | Yes | mapped | No | Yes, inloop |

CE12-2.2+CE12-2.3 is tested by enabling both QP harmonization and in-loop refinement in CE12-2 software. CE12-2.2+CE12-2.3 is the closest possible setting for comparison with CE12-1.2 which has both luma and chroma refinement. In-loop luma refinement has been implemented in CE12-2.3 so additional test are performed with both luma and chroma refinement enabled.

Summary results of group 3 are presented in Table 10. It is shown that refinement provides large chroma improvement both in-loop and out-of-loop (larger in out-of-loop).Luma gain is slightly dropped by 0.1% in AI for in-loop refinement and slightly improved by 0.1% for out-of-loop refinement. This results is based onthe missing in-loop luma refinement. Additional CE12-2.3 test has been performed with both in-loop luma and chroma refinement.

Table 10. Results of out-of-loop (with luma and chroma refinement) vs in-loop (with luma and chroma refinement)

|  | AI BD-rate |  |  | RA BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | L100 | DE100 | wPsnrY | L100 | DE100 |
| Outloop: CE12-1.2 | -2.3% | -2.9% | -7.5% | -2.5% | -3.0% | -9.0% |
| Inloop: CE12-2.2+CE12-2.3, chroma only | -1.9% | -2.4% | -0.6% | -2.1% | -2.3% | 0.7% |
| Inloop: CE12-2.1a + CE12-2.3_luma & chroma | -2.3% | -2.5% | -0.6% | -2.2% | -2.3% | 0.8% |

### Summary on performance:

Despite the presence of missing modules and different implementations, we can still draw some meaningful conclusions from the previous analysis:

- In-loop reshaping provides significant performance gain compared to HDR Anchor. Comparing two luma curves (DLB versus DRA), DLB curves provides more consistent gain in wPSNRY for AI and RA, better gain in DE100, but less gain in PSNR-L. The bitrate overhead of DLB curve (36bits using dQP model) is only 12% of the bitrate overhead of the DRA curve (296 bits using PWL model).

- QP harmonization is an essential decoder technology for out-of-loop architecture but does not impact and is not needed for in-loop.

- Cross-component chroma refinement provides some gain in both in-loop and out-of-loop designs.

## Decoder Complexity Analysis

A strict decoding environment was created by using a dedicated PC (Table 1). Decoding for each rate point in each test case was performed sequentially without other multi-tasking on the PC. Decoding time reported for CE12-2 and CE12-1 crosscheck are drawn from decoding logs from this strict decoding environment. In addition, complexity evaluations are made with conceptual algorithm analysis.

[Note: It was observed that decoding runtime collected using cluster is highly unreliable and thus cannot be solely used to judge the complexity of the proposal.]

### Complexity of in-loop architecture in CE12-2

All in-loop related operations, such as LUT construction, luma mapping, and chroma residue scaling are all implemented with fixed-point arithmetic at the decoder side. The major computational complexity of in-loop reshaping is the mapping for luma samples using the pair of FwdLUT and InvLUT.

The pair of reshaping LUTs are implemented using 2-byte (10-bit), 1024 entry integer arrays. The in-loop reshaper performs luma sample mapping in-place, meaning no extra storage buffer is needed for luma samples. For luma mapping in inter slices and chroma residue scaling, the operations are performed on residue signal, which means at decoder side, if cbf=0 (no coefficient), there is no need to perform inverse reshaping and chroma scaling. This can potentially save a lot of operations for low bit rate video decoding.

- Inverse luma reshaping in inter slices: Yr=InvLUTYres+FwdLUTYpred

When cbf_luma = 0, Yres=0, it is simplified as Yr=Ypred, and no mapping is required.

- Inverse chroma residue scaling: CxRes = CxResScale / cScaleYCU

When cbf_cx = 0, CxResScale=CxRes=0. There’s no need to calculate and perform scaling.

Otherwise, when QP harmonization is not enabled, cScaleY , Y=0:1023 is calculated only once after parsing reshaping model and is stored in a static LUT. For each CU, the cScaleYCU is used for all residue samples of the Cb and Cr components in the CU.

The loop filter application in CE12-2 in-loop design requires an additional operation of FwdLUT and InvLUT mapping in inter slices with TemporalId_Thr <= 2 to apply loop filtering in reshaped domain. This additional complexity can be removed by configuring loop filtering application always in original domain, as in [8].

The in-loop reshaper does not employ any additional SIMD optimization expect for those already in the VTM2.0.1 codebase. The decoding running time increase over VTM2.0.1 is limited to under 6%, measured on the dedicated PC with YUV output enabled. In addition, the complexity of in-loop reshaping and its components is mainly determined by picture resolution and will not change if decoder complexity increases due to the enabling of more complex coding tools (e.g. BMS coding tools under test).

### Complexity of out-of-loop architecture in CE12-1

Measured under the identical decoding environment as for CE12-2, the decoding running time for CE12-1 increase over VTM2.0.1 is 11% with YUV output enabled. When YUV output is not enabled, the inverse out-of-loop DRA and post refinement are not invoked (for being post process on output pictures), so it is not reasonable to measure decoding time without YUV output.

The measured decoding complexity of out-of-loop (11%) is almost double compared with in-loop (6%). To understand better the complexity difference, the implementation details were studied. It was observed that:

- The luma inverse mapping process in CE12-1 is similar to the intra slice in-loop inverse mapping process in CE12-2. Both use an InvLUT to perform table look up.

- The out-of-loop inverse mapping for all three components must be performed for all samples all the time (when YUV output is enabled). In comparison, for in-loop design, forward and inverse luma mapping is skipped for a CU in inter slices if the CU does not have luma coefficient, and inverse chroma (Cx) residue scaling is skipped for a CU in both inter and intra slices if the CU does not have chroma (Cx) coefficient.

- The inverse chroma process in CE12-1 is more complex than the inverse chroma residue scaling in CE12-2 in both the calculation of scale and in the inverse scaling operation:

  - Calculation of scale:

    - CE12-1 out-of-loop design: the chroma scale is a consolidated value that consists of contributions from chroma DRA, CC-DRA and QP harmonization. The QP harmonization process involves floating point calculations, including usage of log() and exp() functions (e.g in TComDRA.cpp, functions: getScaleDRA2QPd(),getQP2ScaleDRAd(), correctLocalChromaScale()). For a chroma sample Cx with co-located luma sample value Y, the chroma scale factor is derived as:

S = QPHarm_func( S1(CxQPOffset) , S2(Y) , QPY)                         (3)

Chroma DRA: S1 = 1.0/exp(CxQPOffset/6.0 * log(2.0)) , which is conversion from chroma QP Offset CxQPOffset to chroma scaling factor.

CC-DRA: S2(Y), which is scaling factor from luma DRA.

QP Harmonization process in QPHarm_func (S1, S2, QPY) using HEVC Table8-10 f_QPi2QPc[] is designed to rely on floating point calculations:

double_cxQPOffset = (log(S1)*6.0)/log(2.0);

double_deltaLumaQP = (log(S2)*6.0)/log(2.0);

double_localCQPshift1 = QPY – f_QPi2QPc[QPY];

double_localQPi = QPY + double_cxQPOffset - double_deltaLumaQP;

int_QP1 = f_QPi2QPc[(int) double_localQPi];

int_QP2 = f_QPi2QPc[(int) double_localQPi + 1];

double_localCQPshift2 = double_localQPi – (int_QP1 + (int_QP2 - int_QP1) * (double_localQPi – (int) double_localQPi));

double_harmscale = exp((double_localCQPshift2 - double_localCQPshift1 ) /6.0 * log(2.0));

S = S1 * S2 * double_harmscale;

    - CE12-2 in-loop design: the chroma scale is only determined by CU average luma value. The same scale is used for all Cb/Cr residue inside a CU (if there is non-zero residue). A fixed- point version of pow2 is implemented for calculation of the scales. A static fixed-point LUT S(Y) is calculated after SPS reshaping model parsing and is saved for future usage by each CU.

SCU = S(Y) =2-cQPO[YCU]6

  - Scaling operation:

    - CE12-1 out-of-loop design: inverse chroma scaling is performed on decoded chroma signal CXm (zero mean) with the scale SXm derived by eq.(3), and final chroma value:

CX=CXm-2bitDepth-1/SXm+2bitDepth-1

    - CE12-2 in-loop design: scale is performed on chroma residue signal directly, no need for adding and subtracting the zero-mean offset 2bitDepth-1:

CxRes = CxResScale / SCU

# Conclusion

Summary performance gains of each CE12-2 subtests for in-loop reshaping of HDR PQ content under HDR fixed QP CTC compared to VTM2.0.1 HDR Anchor have been reported in this contribution. Additional results and comparisons of HDR coding technologies with other CE12 tests are presented and discussed with detailed performance analysis. In-loop reshaping of HDR PQ achieves significant performance gain compared to HDR anchor with low to moderate increase in complexity.

It is recommended to adopt in-loop reshaping in VVC standard.

# Reference

- X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April, 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8)”, JVET-K0308, Ljubljana, SI, July, 2018.

- F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen, “SDR In-loop Reshaping”, JVET-K0309, Ljubljana, SI, July, 2018.

- E. Francois, D. Rusanovskyy, et al, “CE12: report of dynamic range adaptation and DRA refinement”, JVET-K0298, Ljubljana, SI, July 2018.

- A. Segall, E. Francois, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-K1011, SI, July, 2018.

- E. Francois, D. Rusanovskyy, P. Yin, “Description of Core Experiment 12 (CE12): mapping functions”, JVET-K1032, Ljubljana, SI, July 2018.

- E.Francois, C. Chevance, F. Hiron, “CE12-related: In-loop chroma refinement”, JVET-K0468, Ljubljana, SI, July 2018.

- T. Lu, S. McCarthy, F. Pu, P. Yin, W. Husak, T. Chen, “CE12 related: Universal low complexity reshaper for SDR and HDR video”, JVET-L0247, Macao, CN, October 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgement

We would like to thank Technicolor, Sharp and Qualcomm for crosschecking our CE results.

# Annex A: Syntax

The reshaping related syntax is added in SPS() (Table A-1), slice header(Table A-2) and CTU(Table A-3) to allow sequence/slice/CTU level adaptation. The reshaper model can be signalled in SPS and slice header. In this CE, because default reshaping curve is used for HDR, sps_reshaper_model() is used. The default FwdLUT is signalled by linear dQP model, which is 36bits (Table A-4).

Table A-1. Reshaping syntax in SPS

| SPS() | Descriptor |
| --- | --- |
| …… |  |
| sps_reshaper_enable_flag /*1: reshaping on, else off */ | u(1) |
| if (sps_reshaper_enable_flag) { |  |
| sps_reshaper_signal_type /* 0:SDR, 1:PQ, 2:HLG*/ | u(2) |
| sps_reshaper_chromaAdj_flag | u(1) |
| sps_reshaper_model_present_flag | u(1) |
| if (sps_reshaper_model_present_flag) |  |
| sps_reshaper_model () |  |
| } |  |

sps_reshaper_enable_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type equal to 0 specifies SDR (gamma) transfer characteristics; sps_reshaper_signal_type equal to 1 specifies PQ transfer characteristics; sps_reshaper_signal_type equal to 2 specifies HLG transfer characteristics; and sps_reshaper_signal_type equal to 3 is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved value of sps_reshaper_signal_type.  When sps_reshaper_signal_type is not present, it is inferred to be equal to 0.

sps_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is used in CVS. sps_reshaper_chromaAdj equal to 0 specifies that chroma residue scaling is not used in CVS. When sps_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

sps_reshaper_model_present_flag equal to 1 specifies sps_reshaper_model() is present in SPS. sps_reshaper_model_present_flag equal to 0 specifies sps_reshaper_model() is not present in SPS. When sps_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

Table A-2. Reshaping syntax in slice header

| slice_header() | Descriptor |
| --- | --- |
| …… |  |
| if ( sps_reshaper_enable_flag ) { |  |
| slice_reshaper_model_present_flag | u(1) |
| if ( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if ( slice_reshaper_enable_flag) { |  |
| if ( sps_reshaper_chromaAdj_flag ) |  |
| slice_reshaper_chromaAdj_flag | u(1) |
| slice_reshaper_CTU_control_flag | u(1) |
| } |  |
| } |  |
| …… |  |

slice_reshaper_model_present_flag equal to 1 specifies slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 specifies slice_reshaper_model() is not present in slice header. When slice_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

slice_reshaper_enable_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enable_flag equal to 0 specifies that reshaper is not enabled for the current slice. When slice_reshaper_enable_flag is not present, it is inferred to be equal to 0.

slce_reshaper_CTU_control_flag equal to 1 specifies that reshaper adaption is allowed for each CTU in current slice. slice_reshaper_CTU_control_flag equal to 0 specifies that reshaper is not allowed to be adapted for each CTU in current slice. When slice_reshaper_CUT_control_flag is not present, it is inferred to be equal to 0.

Table A-3. Reshaping syntax in CTU

| coding_tree_unit() | Descriptor |
| --- | --- |
| …… |  |
| if ( slice_reshape_CTU_control_flag ) { |  |
| reshaper_CTU_flag | ae(v) |
| } |  |
| …… |  |

reshaper_CTU_flag equal to 1 specifies that reshaper is used for the current CTU. reshaper_CUT_flag equal to 0 specifies that reshaper is not used for the current CTU. When reshaper_CTU_flag is not present, it is inferred to equal to slice_reshaper_enabled_flag.

slice_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is enabled for the current slice. slice_reshaper_chromaAdj_flag equal to 0 specifies that chroma residue scaling is not enabled for the current slice. When slice_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

Table A-4. Reshaper model syntax in SPS

| sps_reshaper_model() { | descriptor |
| --- | --- |
| full_range_input_flag | u(1) |
| dQP_model_scale_int_prec | ue(v) |
| if (dQP_model_scale_int_prec > 0) { |  |
| dQP_model_scale_int | u(v) |
| } |  |
| dQP_model_scale_frac_prec_minus16 | ue(v) |
| dQP_model_scale_frac | u(v) |
| if (dQPModelScaleAbs) { |  |
| dQP_model_scale_sign_flag | u(1) |
| } |  |
| dQP_model_offset_int_prec_minus3 | ue(v) |
| dQP_model_offset_int | u(v) |
| dQP_model_offset_frac_prec_minus1 | ue(v) |
| dQP_model_offset_frac | u(v) |
| if (dQPModelOffsetAbs) { |  |
| dQP_model_offset_sign_flag | u(1) |
| } |  |
| dQP_model_abs_prec_minus3 | ue(v) |
| dQP_model_max_abs | u(v) |
| if (dQP_model_max_abs) { |  |
| dQP_model_max_sign_flag | u(1) |
| } |  |
| dQP_model_min_abs | u(v) |
| if (dQP_model_min_abs) { |  |
| dQP_model_min_sign_flag | u(1) |
| } |  |
| } |  |

full_range_input_flag equal to 0 specifies a standard dynamic range input video signal. full_range_input_flag equal to 0 specifies a full range input video signal.  When full_range_input_flag is not present, it is inferred to be equal to 0.

dQP_model_scale_int_prec specifies the number of bits used for the representation of dQP_model_scale_int.

dQP_model_scale_int specifies the integer value of dQP model scale. When dQP_model_scale_int is not present, it is inferred to be equal to 0.

dQP_model_scale_frac_prec_minus16 plus 16 specifies the number of bits used for the representation of dQP_model_scale_frac.

dQP_model_scale_frac specifies the fractional value of the dQP model scale.

The variable dQPModelScaleAbs is derived as:

dQPModelScaleAbs = dQP_model_scale_int << (dQP_model_scale_frac_prec_minus16 + 16) + dQP_model_scale_frac

dQP_model_scale_sign_flag specifies the sign of dQP model scale as follows:

- If dQP_model_scale_sign_flag is equal to 0, the corresponding variable dQPModelScale has a positive value.

- Otherwise (dQP_mode_scale_sign_flag is equal to 1), the corresponding variable dQPModelScale has a negative value.

When dQP_model_scale_sign is not present, it is inferred to be equal to 0.

The variable dQPModelScale = dQPModelScaleAbs * ( 1 – 2 * dQP_model_scale_sign_flag);

dQP_model_offset_int_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_offset_int.

dQP_model_offset_int specifies the integer value of dQP model offset.

dQP_model_ offset _frac_prec_minus1 plus 1 specifies the number of bits used for the representation of dQP_model_ offset_frac.

dQP_model_ offset_frac specifies the fractional value of the dQP model offset.

The variable dQPModelOffsetAbs is derived as:

dQPModelOffsetAbs = dQP_model_offset_int << (dQP_model_offset_frac_prec_minus1 + 1) + dQP_model_offset_frac

dQP_model_offset_sign_flag specifies the sign of dQP model offset as follows:

- If dQP_model_offset_sign_flag is equal to 0, the corresponding dQPModelOffset has a positive value.

- Otherwise (dQP_model_offset_sign_flag is equal to 1), the corresponding dQPModelOffset has a negative value.

When dQP_model_offset_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelOffset = dQPModelOffsetAbs * ( 1 – 2 * dQP_model_offset_sign_flag);

dQP_model_abs_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_max_abs and dQP_model_min_abs.

dQP_model_max_abs specifies the absolute value of the maximum dQP value.

dQP_model_max_sign_flag specifies the sign of the maximum dQP value as follows:

- If dQP_model_max_sign_flag is equal to 0, the corresponding dQPModelMax has a positive value.

- Otherwise (dQP_model_max_sign_flag is equal to 1), the corresponding dQPModelMax has a negative value.

When dQP_model_max_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMax = (dQP_model_max_abs) * ( 1 – 2 * dPQ_model_max_sign_flag ).

dQP_model_min_abs specifies the specifies the absolute value of the minimal dQP value.

dQP_model_min_sign_flag specifies the sign of the minimal dQP value as follows:

- If dQP_model_min_sign_flag is equal to 0, the corresponding dQPModelMin has a positive value.

- Otherwise (dQP_model_min_sign_flag is equal to 1), the corresponding dQPModelMin has a negative value.

When dQP_model_min_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMin = (dQP_model_min_abs) * ( 1 – 2 * dQP_model_min_sign_flag ).