| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0247-v2 |
| --- | --- |

| Title: | CE12-related: Universal low complexity reshaper for SDR and HDR video |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Sean McCarthy, Fangjun Pu, Peng Yin, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, smcca}@dolby.com |
| Source: | Dolby Laboratories, Inc., |  |  |

_____________________________

# Abstract

In-loop reshaping was tested in CE-12 mapping functions for both SDR and HDR video.

This contribution recommends a universal low complexity in-loop reshaping architecture for both SDR and HDR. The proposed universal reshaper has the key advantage that it provides significant performance gains with minimal impact on implementation. As such, it provides the best balance between complexity and improved compression efficiency for both SDR and HDR.

Specifically, the proposed reshaper benefits by performing source mapping for both intra and inter slices at the same location in the decoder workflow.  The proposed reshaper also benefits by applying loop filtering in the original (non-mapped) domain for which loop filters were originally designed and optimized. In addition, the proposed reshaper enables slice level adaption needed to optimize coding efficiency at the picture level.

The proposed reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) complementary chroma residue scaling applied to chroma components

The proposed universal reshaper has the following performance gains:

- For SDR video, average BDRate for Y/U/V:

  - RA {-1.32%, 2.07%, 1.62%}

  - AI { -0.96%, 2.56%, 2.13%};

- For HDR video, average BDRate for wPSNRY/PSNR100/DE100:

  - RA {-1.8%, -2.0%, 2.3%}

  - AI { -1.7%, -2.0%, 1.4%}

Performance gains are larger for higher resolution video (UHD and HD):

- For SDR video, average BDRate for Y/U/V:

  - RA {-1.43%, 2.18%, 1.47%}

  - AI { -1.36%, 2.51%, 1.84%}

The decoding running time increase is 5% or less. The encoding running time increase is 7% or less.

It is recommended that the proposed universal reshaper be adopted as a core coding tool in VVC for SDR and HDR.

# Introduction

Reshaping, or source mapping, of video content was first explored several years ago motivated by a desire to make HDR content more compatible with HEVC coding technologies. However, it became clear that reshaping has the additional benefit of providing consistently demonstrable performance gains for both HDR and, in the case of in-loop reshaping, for SDR video.

In general terms, reshaping delivers performance gains because reshaping provides finer-grained pixel-level control of source coding compared to the block-level operations and coarser quantization used in the anchor. Source mapping using in-loop reshaping has been tested in both VTM1 and VTM2 where it has shown consistent gain for both SDR and HDR video compared to the CTC anchor.

The proposed universal reshaper corresponds to the lowest complexity in-loop reshaper architecture tested in the CE for both SDR (CE12-4) and HDR video (CE12-2.1a ILFOPT=0).

Based on the CE results, it is recommended that the proposed universal reshaper be adopted as a core coding tool in VVC for SDR and HDR.

# Proposal

The proposed universal reshaping reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) complementary chroma residue scaling applied to chroma components.

Key elements of the proposed reshaper from the decoder side are highlighted below. For more detailed descriptions, especially how the encoder works, please refer to CE12 report [3][4].

## Universal low complexity in-loop luma reshaping architecture

The in-loop luma reshaper is implemented as a pair of look-up tables (LUTs); but only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT.

Each LUT is a one-dimensional, 10-bit, 1024-entry mapping table (1D-LUT). One LUT is a forward LUT, FwdLUT, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. The other LUT is an inverse LUT, InvLUT, that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

For intra slices, only the InvLUT is applied.  For inter slices, both FwdLUT and InvLUT are applied.

LUTs are applied before loop filtering for both intra and inter slices. Processing operations and data flow are identical for SDR and HDR.

The processing operations for intra slices are illustrated in Figure 1.  InvLUT maps intra reconstructed values in the reshaped domain to intra reconstructed values in the original domain. (Yi=InvLUT[Yr])

The processing operations for inter slices are illustrated in Figure 2.  FwdLUT maps motion-compensation values in the original domain to the reshaped domain. (FwdLUT[Ypred])  InvLUT then maps inter reconstructed values in the reshaped domain to inter reconstructed values in the original domain. (Yi=InvLUT[Yres+FwdLUT[Ypred]])

Figure 1. Intra slice reconstruction with in-loop luma reshaper

Figure 2. Inter slice reconstruction with in-loop luma reshaper

## Chroma residue scaling

Chroma residue scaling is a simple multiplicative process implemented with fixed-point integer operation. Chroma residue scaling compensates for luma signal interaction with the chroma signal. Chroma residue scaling is applied at the TU level.

Encoder side: cResScale = cRes * cScale

Decoder side: cRes = cResScale/cScale

cRes is the original chroma residual signal and cResScale is the scaled chroma residual signal. cScale is a scaling factor calculated using FwdLUT as described in [1].

# Syntax

The reshaping related syntax is shown in Table 1 for SPS() and in Table 2 for slice header() to enable sequence and slice level adaptation, respectively.

The reshaper model is signalled in the SPS and in the slice header. The FwdLUT is signalled in the SPS by a linear dQP model, sps_reshaper_model(), which is about 40bits. In the slice header, an adaptive reshaping curve is signaled, slice_reshaper_model(), which is about 80 bits. More detail about reshaping model syntax and semantics is presented in Annex A.

Table 1. Reshaping syntax in SPS

| SPS() | Descriptor |
| --- | --- |
| …… |  |
| sps_reshaper_enable_flag /*1: reshaping on, else off */ | u(1) |
| if (sps_reshaper_enable_flag) { |  |
| sps_reshaper_signal_type /* 0:SDR, 1:PQ, 2:HLG*/ | u(2) |
| sps_reshaper_chromaAdj_flag | u(1) |
| sps_reshaper_model_present_flag | u(1) |
| if (sps_reshaper_model_present_flag) |  |
| sps_reshaper_model () |  |
| } |  |

sps_reshaper_enable_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type equal to 0 specifies SDR (gamma) transfer characteristics; sps_reshaper_signal_type equal to 1 specifies PQ transfer characteristics; sps_reshaper_signal_type equal to 2 specifies HLG transfer characteristics; and sps_reshaper_signal_type equal to 3 is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved value of sps_reshaper_signal_type.  When sps_reshaper_signal_type is not present, it is inferred to be equal to 0.

sps_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is used in CVS. sps_reshaper_chromaAdj equal to 0 specifies that chroma residue scaling is not used in CVS. When sps_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

sps_reshaper_model_present_flag equal to 1 specifies sps_reshaper_model() is present in SPS. sps_reshaper_model_present_flag equal to 0 specifies sps_reshaper_model() is not present in SPS. When sps_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

Table 2. Reshaping syntax in slice header

| slice_header() | Descriptor |
| --- | --- |
| …… |  |
| if ( sps_reshaper_enable_flag ) { |  |
| slice_reshaper_model_present_flag | u(1) |
| if ( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if ( slice_reshaper_enable_flag) { |  |
| if ( sps_reshaper_chromaAdj_flag ) |  |
| slice_reshaper_chromaAdj_flag | u(1) |
| } |  |
| } |  |
| …… |  |

slice_reshaper_model_present_flag equal to 1 specifies slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 specifies slice_reshaper_model() is not present in slice header. When slice_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

slice_reshaper_enable_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enable_flag equal to 0 specifies that reshaper is not enabled for the current slice. When slice_reshaper_enable_flag is not present, it is inferred to be equal to 0.

slice_reshaper_chromaAdj_flag equal to 1 specifies that chroma residue scaling is enabled for the current slice. slice_reshaper_chromaAdj_flag equal to 0 specifies that chroma residue scaling is not enabled for the current slice. When slice_reshaper_chromaAdj_flag is not present, it is inferred to be equal to 0.

# Simulation results

The proposed universal low complexity reshaper is implemented in BMS2.0.1 software with VTM tool setting (JEM_TOOLS=0) and with JEM-comparable HDR implementation (WCG_EXT=1). The decoding process is implemented using fixed point. The test was done by disabling BMS tools and using VTM CTC with AI and RA. AVX2 is used in run time for SIMD optimization. The encoding time is measured using cluster. The decoding time is measured using a PC station specified by Table 3. The following Tables 3-5 show BDRate for fixed QP test condition for SDR and HDR.

The tests show consistent gain for both SDR and HDR video. Under RA, the proposed universal reshaper has average -1.32% BDRate gain for Y for SDR video (average -1.43% for UHD and HD SDR video), and -1.8% BDRate gain for wPSNRY for HDR PQ video. Under AI, it has average -0.96% BDRate gain for Y for SDR video (average -1.36% for UHD and HD SDR content), and -1.7% BDRate gain for wPSNRY for HDR PQ video.

The proposed reshaper uses basic LUT operations to minimize computational complexity and facilitate implementation. LUTs are implemented using 2-byte (10-bit), 1024 entry integer arrays. All related operations, such as mapping and LUT construction, are performed with fixed point arithmetic at the decoder side. The proposed reshaper performs luma sample mapping in-place, meaning no extra storage buffer is needed for luma samples. In the software, there is no SIMD implementation for the proposal. The running time for simulation shown in this contribution is with SIMD AVX2 enabled. The decoding running time increase over VTM2.0.1 for both SDR and HDR is within 5%.  The decoding complexity is mainly due to FwdLUT and InvLUT construction and LUT mapping. The encoding time change for HDR is negligible. The encoding time for SDR is within 7%, which is mainly due to the calculation of pixel variance for every intra slice.

Table 3. Technical specification of the PC workstation for decoding

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

Table 4. BDRate for in-loop reshaper for SDR video (CE12-4): AI and RA

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | 2.57% | 0.68% | 108% | 106% |
| Class A2 | -1.53% | 2.39% | 2.48% | 107% | 105% |
| Class B | -0.84% | 2.55% | 2.16% | 105% | 103% |
| Class C | -0.40% | 2.75% | 2.88% | 107% | 105% |
| Class E | -0.24% | 2.50% | 2.17% | 113% | 107% |
| Overall | -0.96% | 2.56% | 2.13% | 107% | 105% |
| Class D | -0.54% | 2.99% | 2.61% | 107% | 105% |
| Class F (optional) | -0.71% | 1.81% | 1.82% | 106% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.69% | 3.31% | 2.22% | 106% | 107% |
| Class A2 | -1.01% | 2.13% | 2.09% | 101% | 105% |
| Class B | -1.53% | 1.52% | 0.64% | 106% | 105% |
| Class C | -1.00% | 1.76% | 2.05% | 111% | 103% |
| Overall | -1.32% | 2.07% | 1.62% | 106% | 105% |
| Class D | -0.48% | 2.14% | 2.58% | 109% | 101% |
| Class F (optional) | -0.92% | 1.19% | 1.33% | 106% | 105% |

Table 5. BDRate for in-loop reshaper for HDR video (CE12-2.1a ILFOPT=0): AI and RA

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |
|  | BD-rate |  |  |
|  | wPsnrY | PSNRL100 | DE100 |
| Market | -1.7% | -2.2% | 1.4% |
| SunRise | -1.1% | -1.5% | 4.2% |
| ShowGirl2 | -1.7% | -1.9% | -0.4% |
| BalloonFestival | -2.7% | -3.4% | 2.7% |
| Hurdles | 0.0% | -0.4% | 1.3% |
| Starting | -2.0% | -2.5% | 0.3% |
| Cosmos1 | -2.4% | -2.3% | 0.6% |
| Average HDR-B | -1.7% | -2.0% | 1.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Over VTM2.0.1 WCG_EXT=1 JEM_TOOLS=0 |  |  |
|  | BD-rate |  |  |
|  | wPsnrY | PSNRL100 | DE100 |
| Market | -1.3% | -1.4% | 2.3% |
| SunRise | -0.4% | -0.5% | 6.0% |
| ShowGirl2 | -1.6% | -1.9% | -0.7% |
| BalloonFestival | -2.8% | -3.3% | 2.9% |
| Hurdles | -1.9% | -2.4% | 2.1% |
| Starting | -2.4% | -2.9% | 1.9% |
| Cosmos1 | -2.0% | -1.9% | 1.6% |
| Average HDR-B | -1.8% | -2.0% | 2.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% |  |  |

# Conclusion

The proposed universal low complexity reshaper provides significant performance gain for both SDR and HDR video.

For SDR video, performance gains under VTM2.0.1 test conditions in terms of average BDRate for Y/U/V are:

- RA {-1.32%, 2.07%, 1.62%}

- AI { -0.96%, 2.56%, 2.13%}

Higher gain is achieved for higher resolution video (UHD and HD). [SDR video average BDRate for Y: RA, -1.43%; AI, -1.36%]

For HDR video, performance gains under VTM2.0.1 test conditions in terms of average BDRate for wPSNRY/PSNR100/DE100:

- RA{-1.8%, -2.0%, 2.3%}

- AI{ -1.7%, -2.0%, 1.4%}

The decoding running time increase is within 5%. The encoding running time increase is within 7%.

It is recommended that the proposed universal reshaper be adopted as a core coding tool in VVC for SDR and HDR.

# Acknowledgement

We would like to thank Technicolor for crosschecking the HDR test results and ByteDance and Qualcomm for crosschecking the SDR results.

# Reference

- E. Francois, D. Rusanovskyy, P. Yin, “Description of Core Experiment 12 (CE12): mapping functions”, JVET-K1032, Ljubljana, SI, July 2018.

- X. Xiu, P. T. Lu, Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12-2: HDR In-loop Reshaping”, JVET-L0245, Macao, CN, October 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12-4: SDR In-loop Reshaping”, JVET-L0246, Macao, CN, October 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Syntax on reshaper model

Table A-1. Reshaper model syntax in SPS

| sps_reshaper_model() { | descriptor |
| --- | --- |
| full_range_input_flag | u(1) |
| dQP_model_scale_int_prec | ue(v) |
| if (dQP_model_scale_int_prec > 0) { |  |
| dQP_model_scale_int | u(v) |
| } |  |
| dQP_model_scale_frac_prec_minus16 | ue(v) |
| dQP_model_scale_frac | u(v) |
| if (dQPModelScaleAbs) { |  |
| dQP_model_scale_sign_flag | u(1) |
| } |  |
| dQP_model_offset_int_prec_minus3 | ue(v) |
| dQP_model_offset_int | u(v) |
| dQP_model_offset_frac_prec_minus1 | ue(v) |
| dQP_model_offset_frac | u(v) |
| if (dQPModelOffsetAbs) { |  |
| dQP_model_offset_sign_flag | u(1) |
| } |  |
| dQP_model_abs_prec_minus3 | ue(v) |
| dQP_model_max_abs | u(v) |
| if (dQP_model_max_abs) { |  |
| dQP_model_max_sign_flag | u(1) |
| } |  |
| dQP_model_min_abs | u(v) |
| if (dQP_model_min_abs) { |  |
| dQP_model_min_sign_flag | u(1) |
| } |  |
| } |  |

full_range_input_flag equal to 0 specifies a standard dynamic range input video signal. full_range_input_flag equal to 0 specifies a full range input video signal.  When full_range_input_flag is not present, it is inferred to be equal to 0.

dQP_model_scale_int_prec specifies the number of bits used for the representation of dQP_model_scale_int.

dQP_model_scale_int specifies the integer value of dQP model scale. When dQP_model_scale_int is not present, it is inferred to be equal to 0.

dQP_model_scale_frac_prec_minus16 plus 16 specifies the number of bits used for the representation of dQP_model_scale_frac.

dQP_model_scale_frac specifies the fractional value of the dQP model scale.

The variable dQPModelScaleAbs is derived as:

dQPModelScaleAbs = dQP_model_scale_int << (dQP_model_scale_frac_prec_minus16 + 16) + dQP_model_scale_frac

dQP_model_scale_sign_flag specifies the sign of dQP model scale as follows:

- If dQP_model_scale_sign_flag is equal to 0, the corresponding variable dQPModelScale has a positive value.

- Otherwise (dQP_mode_scale_sign_flag is equal to 1), the corresponding variable dQPModelScale has a negative value.

When dQP_model_scale_sign is not present, it is inferred to be equal to 0.

The variable dQPModelScale = dQPModelScaleAbs * ( 1 – 2 * dQP_model_scale_sign_flag);

dQP_model_offset_int_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_offset_int.

dQP_model_offset_int specifies the integer value of dQP model offset.

dQP_model_ offset _frac_prec_minus1 plus 1 specifies the number of bits used for the representation of dQP_model_ offset_frac.

dQP_model_ offset_frac specifies the fractional value of the dQP model offset.

The variable dQPModelOffsetAbs is derived as:

dQPModelOffsetAbs = dQP_model_offset_int << (dQP_model_offset_frac_prec_minus1 + 1) + dQP_model_offset_frac

dQP_model_offset_sign_flag specifies the sign of dQP model offset as follows:

- If dQP_model_offset_sign_flag is equal to 0, the corresponding dQPModelOffset has a positive value.

- Otherwise (dQP_model_offset_sign_flag is equal to 1), the corresponding dQPModelOffset has a negative value.

When dQP_model_offset_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelOffset = dQPModelOffsetAbs * ( 1 – 2 * dQP_model_offset_sign_flag);

dQP_model_abs_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_max_abs and dQP_model_min_abs.

dQP_model_max_abs specifies the absolute value of the maximum dQP value.

dQP_model_max_sign_flag specifies the sign of the maximum dQP value as follows:

- If dQP_model_max_sign_flag is equal to 0, the corresponding dQPModelMax has a positive value.

- Otherwise (dQP_model_max_sign_flag is equal to 1), the corresponding dQPModelMax has a negative value.

When dQP_model_max_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMax = (dQP_model_max_abs) * ( 1 – 2 * dPQ_model_max_sign_flag ).

dQP_model_min_abs specifies the specifies the absolute value of the minimal dQP value.

dQP_model_min_sign_flag specifies the sign of the minimal dQP value as follows:

- If dQP_model_min_sign_flag is equal to 0, the corresponding dQPModelMin has a positive value.

- Otherwise (dQP_model_min_sign_flag is equal to 1), the corresponding dQPModelMin has a negative value.

When dQP_model_min_sign_flag is not present, it is inferred to be equal to 0.

The variable dQPModelMin = (dQP_model_min_abs) * ( 1 – 2 * dQP_model_min_sign_flag ).

Table A-2. Reshaper model syntax in slice header

| slice_reshaping_model() { | Descriptor |
| --- | --- |
| reshape_model_profile_type | u(4) |
| reshape_model_min_bin_idx | ue(v) |
| reshape_model_delta_max_bin_idx | ue(v) |
| reshape_model_num_cw_minus1 | ue(v) |
| for ( i = 0; i < reshape_model_num_cw_minus1 + 1; i++) { |  |
| reshape_model_delta_abs_CW [ i ] | u(5) |
| if ( reshape_model_delta_abs_CW > 0 ) |  |
| reshape_model_delta_sign_CW_flag [ i ] | u(1) |
| } |  |
| for ( i = reshape_model_min_bin_idx; i <= reshape_model_max_bin_idx; i++ ) { |  |  |
| reshape_model_bin_profile_delta [ i ] | ue(v) |
| } |  |
| } |  |

reshape_model_profile_type specifies the profile type to be used in the reshaper construction process.

reshape_model_min_bin_idx specifies the minimum bin index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to 31, inclusive.

reshape_model_delta_max_bin_idx is set equal to the maximum allowed bin index (31) minus the maximum bin index to be used in the reshaper construction process.

reshape_model_num_cw_minus1 plus 1 specifies the number of codewords to be signalled.

reshape_model_delta_abs_CW [ i ] specifies the i-th absolute delta codeword value.

reshape_model_delta_sign_CW_flag [ i ] specifies the sign of the i-th delta codeword as follows:

- If reshape_model_delta_sign_CW_flag [ i ] is equal to 0, the corresponding variable RspModelDeltaCW [ i ] has a positive value.

- Otherwise (reshape_model_delta_sign_CW_flag [ i ]  is equal to 0), the corresponding variable RspModelDeltaCW [ i ] has a negative value.

When reshape_model_delta_sign_CW_flag [ i ] is not present, it is inferred to be equal to 0.

The variable RspModelDeltaCW[i] = (1 - 2* reshape_model_delta_sign_CW [i]) * reshape_model_delta_abs_CW [ i ];

The variable OrgCW is set equal to 1 << ( BitDepthY – 5 );

The variable RspModelCW [ i ] = OrgCW + RspModelDeltaCW [ i ].

reshape_model_bin_profile_delta [ i ]  specifies the delta value to be used to adjust the profile of the i-th bin in the reshaper construction process.

The variable RspCW[i] for ith bin is derived as following:

- if reshape_model_bin_profile_delta [ i ]  is equal to 0, RspCW [i] = OrgCW + 1.

- Otherwise, CW [i] = RspModelCW[ j-1 ] when  reshape_model_bin_profile_delta [ i ] is equal to j.