| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0255 |
| --- | --- |

| Title: | CE10.2.3: A simplified design of overlapped block motion compensation based on the combination of CE10.2.1 and CE10.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +886-3-5670766 zhi-yi.lin@ mediatek.com +1-858-210-4830 xiaoyu.xiu@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 10.2.3 for simplified design of overlapped motion compensation (OBMC). The CE test is the combination of CE10.2.1 with OBMC bandwidth reduction, OBMC line buffer reduction, parallel OBMC sample blending, OBMC merge and encoder L-shape buffer reuse and CE10.2.2 with OBMC early termination based on motion vector (MV) similarity.

Two sets of simulation results are provided in this contribution. In Test One (CE10.2.3.a), the OBMC is only enabled at coding unit (CU) boundaries. In Test Two (CE10.2.3.b), the OBMC is enabled at both CU boundaries and sub-block boundaries inside one CU. Compared to the VTM-2.0.1, for Test One, the proposed solution provides average {Y, U, V} BD-rate savings of {0.75%, 1.36%, 1.51%} and {1.10%, 1.19%, 1.12%} for RA and LD, respectively. The encoding and decoding time are 102% and 105% for RA and 103% and 107% for LD. For Test Two, the corresponding {Y, U, V} BD-rate savings are {1.02%, 1.82%, 1.94%} and {1.43%, 2.02%, 1.98%} for RA and LD, respectively. The encoding and decoding time are 105% and 112% for RA and 106% and 115% for LD.

# Introduction

Overlapped block motion compensation (OBMC) is one coding tool in joint exploration model (JEM)-7.0 [1], which reduce the blocking artifacts at the motion compensation stage by combining multiple prediction signals using the motion vectors (MVs) of spatial neighboring blocks of the current coding unit (CU). Although it can improve the coding efficiency, the OBMC also introduces complexity increase to both encoder and decoder.

This contribution proposes a combination of CE10.2.1 and CE10.2.2 to reduce the OBMC complexity. Specifically, in this combined CE test, the following modifications are applied on top of the current OBMC design in the JEM-7.0.

CE10.2.1

OBMC bandwidth reduction at block boundaries: reference padding is applied to replicate the samples on the boundary of regular reference area for regular motion compensation (MV) to the additional reference samples that are needed by the OBMC. By this change, it can ensure that the number of reference samples that needs to be accessed by the OBMC is kept the same as that of regular bi-prediction.

OBMC line-buffer reduction: to reduce the line-buffer required by the OBMC, if the current CU is located at the top row of one CTU, the rows of the samples that need to be modified by the OBMC from its top neighbors are reduced from 4 to 2.

OBMC sample blending: instead of doing the OBMCs of one block in order of top, left, bottom and right, the predication samples from neighboring block are blended with equal weighting, i.e., independent on the order of apply the OBMCs from different neighbors.

OBMC merge: for the top and left boundaries of one OBMC CU, if adjacent neighboring blocks share the same MV, the OBMCs from those neighboring blocks are merged into one single OBMC with enlarged block size.

L-shape buffer reuse (encoder-side only): one L-shape buffer is used to store the OBMC samples located at the first row and the first column of neighboring 4x4 blocks. The OBMC samples stored in the buffer can be shared by different coding mode during rate-distortion optimization (RDO) process of the CU.

Additionally, the OBMC is always enabled when the size of the current CU is no smaller than 64 with removed obmc_flag in bit-stream.

CE10.2.2

Some OBMC from one spatial neighboring block is skipped based on the similarity between the MVs of the current block and the neighboring block. Specifically, each time before OBMC is invoked using the motion information from a given neighboring block, the neighboring MV(s) is compared to the current block’s MV(s) based on the following three conditions:

- If all the prediction lists (i.e., either L0/L1 in uni-prediction or both L0 and L1 in bi-prediction) that are used by the neighboring block are also used for the prediction of the current block;

- If the same reference picture(s) are used by the neighboring MV(s) and the current MV(s);

- If MV difference between the neighboring MV(s) and the current MV(s) is no larger than pre-defined MV difference threshold (both L0 and L1 MVs are checked if bi-pred is used).

If all the 3 conditions are met, then motion compensation is not performed (this is applicable to luma and chroma) using that given neighboring motion. In the current implementation, two different threshold values i.e., 1.5-pel and 1-pel, are applied when the neighboring block is uni-predicted or bi-predicted.

# Simulation results

The proposed method was implemented based on the BMS-2.0.1 reference software [2] and tested using the VTM configuration. The test condition and evaluation criteria specified in CE10 [3] are used for the simulations.

## CE10.2.3.a

The simulation results are provided in the section when only enabling the proposed OBMC method to the prediction samples on CU boundaries.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.90% | -1.15% | 101% | 102% |
| Class A2 | -0.73% | -1.52% | -1.66% | 102% | 104% |
| Class B | -0.65% | -1.45% | -1.47% | 103% | 104% |
| Class C | -1.20% | -1.46% | -1.71% | 103% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.75% | -1.36% | -1.51% | 102% | 105% |
| Class D | -0.94% | -1.31% | -1.04% | 103% | 111% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.86% | -1.35% | -1.28% | 103% | 105% |
| Class C | -1.33% | -1.17% | -1.27% | 103% | 111% |
| Class E | -1.20% | -0.93% | -0.67% | 103% | 105% |
| Overall | -1.10% | -1.19% | -1.12% | 103% | 107% |
| Class D | -1.32% | -0.01% | 0.10% | 103% | 109% |

Additionally, supplementary results are provided without the OBMC parallel blending being applied.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.86% | -1.18% | 102% | 102% |
| Class A2 | -0.71% | -1.47% | -1.62% | 102% | 104% |
| Class B | -0.65% | -1.42% | -1.48% | 103% | 104% |
| Class C | -1.19% | -1.52% | -1.67% | 103% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.74% | -1.34% | -1.50% | 102% | 105% |
| Class D | -0.94% | -1.19% | -1.03% | 103% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.83% | -1.33% | -1.36% | 103% | 105% |
| Class C | -1.32% | -1.34% | -1.41% | 103% | 109% |
| Class E | -1.20% | -1.74% | -0.82% | 103% | 106% |
| Overall | -1.09% | -1.44% | -1.25% | 103% | 107% |
| Class D | -1.24% | -0.05% | -0.15% | 103% | 110% |

## CE10.2.3.b

The simulation results are provided in the section when only enabling the proposed OBMC method to the prediction samples on both CU boundaries and sub-CU boundaries inside one CU.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.97% | -1.35% | 103% | 105% |
| Class A2 | -1.24% | -2.10% | -2.09% | 104% | 114% |
| Class B | -0.83% | -1.84% | -1.79% | 105% | 111% |
| Class C | -1.49% | -2.21% | -2.46% | 106% | 118% |
| Class E |  |  |  |  |  |
| Overall | -1.02% | -1.82% | -1.94% | 105% | 112% |
| Class D | -1.40% | -2.25% | -2.11% | 107% | 123% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.06% | -1.66% | -1.77% | 106% | 113% |
| Class C | -1.74% | -1.96% | -2.62% | 107% | 121% |
| Class E | -1.63% | -2.70% | -1.48% | 106% | 111% |
| Overall | -1.43% | -2.02% | -1.98% | 106% | 115% |
| Class D | -1.77% | -1.69% | -1.98% | 108% | 125% |

# References

J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of joint exploration test mode (JEM7)”, JVET-G1001, August. 2017, Torino, Italy.

BMS-2.0.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1.

C.-W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction”, JVET-K1030, July 2018, Ljubljana, Slovenia.

# Patent rights declaration(s)

MediaTek Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).