| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0256_v2 |
| --- | --- |

| Title: | CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Bi-directional optical flow (BIO) is include in benchmark set (BMS)-2.1 to enhance the conventional bi-prediction by improving both the granularity and the accuracy of the motion vectors (MVs) at motion compensation stage. Complexity analysis shows the current BIO implementation in the BMS-2.1 requires 33-bit multiplier and has the maximal bit-width of 43-bit for intermediate parameters. Additionally, when the BIO is enabled, the numbers of multiplications are 329% of the worst-case numbers of regular bi-prediction for 4x4 blocks, representing a significant increase in worst-case computation complexity.

In this contribution, two methods are proposed to address the BIO’s complexity issues. Firstly, one bit-width control method is proposed to ensure BIO can be implemented with at most 15-bit multiplier and the intermediate values are within the 32-bit range. Secondly, one method is proposed to lower the BIO’s computational complexity by using bilinear filters to interpolate prediction samples and reducing the area of extended region for BIO derivation. Further, BIO is disabled if either of the following is true: 1) the CU has height equal to 4 or the CUs is in size of 4×8, or 2) the CU is coded with sub-block modes. Analysis results show that with the proposed complexity reduction methods the worst-case number of multiplications involved in the BIO can be reduced to 103% of that of regular bi-prediction.

Experimental results show that compared to the VTM-2.1, the combination of all the above simplifications provides average {Y, U, V} BD-rate savings of {1.28%, 0.52% and 0.37%} with encoding and decoding time of 102% and 102%, respectively. The existing BIO implementation in the BMS-2.1 provides average {Y, U, V} BD-rate saving of {1.41%, 0.58%, 0.40%} with encoding and decoding time of 103% and 113%. Based on the simulation results, it is asserted that with the proposed methods, BIO becomes an implementable tool with desirable performance vs. complexity trade-off.

# Introduction

At the 11th JVET meeting, bi-directional optical flow (BIO) was included in the benchmark set (BMS)-2.1 reference software [1], which provides the motion refinements for 4×4 sub-blocks within one bi-predicted CU that has both forward and backward prediction signals. The motion refinement vx,vy  of each 4×4 sub-block is calculated by minimizing the difference between L0 and L1 prediction samples after the BIO is applied inside one 6×6 window Ω around the sub-block. Specifically, the value of vx,vy  is derived as

| vx=S1+r>m?clip3-thBIO,thBIO,-S3≫log2S1+r:0 vy=S5+r>m?clip3-thBIO,thBIO,-S6-vxS2≫log2S5+r:0 | (1) |
| --- | --- |

where ∙ is the floor function; thBIO is the motion refinement threshold to prevent the propagated errors due to irregular local motion, which is equal to 218-BD; r and m are the regulation parameters which are defined as

| r=500∙4BD-8 m=700∙4BD-8 | (2) |
| --- | --- |

Further, the values of S1, S2, S3, S5 and S6 are calculated as

| S1=(i,j)∈Ωψxi,j∙ψxi,j, S3=(i,j)∈Ωθi,j∙ψxi,j∙2L S2=(i,j)∈Ωψxi,j∙ψyi,j S5=i,j∈Ωψyi,j∙ψyi,j∙2 S6=(i,j)∈Ωθi,j∙ψyi,j∙2L+1 | (3) |
| --- | --- |

where

| ψxi,j=∂I1∂xi, j+∂I0∂xi, j ψyi,j=∂I1∂yi, j+∂I0∂yi, j θi,j=I1i, j-I0i, j | (4) |
| --- | --- |

where L is the internal bit-depth increase to keep data precision of the BIO derivation, which is set to 5 in the BMS-2.1 [1]; Iki, j are the sample value at coordinate i, j of the prediction signal in list k, k=0,1, which are generated at intermediate high precision (i.e., 16-bit); ∂Ik∂xi, j and ∂Ik∂yi, j are the horizontal and vertical gradients of the sample that are obtained by directly calculating the difference between its two neighboring samples, i.e.,

| ∂Ik∂xi, j=Iki+1, j-Iki-1, j≫4 ∂Ik∂yi, j=Iki, j+1-Iki, j-1≫4 | (5) |
| --- | --- |

Based on the motion refinement derived in (1), the final bi-prediction samples of the CU are calculated by interpolating the L0/L1 prediction samples along the motion trajectory based on the optical flow model, as indicated by

| predBIOx,y=I0x,y+I1x,y+b+οoffset≫shift b=rndvx∂I1x, y∂x-∂I0x, y∂x2L+1+rndvy∂I1x, y∂y-∂I0x, y∂y2L+1 | (6) |
| --- | --- |

where shift and οoffset are the offset and right shift that are applied to combine the L0 and L1 prediction signals for bi-prediction, which are equal to 15-BD and 1≪14-BD+2∙1≪13, respectively; rnd. is rounding function.

# BIO complexity analysis

## Bit-width analysis of the BIO

Assuming the input video is 10-bit (i.e., BD=10), Table 1 summarizes the bit-widths of the intermediate parameters that are used at each BIO stage as illustrated in Sec. 1.

Table 1 The bit-widths of intermediate parameters of the BIO design in BMS-2.1

| Operation | Parameter | Bit-width |
| --- | --- | --- |
| L0/L1 prediction | I0i, j, I1i, j | 16 |
| Gradient derivation | ∂I0∂xi, j, ∂I0∂yi, j | 13 |
|  | ∂I1∂xi, j, ∂I1∂yi, j | 13 |
| Correlation parameter calculation | θi,j=I1i, j-I0i, j | 17 |
|  | ψxi,j=∂I1∂xi, j+∂I0∂xi, j | 14 |
|  | ψyi,j=∂I1∂yi, j+∂I0∂yi, j | 14 |
|  | ψxi,j∙ψxi,j | 27 |
|  | ψxi,j∙ψyi,j | 27 |
|  | θi,j∙ψxi,j∙25 | 35 |
|  | ψyi,j∙ψyi,j∙2 | 28 |
|  | θi,j∙ψyi,j∙26 | 36 |
| Summation | S1=(i,j)∈Ωψxi,j∙ψxi,j | 33 |
|  | S2=(i,j)∈Ωψxi,j∙ψyi,j | 33 |
|  | S3=(i,j)∈Ωθi,j∙ψxi,j∙25 | 41 |
|  | S5=(i,j)∈Ωψyi,j∙ψyi,j∙2 | 34 |
|  | S6=(i,j)∈Ωθi,j∙ψyi,j∙26 | 42 |
| Motion refinement derivation | vx=S1+r>m?clip3-thBIO,thBIO,-S3≫log2S1+r:0 | 9 |
|  | S6-vxS2 | 43 |
|  | vy=S5+r>m?clip3-thBIO,thBIO,-S6-vxS2≫log2S5+r:0 | 9 |
| Final bi-prediction generation | vx∙∂I1∂xi, j-∂I0∂xi, j | 22 |
|  | vy∙∂I1∂yi, j-∂I0∂yi, j | 22 |

As can be seen from Table 1, the extreme bit-width of the whole BIO process happens at the calculation of the vertical motion refinement vy in (1) where S6 (42-bit) is subtracted by the multiplication of vx (9-bit) and S2 (33-bit). Therefore, the maximum bit-width of the existing BIO design is equal to 42 + 1 = 43 bit. Additionally, as the multiplication (i.e., vxS2) takes S2 as input, 33-bit multiplier becomes necessary to calculate the value of vy. Therefore, the BIO implementation in the BMS-2.1 [1] would require 33-bit multiplier and has the maximal bit-width of 43-bit for intermediate parameters.

## Computational analysis of the BIO

In the following, the number of operations (i.e., multiplications and additions) that are performed at each stage of generating the final BIO bi-prediction signal in the BMS-2.1 [1] are studied. To facilitate the discussion, it is assumed the current CU size is W×H where W is the width and H is the height of the CU.

- The generation of the L0 and L1 prediction samples

As discussed earlier, to derive the BIO motion refinement of each 4×4 block, both the prediction samples and the gradients need to be calculated for all the samples in a 6×6 surrounding window. Therefore, the gradients of (W+2)×(H+2) samples are needed in total. Additionally, as indicated in (5), both the horizontal and vertical gradients are calculated as the difference between two neighboring samples. So, the calculation of (W+2)×(H+2) gradient values would need the prediction samples in a (W+4)×(H+4) region for both lists L0 and L1. Based on the separable 8-tap filters for motion compensation (MC), both the numbers of multiplications and additions that are needed to generate L0 and L1 prediction samples are equal to ((W+4)×(H+4+7)×8+(W+4)×(H+4)×8)×2.

- Gradient calculation

As shown in (5), only one addition is needed per sample to derive one gradient. Considering both horizontal and vertical gradients in one extended region (W+2)×(H+2) are needed for both L0 and L1, the total number of additions that are required for the gradient derivation is equal to ((W+2)×(H+2))×2×2.

- Correlation parameter calculation

As shown in (3) and (4), there are five correlation parameters (i.e., S1, S2, S3, S5 and S6) that need to be calculated for all the samples in the extended region (W+2)×(H+2). Additionally, there are five multiplications and three additions required to calculate the five parameters at each sample position. Therefore, the total numbers of multiplications and additions for calculating the correlation parameters are equal to ((W+2)×(H+2))×5 and ((W+2)×(H+2))×3, respectively.

- Summation

The motion refinements are separately derived for each 4x4 block within one BIO CU. To derive the motion refinement of each 4x4 sub-block, it needs to calculate the sums of the five correlation parameters inside the 6x6 surrounding area. Therefore, the summation of the five correlation parameters (as indicated in (3)) needs in total (W/4)×(H/4)×6×6×5 additions.

- Motion refinement derivation

As shown in (1), to derive one local motion refinement vx,vy , there are two additions for adding the regulation parameter r to S1 and S3. Additionally, there are one multiplication and one addition needed to calculate the value of vy. So, to derive the motion refinements for all 4x4 blocks inside the CU, the numbers of multiplications and additions are equal to (W/4)×(H/4) and (W/4)×(H/4)×3, respectively.

- Generation of the bi-prediction signal

As shown in (6), given the motion refinements, two multiplications and six additions are further needed to derive the final predication sample at each position. Therefore, there are in total W×H×2 multiplications and W×H×6 additions that need to be performed at this stage.

- Early termination

In BMS-2.1 [1], the BIO can be early skipped based on the SAD between L0 and L1 prediction samples of the CU. Therefore, there are W×H ×2 -1 additions that are needed to calculate the SAD value.

Based on the above analysis, Table 2 and Table 3 depict the numbers of multiplications and additions that need to be applied per sample when the BIO is enabled, and compares them to the corresponding complexity of regular 4×4 bi-prediction that corresponds to the worst-case MC complexity. For 4x4 bi-prediction, based on the 8-tap interpolation filters, the total amounts of multiplications and additions are equal to (4×(4+7)×8+4×4×8)×2 = 960 (i.e., 60 per sample) and (4×(4+7)×8+4×4×8)×2+4×4×2=992 (i.e., 62 per sample). In both tables, the last column shows the percentage of the corresponding BIO complexity relative to that of 4x4 bi-prediction. Due to limited space, the complexity data of the CU sizes which are far smaller than the worst-case complexity of regular bi-prediction are not included in Table 2 and Table 3. Readers are referred to the attached spreadsheets for a complete BIO complexity analysis for different CU sizes.

Table 2 The number of multiplications per sample performed by the BIO in the BMS-2.1, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 184.0 | 0.0 |  | 11.3 | 0.0 | 0.1 | 2.0 | 0.0 | 197.3 | 329% |
| 4×8 | 124.0 | 0.0 |  | 9.4 | 0.0 | 0.1 | 2.0 | 0.0 | 135.4 | 226% |
| 8×4 | 138.0 | 0.0 |  | 9.4 | 0.0 | 0.1 | 2.0 | 0.0 | 149.4 | 249% |
| 4×16 | 94.0 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 104.5 | 174% |
| 16×4 | 115.0 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 125.5 | 209% |
| 4×32 | 79.0 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 89.0 | 148% |
| 32×4 | 103.5 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 113.5 | 189% |
| 4×64 | 71.5 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 81.3 | 135% |
| 64×4 | 97.8 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 107.5 | 179% |
| 4×128 | 67.8 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 77.4 | 129% |
| 128×4 | 94.9 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 104.6 | 174% |
| 8×8 | 93.0 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 102.9 | 171% |

Table 3 The number of additions per sample performed by the BIO in the BMS-2.1, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 184.0 | 9.0 |  | 6.8 | 11.3 | 0.2 | 6.0 | 1.9 | 219.1 | 353% |
| 4×8 | 124.0 | 7.5 |  | 5.6 | 11.3 | 0.2 | 6.0 | 2.0 | 156.5 | 252% |
| 8×4 | 138.0 | 7.5 |  | 5.6 | 11.3 | 0.2 | 6.0 | 2.0 | 170.5 | 275% |
| 4×16 | 94.0 | 6.8 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 125.2 | 202% |
| 16×4 | 115.0 | 6.8 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 146.2 | 236% |
| 4×32 | 79.0 | 6.4 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 109.6 | 177% |
| 32×4 | 103.5 | 6.4 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 134.1 | 216% |
| 4×64 | 71.5 | 6.2 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 101.8 | 164% |
| 64×4 | 97.8 | 6.2 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 128.0 | 206% |
| 4×128 | 67.8 | 6.1 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 97.8 | 158% |
| 128×4 | 94.9 | 6.1 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 125.0 | 202% |
| 8×8 | 93.0 | 6.3 |  | 4.7 | 11.3 | 0.2 | 6.0 | 2.0 | 123.4 | 199% |

As shown in Table 2 and Table 3, compared to the worst-case complexity of regular bi-prediction, the computational complexity shows a significant increase after the BIO is enabled. The peak complexity increase comes from 4x4 BIO CUs, where the numbers of multiplications and additions are 329% and 353% of that of worst-case bi-prediction.

# Proposed methods

In the following, two methods are proposed to address the complexity issues as mentioned earlier for practical hardware implementation of the BIO.

## The proposed BIO bit-width control method

One improved bit-width control method is proposed to reduce the bit-width required by the BIO. In the proposed method, the BIO parameters ψxi,j, ψyi,j and θi,j in (4) are firstly shifted down by na and nb bits, respectively, to reduce the overall bit-width of intermediate parameters:

| ψxi,j=∂I1∂xi, j+∂I0∂xi, j≫na ψyi,j=∂I1∂yi, j+∂I0∂yi, j≫na θi,j=I1i, j≫nb-I0i, j≫nb | (7) |
| --- | --- |

Moreover, the original L-bit internal bit-width increase is removed in the proposed method. With such change, the equations in (3) for calculating the BIO correlation parameters become

| S1=(i,j)∈Ωψxi,j∙ψxi,j, S3=(i,j)∈Ωθi,j∙ψxi,j S2=(i,j)∈Ωψxi,j∙ψyi,j S5=i,j∈Ωψyi,j∙ψyi,j S6=(i,j)∈Ωθi,j∙ψyi,j | (8) |
| --- | --- |

Calculation of the motion refinement vx,vy  is simplified by removing the regulation parameters r and m, and is performed as follows:

| vx=S1>0?clip3-thBIO',thBIO',-S3∙2nb-na≫log2S1:0 vy=S5>0?clip3-thBIO',thBIO',-S6∙2nb-na-vxS2,m≪nS2+vxS2,s2≫log2S5:0 | (10) |
| --- | --- |

Where S2,m=S2≫nS2 , S2,s=S2&2nS2-1, and thBIO'=213-BD.

Finally, due to the removed L-bit internal bit-width increase, the value of the delta b that is applied to enhance the bi-prediction sample in (6) is simplified as

| b=rndvx∂I1x, y∂x-∂I0x, y∂x2+rndvy∂I1x, y∂y-∂I0x, y∂y2 | (11) |
| --- | --- |

In the contribution, the values  naand nb are set equal to 3 and 6 and the value of nS2 is set equal to 12. Table 4 summarizes the bit-widths of the intermediate parameters when the proposed bit-width control method is applied for the BIO. As shown in the table, the internal bit-width of the whole BIO process does not exceed 32-bit. Additionally, the multiplication with the worst possible input happens at the product of vxS2,m in (10) with inputs of 15-bit and 4-bit. Therefore, 15-bit multiplier is sufficient for BIO with the proposed bit-width reduction, which is the same bit-width for the multiplier required by regular MC.

Table 4 The bit-widths of intermediate parameters based on the proposed BIO bit-with control method

| Operation | Parameter | Bit-width |
| --- | --- | --- |
| L0/L1 prediction | I0i, j, I1i, j | 16 |
| Gradient derivation | ∂I0∂xi, j, ∂I0∂yi, j | 13 |
|  | ∂I1∂xi, j, ∂I1∂yi, j | 13 |
| Correlation parameter calculation | θi,j=I1i, j≫6-I0i, j≫6 | 11 |
|  | ψxi,j=∂I1∂xi, j+∂I0∂xi, j≫3 | 11 |
|  | ψyi,j=∂I1∂yi, j+∂I0∂yi, j≫3 | 11 |
|  | ψxi,j∙ψxi,j | 21 |
|  | ψxi,j∙ψyi,j | 21 |
|  | θi,j∙ψxi,j | 21 |
|  | ψyi,j∙ψyi,j | 21 |
|  | θi,j∙ψyi,j | 21 |
| Summation | S1=(i,j)∈Ωψxi,j∙ψxi,j | 27 |
|  | S2=(i,j)∈Ωψxi,j∙ψyi,j | 27 |
|  | S3=(i,j)∈Ωθi,j∙ψxi,j | 27 |
|  | S5=(i,j)∈Ωψyi,j∙ψyi,j | 27 |
|  | S6=(i,j)∈Ωθi,j∙ψyi,j | 27 |
| Motion refinement derivation | vx=S1>0?clip3-thBIO',thBIO',-S3∙23≫log2S1:0 | 4 |
|  | S2,m=S2≫12 | 15 |
|  | S2,s=S2&212-1 | 12 |
|  | vxS2,m≪12+vxS2,s2 | 30 |
|  | S6∙23-vxS2,m≪12+vxS2,s2 | 31 |
|  | vy=S5>0?clip3-thBIO',thBIO',-S6∙23-vxS2,m≪12+vxS2,s2≫log2S5:0 | 4 |
| Final bi-prediction generation | vx∙∂I1∂xi, j-∂I0∂xi, j | 17 |
|  | vy∙∂I1∂yi, j-∂I0∂yi, j | 17 |

## The proposed BIO computation complexity reduction method

As shown in Table 2 and Table 3, the complexity due to interpolating additional samples in the extended region is the complexity bottleneck of the BIO. Based on such observation, two methods are proposed in the following to reduce the worst-case computational complexity of the BIO.

- Solution one

In the first solution, it is proposed to use bilinear filters to replace the 8-tap filters for the generation of the prediction samples in the extended area of one BIO CU. Additionally, in the proposed method, the number of extended samples used by the BIO is reduced from two row/columns to one single row/column on each CU boundary, i.e., the extended region is reduced from (W+4)×(H+4) to (W+2)×(H+2). To compensate for the reduction in extended region, the proposed method sets both the sample values and the gradients of the positions outside the CU area (as indicated by the blank squares in Figure 1) to the values of their nearest neighbors on CU boundaries. Figure 1 illustrates the idea of the proposed BIO derivation method.

Figure 1 Illustration of the proposed BIO derivation method

To further reduce the BIO worst-case complexity, BIO is disabled CU sizes that significantly increase the worst-case complexity, including all the CUs with height equal to 4 and 4×8 CUs. Additionally, BIO is also disabled for all the CUs that are predicted with sub-block partitions (i.e., affine and ATMVP).

Table 5 and Table 6 show the numbers of operations per samples that are performed after solution one is applied. In the tables, the shadowed rows represent the CU sizes that disable the BIO; therefore, for those rows, the corresponding BIO-related operations are equal to 0 and their respective complexity are the same as regular bi-prediction of the CUs with equal size. As can been seen, with the proposed methods, the number of multiplications and additions in worst-case are reduced to 103% and 132% of that of regular bi-prediction.

Table 5 The number of multiplications per sample performed by Solution One, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 77% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×16 | 45.1 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 55.6 | 93% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×32 | 40.6 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 50.6 | 84% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×64 | 38.3 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 48.1 | 80% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×128 | 37.1 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 46.8 | 78% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 8×8 | 51.6 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 61.5 | 103% |

Table 6 The number of additions per sample performed by Solution One, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 48.0 | 77% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 4×16 | 45.1 | 6.8 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 76.4 | 123% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 4×32 | 40.6 | 6.4 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 71.1 | 115% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 4×64 | 38.3 | 6.2 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 68.5 | 111% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 4×128 | 37.1 | 6.1 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 67.2 | 108% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 100% |
| 8×8 | 51.6 | 6.3 |  | 4.7 | 11.3 | 0.2 | 6.0 | 2.0 | 82.0 | 132% |

In another comparison, Table 7 and Table 8 summarize the per-sample numbers of multiplications and additions of solution one relative to 8×8 bi-prediction which represents the worst-case MC complexity of the HEVC standard. As shown in Table 7 and Table 8, compared to the 8×8 bi-prediction, solution one increases the number of multiplications and additions by 34% and 71%.

Table 7 The number of multiplications per sample performed by Solution One compared to 8×8 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 8x8 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 100% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×16 | 45.1 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 55.6 | 121% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×32 | 40.6 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 50.6 | 110% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×64 | 38.3 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 48.1 | 105% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×128 | 37.1 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 46.8 | 102% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 8×8 | 51.6 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 61.5 | 134% |

Table 8 The number of additions per sample performed by Solution One compared to 8×8 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 48.0 | 100% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 4×16 | 45.1 | 6.8 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 76.4 | 159% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 4×32 | 40.6 | 6.4 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 71.1 | 148% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 4×64 | 38.3 | 6.2 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 68.5 | 143% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 4×128 | 37.1 | 6.1 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 67.2 | 140% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 2.0 | 0.0 | 62.0 | 129% |
| 8×8 | 51.6 | 6.3 |  | 4.7 | 11.3 | 0.2 | 6.0 | 2.0 | 82.0 | 171% |

- Solution two

Although the first method reduces the size of the extended region to (W+2)×(H+2), it still needs to interpolate additional samples beyond the CU boundaries. As shown in Table 5 and Table 6, the interpolation of those additional samples still brings some increases to the BIO worst-case complexity. To further reduce the complexity, in the second solution, it is proposed to directly use integer reference samples (without interpolation) as the extended samples (as indicated by the blank squares in Figure 1) in the extension region (W+2)×(H+2) for the BIO derivation. Additionally, as the same as the first solution, the second solution pads both the samples and gradients of the position in the extended region from their nearest neighbors on CU boundaries and disable BIO when either of the following is true: 1) the CU has height equal to 4 or the CUs is in size of 4×8, or 2) the CU is coded with sub-block modes.

Table 9 and Table 10 show the numbers of operations per samples that are performed after the solution two is applied. As shown in the two tables, due to the skipped interpolation of extended samples, solution two can reduce the worst-case number of multiplications to be the same as that of regular bi-prediction while only increasing the worst-case number of additions by 20%.

Table 9 The number of multiplications per sample performed by Solution Two, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 77% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×16 | 39.0 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 49.5 | 83% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×32 | 35.5 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 45.5 | 76% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×64 | 33.8 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 43.5 | 73% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 4×128 | 32.9 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 42.6 | 71% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 100% |
| 8×8 | 46.0 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 55.9 | 93% |

Table 10 The number of additions per sample performed by Solution Two, compared to 4×4 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 4x4 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 74% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 4×16 | 39.0 | 4.0 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 67.5 | 109% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 4×32 | 35.5 | 4.0 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 63.7 | 103% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 4×64 | 33.8 | 4.0 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 61.8 | 100% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 4×128 | 32.9 | 4.0 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 60.9 | 98% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 97% |
| 8×8 | 46.0 | 4.0 |  | 4.7 | 11.3 | 0.2 | 6.0 | 2.0 | 74.1 | 120% |

Table 11 and Table 12 summarize the per-sample numbers of multiplications and additions of solution two relative to 8×8 bi-prediction. As shown in Table 11, the worst-case complexity of bi-prediction now occurs at 4×4, 8×4, 16×4, 32×4, 64×4 and 128×4 CUs where the BIO is not applied. In other words, with solution two, the BIO does not increase the worst-case number of multiplications that are need for bi-prediction.

Table 11 The number of multiplications per sample performed by Solution Two, compared to 8×8 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 8x8 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 100% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×16 | 39.0 | 0.0 |  | 8.4 | 0.0 | 0.1 | 2.0 | 0.0 | 49.5 | 108% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×32 | 35.5 | 0.0 |  | 8.0 | 0.0 | 0.1 | 2.0 | 0.0 | 45.5 | 99% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×64 | 33.8 | 0.0 |  | 7.7 | 0.0 | 0.1 | 2.0 | 0.0 | 43.5 | 95% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 4×128 | 32.9 | 0.0 |  | 7.6 | 0.0 | 0.1 | 2.0 | 0.0 | 42.6 | 93% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 130% |
| 8×8 | 46.0 | 0.0 |  | 7.8 | 0.0 | 0.1 | 2.0 | 0.0 | 55.9 | 121% |

Table 12 The number of additions per sample performed by Solution Two, compared to 8×8 bi-pred

| CU size (W×H) |  |  | BIO |  |  |  |  |  |  | Percentage to 8x8 bi-pred |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | L0/L1 Pred | Gradient |  | Correlation | Summation | Motion refinement | Final bi-prediction | Early Skip | Total |  |
| 4×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 4×8 | 46.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 46.0 | 96% |
| 8×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 4×16 | 39.0 | 4.0 |  | 5.1 | 11.3 | 0.2 | 6.0 | 2.0 | 67.5 | 141% |
| 16×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 4×32 | 35.5 | 4.0 |  | 4.8 | 11.3 | 0.2 | 6.0 | 2.0 | 63.7 | 133% |
| 32×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 4×64 | 33.8 | 4.0 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 61.8 | 129% |
| 64×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 4×128 | 32.9 | 4.0 |  | 4.6 | 11.3 | 0.2 | 6.0 | 2.0 | 60.9 | 127% |
| 128×4 | 60.0 | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 60.0 | 125% |
| 8×8 | 46.0 | 4.0 |  | 4.7 | 11.3 | 0.2 | 6.0 | 2.0 | 74.1 | 154% |

## Discussion on BIO and DMVR

Additionally, in Table 13, the computational complexity of solution two and decoder-side motion vector derivation (DMVR) are compared for 8×8 CUs. In Table 13, the complexity data of the DMVR are obtained from JVET-K0480 [3] where the analysis is based on the original DMVR design in the JEM rather than the current DMVR design in BMS-2.1. As shown in Table 13, the computational complexity of the proposed BIO is better than that of the DMVR. Additionally, as the BIO separately derives the motion refinements based on 4×4 units, most of the calculations and executions involved in the BIO derivation for one CU can be carried out simultaneously. This makes the BIO very friendly for high-degree parallelism processing for both hardware and software. As demonstrated in Sec. 4, after properly reducing the operational bit-width, straightforward SIMD implementations can provide significant complexity improvement for the BIO. Therefore, it is asserted that the proposed BIO design can provide a better trade-off between performance and implementation cost.

Table 13 Computational complexity comparison between the proposed BIO and the DMVR in JEM

|  | Mul (total/per sample) | Add (total/per sample) |
| --- | --- | --- |
| The proposed BIO | 3578/55.9 | 4742/74.1 |
| The DMVR in JEM (JVET-K0480) | 4320/67.5 | 6194/96.8 |

# Simulation results

The proposed solutions are implemented based on the BMS-2.1 reference software [1] and tested using the VTM configuration. The test condition and evaluation criteria that are used in CE9 [2] are applied.

Additionally, the proposed internal bit-width reduction method makes it possible to do SIMD acceleration of the BIO process. Therefore, in the simulation, some BIO derivation processes, including gradient derivation, correlation parameter calculation, summation and motion refinement derivation, are implemented with SSE42 SIMD code, when the proposed bit-width control method is enabled.

## Performance of the BIO in the BMS-2.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -0.42% | -0.18% | 102% | 118% |
| Class A2 | -1.83% | -0.53% | -0.23% | 102% | 114% |
| Class B | -1.30% | -0.48% | -0.42% | 103% | 121% |
| Class C | -1.52% | -0.87% | -0.66% | 104% | 120% |
| Class E |  |  |  |  |  |
| Overall | -1.41% | -0.58% | -0.40% | 103% | 119% |
| Class D | -2.28% | -1.40% | -1.17% | 104% | 122% |

## Performance of the proposed BIO bit-width control method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -0.35% | -0.12% | 102% | 108% |
| Class A2 | -1.81% | -0.44% | -0.24% | 102% | 104% |
| Class B | -1.26% | -0.46% | -0.39% | 103% | 110% |
| Class C | -1.49% | -0.78% | -0.64% | 103% | 112% |
| Class E |  |  |  |  |  |
| Overall | -1.38% | -0.52% | -0.37% | 103% | 109% |
| Class D | -2.23% | -1.34% | -1.15% | 102% | 114% |

## Performance of the proposed BIO computational complexity reduction

- Solution one

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -0.38% | -0.14% | 103% | 113% |
| Class A2 | -1.71% | -0.48% | -0.24% | 103% | 109% |
| Class B | -1.23% | -0.44% | -0.43% | 103% | 115% |
| Class C | -1.39% | -0.78% | -0.67% | 103% | 115% |
| Class E |  |  |  |  |  |
| Overall | -1.31% | -0.53% | -0.40% | 103% | 113% |
| Class D | -2.05% | -1.26% | -1.15% | 103% | 117% |

- Solution two

## Performance of the proposed BIO bit-width control method and BIO computational complexity reduction

- Bit-width control + solution one

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -0.36% | -0.10% | 102% | 103% |
| Class A2 | -1.71% | -0.48% | -0.22% | 101% | 99% |
| Class B | -1.19% | -0.44% | -0.40% | 102% | 103% |
| Class C | -1.36% | -0.78% | -0.65% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.28% | -0.52% | -0.37% | 102% | 102% |
| Class D | -2.04% | -1.10% | -0.99% | 101% | 102% |

- Bit-width control + solution two

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -0.37% | -0.10% | 100% | 103% |
| Class A2 | -1.68% | -0.43% | -0.23% | 99% | 99% |
| Class B | -1.14% | -0.43% | -0.40% | 102% | 102% |
| Class C | -1.27% | -0.75% | -0.67% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.24% | -0.50% | -0.38% | 101% | 102% |
| Class D | -1.85% | -1.19% | -1.03% | 101% | 97% |

# References

BMS-2.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.1.

S. Esenlik, Y.-W. Chen, F. Chen, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-K1029, July 2018, Ljubljana, Slovenia.

M. Zhou, B. Heng, “Non-CE9: A computational complexity analysis for DMVR”, JVET-K0480, July 2018, Ljubljana, Slovenia.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).