| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0260-r2 |
| --- | --- |

| Title: | CE4-related: Affine motion estimation improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Xiaoyu Xiu, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Affine motion model was adopted in VTM-2.0 to improve the motion compensated prediction for more complicated motion beyond translational motion, such as rotation and zoom. Affine motion estimation at the encoder uses an iteration method based on optical flow and it is quite different from conventional motion estimation for translational motion model. In this contribution, encoder-side modifications are proposed to improve affine motion estimation method for better coding efficiency. Four modifications are included: adding more affine MV predictors for starting point selection; sub-block based affine motion estimation; additional affine MVs checking; and affine MV refinement. Experimental results show that compared to VTM-2.0.1 anchor, the proposed encoder modifications provide average {Y, U, V} BD-rate savings of {-0.32%, -0.14%, -0.28%}, {-0.21%, -0.22%, -0.14%} for RA and LD, respectively.

# Introduction

At the 11th JVET meeting, 4-parameter and 6-parameter affine motion models were adopted in VTM-2.0 reference software [1]. The motion vectors of each 4x4 sub-block within one affine coding block are derived using the MVs at the control points. The control points for 4-paramter affine model are defined as top-left and top-right. The control points for 6-paramter affine model are defined as top-left, top-right and bottom-left. Denote three control point’s MVs as (v0x, v0y), (v1x, v1y), (v2x, v2y), respectively, and denote the current coding block’s width as w and its height as h. The motion vector (MVx, MVy) of 4x4 sub-block centered at (x, y) is derived with MVs at control points by Eq.(1)(2) for 4-parameter affine model and Eq.(3)(4) for 6-parameter affine model.

| MVx= v0x+v1x-v0x*xw-v1y-v0y*yw | (1) |
| --- | --- |
| MVy= v0y+v1y-v0y*xw+v1x-v0x*yw | (2) |
| MVx= v0x+v1x-v0x*xw+v2x-v0x*yh | (3) |
| MVy= v0y+v1y-v0y*xw+v2y-v0y*yh | (4) |

For simplicity of explanation, we use 6-parameter affine model in the following parts. Affine ME in VTM-2.0 is an iteration process based on optical flow. At each iteration step, affine ME first estimates affine model parameters using optical flow, and then convert them to control point’s delta MVs relative to MVs at previous iteration. The control point’s MVs of current iteration step are derived by adding delta MVs and the MVs of previous iteration step.

The delta MV (dvxki,j, dvxki,j) of sample located at position (i, j) at step k can be represented with 6-parameter affine model as:

| dvxki,j=c*i+d*j+advyk(i, j)=e*i+f*j+b | (5) |
| --- | --- |

where (a, b) are translation parameters, (c, d) are zoom and rotation parameters for horizontal direction, and (e, f) are zoom and rotation parameters for vertical direction, respectively. The parameter k is the current iteration. Denote Ii,j and I'ki,j as the original luminance signal and the prediction luminance signal at step k, respectively. The spatial gradient gxk(i,j) and gyk(i,j) are derived by applying Sobel filter to the prediction signal I'k(i,j) in horizontal and vertical direction, respectively. Based on the optical flow equation, the relationship between the change of luminance and its spatial gradient and temporal movement (i.e. delta MV) is formulated as:

| I'ki,j-I(i,j)= gxki,j*dvxk(i,i)+gyki,j*dvyk(i,i) | (6) |
| --- | --- |

Substituting dvxk(i,j) and dvyk(i,j) in Eq.(6) with Eq.(5), we get the equation for parameter (a, b, c, d, e, f).

| I'ki,j-Ii,j= (gxki,j*i)*c +(gxki,j*j)*d+(gyki,j*i)*e+(gyki,j*j)*f +gxki,j*a+gyki,j*b | (7) |
| --- | --- |

Because Eq.(7) are applicable to samples in the current coding block, we solve (a, b, c, d, e, f) using least square method. The control point’s delta MVs at step k is calculated by Eq.(5) using control point’s position. The final control point’s MVs at step k is calculated by adding delta MV and the control point’s MVs of step (k-1). The iteration will stop until the delta MVs of all control points are all zeros or the number of iterations has reached the maximum number of iterations.

From above description, affine ME process is different from conventional ME process for translational motion model. The following aspects will affect the accuracy of affine ME of the current design: (a) the starting point of affine ME; (b) the sample based optical flow used in estimation does not match sub-block based affine motion compensation; (c) the MVD signaling overhead is not considered in the iteration; (d) it is difficult to control the granularity of control point’s MVs in affine ME. In this proposal, the following four aspects are proposed to improve the affine ME scheme:

(1) One coding block will be coded in different tree partitions such as non-split, BT, TT and QT. The affine ME is performed on each partition. Those partitions are spatially correlated and the previous estimated 4-parameter affine MVs can be used as predictor for current partition’s ME. A cyclic list is proposed to store those most recent estimated 4-parameter affine MVs and their block position and size, and all of them will be used in the starting point selection for current block’s 4-parameter affine ME. This is aimed at improving the affine 4-parameter ME starting point.

(2) In the existing affine ME in VTM-2.0, each sample has different delta motion vector calculated with Eq.(5). In the proposed affine ME improvements, the delta MV of each sample within one 4x4 sub-block will have same delta MV as the center position of 4x4 sub-block. The Eq.(5) is replaced by Eq.(8), where (isb, jsb) is the center position of the sub-block that sample (i, j) belongs to. In this way, the sub-block based affine prediction is considered in the affine ME process.

| dvxki,j=c*isb+d*jsb+advyk(i, j)=e*isb+f*jsb+b | (8) |
| --- | --- |

(3) We check some additional affine MV candidates after existing affine ME. These additional MV candidates are generated by considering affine MVP to reduce the MVD signaling overhead. Suppose we have the control point MVs {MV0, MV1, MV2} for one given reference picture, and affine MV predictor for this reference picture is {MVP0, MVP1, MVP2}. We will check ME cost for the following combined MVs: {MVP0, MV1, MV2}, {MV0, MVP1, MV2}, {MV0, MV1, MVP2}, {MV0, MVP1, MVP2}.

(4) Regarding 4-parameter affine search, from Eqs.(1)-(2), we know MVs at top-right control point is related to rotation and zoom motion. We will refine top-right MV after existing 4-parameter affine ME. Given affine control point’s MVs {MV0, MV1} in 1/16-pel unit after affine ME, the following affine MVs are checked: {MV0, (MV1x -4, MV1y)}, {MV0, (MV1x +4, MV1y)}, {MV0, (MV1x, MV1y -4)}, {MV0, (MV1x, MV1y +4)}.

Items (3) and (4) are applied after the existing affine ME in VTM-2.0 is performed, and they will refine the affine MVs at control points estimated from existing affine ME process.

# Simulation results

To evaluate the performance and complexity impact, the proposed encoder modifications were implemented based on the BMS-2.0.1 reference software and tested using the VTM configuration. The test condition and evaluation criteria that are used in CTC [2] are applied. In the simulation, the cyclic list size is set to 4 for RA and 3 for LD. To reduce the encoding complexity, improvement (3)(4) is only applied when affine ME cost is smaller than the translational ME cost. Table 1 lists the simulation results compared to VTM-2.0.1 anchor. The affine motion estimation improvements achieve average Y BD-rate reduction by 0.32% with 103% encoding time for RA and 0.21% for LB with 104% encoding time.

Table 1. Simulation results compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.14% | -0.18% | 103% | 101% |
| Class A2 | -0.45% | -0.29% | -0.47% | 103% | 100% |
| Class B | -0.31% | -0.19% | -0.34% | 103% | 101% |
| Class C | -0.23% | 0.02% | -0.12% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.14% | -0.28% | 103% | 101% |
| Class D | -0.18% | -0.15% | -0.21% | 103% | 102% |
| Class F (optional) | -0.31% | -0.30% | -0.31% | 102% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.25% | -0.22% | 104% | 101% |
| Class C | -0.16% | -0.10% | -0.16% | 103% | 98% |
| Class E | -0.40% | -0.34% | 0.03% | 107% | 99% |
| Overall | -0.21% | -0.22% | -0.14% | 104% | 100% |
| Class D | -0.20% | 0.08% | -0.04% | 103% | 99% |
| Class F (optional) | -0.64% | -0.26% | -0.33% | 104% | 102% |

# References

VTM-2.1 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.1.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, July 2018.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).