| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0262-v1 |
| --- | --- |

| Title: | CE 6-1.7a: MTS simplification by reusing DCT-2 partial butterfly - Change MTS transform to DST-4/DCT-4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe, Tadamasa Toma (Panasonic) Masaru Ikeda, Takeshi Tsukuba (Sony) Karam Naser, Fabrice Le Leannec, Edouard Francois (Technicolor) | Tel: Email: | karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com masaru.Ikeda@sony.com takeshi.tsukuba@sony.com |
| Source: | Panasonic, Sony, Technicolor |  |  |

_____________________________

# Abstract

This contribution reports results of CE6-1.7a. In this contribution, a new set of MTS transforms are proposed. The VTM2 MTS transforms DST-7 and DCT-8 are replaced by DST-4 and DCT-4 respectively. It is reported that the proposed change provides a simplification to the design of MTS, resulting in an average BD-rate luma loss of 0.2%, 0.2% and 0.0% in VTM2 in AI, RA and LDB configurations, respectively.

# Proposed method

In contributions [1][2][3], it has been shown that the transform basis functions of DST-4 and DCT-4 are similar to the current VTM2 [4] DST-7 and DCT-8 transforms. Additionally, the basis functions of N-point DST-4 and DCT-4 are contained in the ones of the 2N-point DCT-2. As DCT-2 is defined up to a size of 64, while MTS is restricted to a size of 32, replacing the existing transforms (DST-7 and DCT-8) by these transforms represent a high simplification of the transform design, since only DCT-2.

In this contribution, a direct replacement is considered, where the DST-7 and DCT-8 basis functions of all transforms sizes (4, 8, 16 and 32) are replaced by the basis functions of DST-4 and DCT-4.

# Experimental Results

Tests were generated using the conditions specified in CE6. VTM2 is used as anchor.

## Test-1: replacing DST-7 by DST-4 and DCT-8 by DCT-4

The following tables summarize the BD-rate results in AI, RA and LDB configurations. An average BD-rate loss in luma of 0.2%, 0.2% and 0.1% is observed in AI, RA and LDB configurations, respectively.

Table 1. BD-rate results of All Intra CTC.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.7% | 0.6% | 0.6% | 101% | 94% |
| Class A2 | 0.5% | 0.2% | 0.1% | 100% | 95% |
| Class B | 0.3% | 0.1% | 0.1% | 100% | 93% |
| Class C | -0.2% | -0.2% | -0.1% | 101% | 99% |
| Class E | 0.0% | -0.1% | -0.2% | 102% | 94% |
| Overall | 0.2% | 0.1% | 0.1% | 101% | 95% |
| Class D | -0.3% | -0.3% | -0.5% | 101% | 99% |
| Class F | -0.1% | -0.2% | -0.2% | 100% | 96% |

Table 2. BD-rate results of Random Access CTC.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.3% | 0.4% | 0.4% | 99% | 98% |
| Class A2 | 0.2% | 0.3% | 0.2% | 97% | 97% |
| Class B | 0.2% | 0.2% | 0.1% | 99% | 97% |
| Class C | 0.0% | 0.1% | 0.2% | 99% | 99% |
| Overall | 0.2% | 0.2% | 0.2% | 99% | 98% |
| Class D | -0.1% | -0.1% | 0.3% | 99% | 98% |
| Class F | 0.0% | -0.1% | 0.0% | 100% | 100% |

Table 3. BD-rate results of Low Delay B CTC.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.1% | 0.1% | 0.3% | 98% | 102% |
| Class C | 0.0% | -0.1% | -0.3% | 97% | 92% |
| Class E | 0.1% | 0.2% | -0.6% | 97% | 95% |
| Overall | 0.0% | 0.0% | -0.1% | 97% | 97% |
| Class D | 0.0% | 0.7% | -0.2% | 100% | 99% |
| Class F | 0.0% | 0.2% | -0.2% | 98% | 97% |

## Test-2: activation of MTS in inter CUs

Another test was carried out to examine the impact of MTS on Inter coding. Namely, the inter MTS was enabled by setting the following parameters:

- EMT = 3

- EMTFast = 3

In this test, DST-4/DCT-4 replace DST-7/DCT-8.

The corresponding results in RA and LDB configurations are given in the tables below. The usage of MTS in inter CUs leads to a performance of 0.1% luma BD-rate loss in RA and 0.2% gain in LDB.

Table 4. BD-rate results of Random Access CTC.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.3% | 0.1% | 0.3% | 101% | 98% |
| Class A2 | 0.3% | 0.2% | 0.2% | 100% | 98% |
| Class B | 0.2% | 0.0% | 0.1% | 101% | 96% |
| Class C | -0.2% | -0.1% | 0.1% | 101% | 97% |
| Overall | 0.1% | 0.1% | 0.2% | 100% | 97% |
| Class D | -0.3% | -0.3% | 0.0% | 101% | 99% |
| Class F | 0.0% | -0.3% | -0.2% | 101% | 100% |

Table 5. BD-rate results of low-delay B CTC.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.1% | -0.2% | 0.0% | 102% | 104% |
| Class C | -0.3% | -0.2% | -0.1% | 101% | 96% |
| Class E | -0.2% | 0.1% | 0.0% | 100% | 96% |
| Overall | -0.2% | -0.1% | 0.0% | 101% | 99% |
| Class D | -0.4% | 0.0% | 0.1% | 101% | 99% |
| Class F | -0.3% | -0.6% | -0.7% | 100% | 99% |

# Comments and Conclusion

This contribution proposes to replace the existing VTM MTS transforms by type IV transforms. This leads to a simplified design of MTS as the new transforms basis are inherited from the core DCT-2 transform. The simplification results in an average luma BD-rate loss of 0.2%, 0.2% and 0.0% in AI, RA and LDB configurations, respectively.

# Reference

- K. Abe, T. Toma, “CE6-related: Type4 only AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0130,

- K. Naser, F. Le Léannec, E. François, “CE6-related: Type4 only AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0265,

- T. Tsukuba, M. Ikeda, T. Suzuki, “CE6-related: AMT with only Type2/Type4 DCT/DST” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0394

- J. Chen, Y. Ye, S. Kim., “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002

# Patent rights declarations

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).