| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12Oct. 2018 | Document: JVET-L0264-v1 |
| --- | --- |

| Title: | CE6-related: Removed MTS CU-Flag and Reduced MTS Pairs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Leannec, Edouard Francois (Technicolor) | Tel: Email: | karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reduce the number of possible pairs of horizontal and vertical transforms in the MTS design from four to three. Indeed, it is reportedly observed that among the four transforms pairs of the current MTS design, the last pair is rarely used. This contribution proposes to remove the last pair and to augment the three remaining pairs with the core transform DCT-II for vertical and horizontal coding. By doing so, it is asserted that the MTS CU-flag does not need to be coded anymore; which removes one context. This reportedly results in simplification in both the encoder and decoder designs. The encoder time is reported to be reduced to 93-94% compared to the VTM2 anchor.

# Introduction

The current MTS design in VTM2‎[1] consists of the following sets of horizontal and vertical transforms:

g_aiTrSubsetIntra[3][2] = { { DST7, DCT8 }, { DST7, DCT8 }, { DST7, DCT8 } };

For every intra prediction mode, a mapping table is defined to select the one set out of the three sets from g_aiTrSubsetIntra, for vertical and horizontal transforms. Currently, g_aiTrSubsetIntra contains the same transforms for the 3 sets, which removes the dependency between the intra prediction mode and transform selection.

Whatever the prediction mode, there are four possible pairs that can be obtained from g_aiTrSubsetIntra:

- {DST7,DST7}

- {DST7,DCT8}

- {DCT8,DST7}

- {DCT8,DCT8}

The two-dimensional first basis functions of each of these pairs are shown in Figure 1. It can be seen that the first pair {DST7,DST7} well matches the general characteristics of the intra-predicted residuals, where the residual energy tends to increase as the residual samples are far from the reference samples. However, the last pair (DCT8,DCT8) is highly unexpected to be selected, since higher residual energy occurs near to the left/top reference samples, which indicates high difference between the reference samples and the samples to be predicted. Therefore, removing this pair (DCT8, DCT8) should have a negligible loss on the RD performance.

Figure 1. An example of the 2D first basis functions of MTS Pairs for 16x16 transform size: Top left (DST7,DST7), Top right (DST7, DCT8), Bottom left (DCT8 DST7) and Bottom right (DCT8, DCT8). Each pair in brackets represents (VerticalTransform, HorizontalTransform).

Removing this pair leads to a redundancy in the coding of the pair index, also called MTS transform index, since 2 bits will be used to code 3 pairs. To solve this, it is proposed to include the core transform {DCT2, DCT2} in the MTS pairs and to remove the MTS CU flag.

# Proposed method

It is proposed to remove the MTS CU index and to place the core transform {DCT2, DCT2} in the MTS transform pairs. Namely, the new MTS pairs are:

- {DCT2,DCT2}

- {DST7,DST7}

- {DST7,DCT8}

- {DCT8,DST7}

At the encoder side, this simplifies the intra coding search procedure that finds the optimal configuration for splitting, prediction and transform selection. This is because the current VTM software considers two passes for the intra search: one with MTS flag disabled and one with MTS flag enabled. With the proposed method, this is reduced to a single pass with MTS flag enabled only.

Moreover, the proposed method simplifies both the encoder and decoder entropy coding design, since the procedure for coding the MTS flag is released.

# Experimental Results

## Basic results

The proposed method was implemented in VTM2.0.1 software ‎[1]and tested within the common testing conditions on random access configuration. To summarize, the 1st tested proposal includes the following changes:

- Replacing {DCT8,DCT8} by {DCT2,DCT2}

- Removing the MST enable flag and using the MTS coding tree described in the previous section

With the default VTM software as anchor, the following results were obtained.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.13% | 0.27% | 0.45% | 94% | 101% |
| Class A2 | 0.03% | 0.14% | 0.07% | 94% | 101% |
| Class B | 0.07% | 0.10% | 0.13% | 94% | 101% |
| Class C | 0.10% | 0.31% | 0.24% | 93% | 99% |
| Overall | 0.08% | 0.20% | 0.21% | 94% | 100% |
| Class D | 0.09% | -0.13% | 0.08% | 93% | 98% |

A slight BD-rate luma loss of less than 0.1% is observed, with an encoding runtime decrease of 6%.

## Performance improvement with Alternative Transform Set

In comparison with the current VTM software, the JEM software had 4 transforms additionally to the default DCT2 transform, and the following sets were defined:

aiTrSubsetIntra[3][2] = { { DST7, DCT8 }, { DST7, DST1 }, { DST7, DCT5 } };

VTM2 software is restricted to have only DST7 and DCT8 as additional transforms. Compared to the JEM, DST1 and DCT5 are replaced by DCT8 (cftransform sets mentioned in Sec.1). However, a better replacement can be taken into consideration with DST1 and DCT5 being both replaced by DCT2. The following sets are now considered:

constintg_aiTrSubsetIntra[3][2] = { { DST7, DCT8 }, { DST7, DCT2 }, { DST7, DCT2 } };

As can be noticed, the second and third sets have a redundant pairs of {DCT2,DCT2}. Hence, it is more appropriate to apply the proposed method (removal of the MTS index and of the last pair, moving the core transform as the first pair of the MTS index set) in this scenario.

Using the same configuration in Sec. 3, the experimental results are:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.08% | 0.31% | 0.37% | 93% | 100% |
| Class A2 | -0.01% | 0.01% | 0.18% | 94% | 102% |
| Class B | 0.05% | 0.04% | 0.18% | 93% | 100% |
| Class C | 0.14% | 0.27% | 0.24% | 93% | 99% |
| Overall | 0.07% | 0.15% | 0.23% | 93% | 100% |
| Class D | 0.10% | 0.00% | 0.26% | 93% | 99% |

# Conclusion

This contribution proposes to reduce the VTM2 MTS pairs to three, via removing the last pair, and removing the MTS CU index. The core transform of DCT2 is signaled as a part of MTS. The first pair index corresponds to {DCT2,DCT2} and the rest are the first three pairs of current VTM2. This results in a simplified encoder search and simplified entropy coding with a little overall loss in RD performance of 0.1% Luma BD-rate in RA configuration.

# Reference

- J. Chen, Y. Ye, S. Kim., “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).