| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0266-v2 |
| --- | --- |

| Title: | CE4: History-based Motion Vector Prediction (Test 4.4.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents a History-based Motion Vector Prediction (HMVP) method for inter coding. In HMVP, a table of HMVP candidates is maintained and updated on-the-fly. After decoding a non-affine inter-coded block, the table is updated by adding the associated motion information as a new HMVP candidate to the last entry of the table. Constrained FIFO rule is applied to remove and add entries to the table. The HMVP candidates could be applied to either merge candidate list or AMVP candidate list. It is asserted that the line buffer size is kept unchanged compared to VTM. Simulation results reportedly show that HMVP with 6 entries of the table achieves 0.58%, 0.72% and 0.78% BD rate reduction for RA Main10 configuration with merge list size equal to 6, 8, and 10, respectively. When HMVP is only applied to the merge mode, it reports 0.43%, 0.59% and 0.65% BD rate reduction for RA Main10 configuration with merge list size equal to 6, 8, and 10, respectively.

# Introduction

In Versatile Video Coding (VVC), motion vectors of non-affine inter-coded blocks can be signaled in two ways: Advanced motion vector prediction (AMVP) mode or merge mode. With AMVP mode, a difference between the real motion vector and a motion vector prediction (MVP), a reference index and a MVP index referring to an AMVP candidate list are signaled. For the merge mode, a merge index referring to a merge candidate list is signaled and all the motion information associated with the merge candidate is inherited.

For both the AMVP candidate list and the merge candidate list, they are derived from temporally or spatially neighboring coded blocks. In the common test conditions, up to 6 merge candidate and up to 2 AMVP candidate may be added to the candidate list for motion vector prediction.

# Proposed method

A history-based MVP (HMVP) method is proposed wherein a HMVP candidate is defined as the motion information of a previously coded block. A table with multiple HMVP candidates is maintained during the encoding/decoding process. The table is emptied when a new slice is encountered. Whenever there is an inter-coded non-affine block, the associated motion information is added to the last entry of the table as a new HMVP candidate. The overall coding flow is depicted in Figure 2.

Figure 2. Decoding flow char with the proposed HMVP method

Figure 3. Example of updating the table in the proposed HMVP method

In this contribution, the table size S is set to be 6, which indicates up to 6 HMVP candidates may be added to the table. When inserting a new motion candidate to the table, a constrained FIFO rule is utilized wherein redundancy check is firstly applied to find whether there is an identical HMVP in the table. If found, the identical HMVP is removed from the table and all the HMVP candidates afterwards are moved forward, i.e., with indices reduced by 1.

HMVP candidates could be used in the merge candidate list construction process. The latest several HMVP candidates in the table are checked in order and inserted to the candidate list after the TMVP candidate. Pruning is applied on the HMVP candidates to the spatial or temporal merge candidate excluding sub-block motion candidate (i.e., ATMVP).

To reduce the number of pruning operations, three simplifications are introduced:

- Number of HMPV candidates to be check denoted by L is set as follows:

| L = (N <=4 ) ? M: (8 - N) | (1) |
| --- | --- |

wherein N indicates number of available non-sub block merge candidate and M indicates number of available HMVP candidates in the table.

- In addition, once the total number of available merge candidates reaches the signaled maximally allowed merge candidates minus 1, the merge candidate list construction process from HMVP list is terminated.

- Moreover, the number of pairs for combined bi-predictive merge candidate derivation is reduced from 12 to 6.

Similarly, HMVP candidates could also be used in the AMVP candidate list construction process. The motion vectors of the last K HMVP candidates in the table are inserted after the TMVP candidate. Only HMVP candidates with the same reference picture as the AMVP target reference picture are used to construct the AMVP candidate list. Pruning is applied on the HMVP candidates. In this contribution, K is set to 4 while the AMVP list size is kept unchanged, i.e., equal to 2.

# Complexity analysis

In terms of complexity analysis, two aspects are considered, including memory requirement and computational complexity increasement. For the additional memory required by HMVP, up to 6 motion candidates are required to be stored. That is, 6 * (32 bits for one MV + 4 bits for one reference index) * 2 = 54 bytes per slice.

For the computational complexity, it is mainly from the additional pruning operations. Two procedures of pruning operations are required by HVMP.

- One is invoked in the table updating process when inserting a new HMVP candidate, wherein up to 6 times of pruning operations are required.

- The other one is invoked in the merge list construction process. Thanks to the pruning process in the table updating process, all HMVP candidates in the table are always different from each other. Therefore, in the merge list construction process, there is no need for a second HMVP candidate to compare with a spatial or temporal candidate if it has been marked as identical to a first HMVP candidate. The number of additional pruning operations could be defined as C(NST, NmrgToBeAdded, NHMVP) wherein NST indicates the number of spatial or temporal merge candidates already in the merge candidate list, NmrgToBeAdded indicates the largest number of candidates from HMVP may be added to the merge list, and NHMVP indicates the number of HMVP candidates to be checked, respectively. In this contribution, NmrgToBeAdded is set to Nmrg - 1.

- When NHMVP is not smaller than NmrgToBeAdded, the worst case happens when the first NmrgToBeAdded - 1, candidates are all different from the first NST candidates and the remaining HMVP candidates are identical to one of the first NST candidates. In total, it requires  times of pruning operations.

- When NHMVP is smaller than NmrgToBeAdded, the worst case happens when all NHMVP, candidates are different from the first NST candidates which requires times of pruning operations.

The details of pruning operations for each value of number of existing spatial or temporal merge candidates denoted by NST and Nmrg in the worst case is shown Table 5.

Table 1. Additional pruning operations introduced by HMVP for the worst case

|  | Number of additional pruning operations in merge list construction process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NST = 0 | NST = 1 | NST = 2 | NST = 3 | NST = 4 | NST = 5 | NST = 6 |
| Nmrg = 6 | C(0, 5, 6) = 0 | C(1, 4, 6) = 4 | C(2, 3, 6) = 7 | C(3, 2, 6) = 9 | C(4, 1, 6) =10 | C(5, 0, 3) = 0 | C(6, 0, 2) = 0 |
| Nmrg = 8 | C(0, 7, 6) = 0 | C(1, 6, 6) = 6 | C(2, 5, 6) = 11 | C(3, 4, 6) = 15 | C(4, 3, 6) =18 | C(5, 2, 3) = 14 | C(6, 1, 2) = 11 |
| Nmrg = 10 | C(0, 9, 6) = 0 | C(1, 8, 6) = 6 | C(2, 7, 6) = 12 | C(3, 6, 6) = 15 | C(4, 5, 6) = 23 | C(5, 4, 3) = 15 | C(6, 3, 2) = 12 |

It should be noted that the above table just lists the worst case of number of pruning operations. In hardware implementation process, the throughput could be higher since the HMVP candidates are ready before decoding one block. Therefore, each HMVP could be compared with any of available merge candidates in parallel.

In addition, due to the reduced number of pairs for combined bi-predictive merge candidates generation process, i.e., from 12 to 6, it could be considered as the number of pruning operations reduced by 3, since there are 2 checks per pair for combined bi-predictive merge candidate generation process and 5 checks per pruning process. Therefore, the number of additional pruning operations for the whole proposal under the worst case is listed as follows:

Table 2. Additional pruning operations of the whole proposal (for the worst case)

|  | Number of pruning in table update | Number of additional pruning operations in merge list construction process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NST = 0 | NST = 1 | NST = 2 | NST = 3 | NST = 4 | NST = 5 | NST = 6 |
| Nmrg = 6 | 6 | -3 | 1 | 4 | 3 | 7 | -3 |  |
| Nmrg = 8 | 6 | -3 | 3 | 8 | 12 | 15 | 11 | 8 |
| Nmrg = 10 | 6 | -3 | 3 | 9 | 12 | 20 | 12 | 9 |

# Simulation results

The tables below show the BD-rate gain and processing time impact of the proposed approach. Two sets of simulations are conducted, following the common test conditions [1] on top of reference software VTM-2.0.1 [2] .

- Only enable HMVP for merge mode (Test 4.4.7.a)

  - Anchor: VTM (by disabling all BMS tools in BMS-1.0)

  - Test: Anchor with HMVP only applied to merge mode

- Enable HMVP for both AMVP and merge mode (Test 4.4.7.b)

  - Anchor: BMS

  - Test: Anchor with Proposal

For each test set, three sub-sets according to merge list size Nmrg set to 6, 8, and 10, are performed according to the CE4 descriptions.

## Simulation results of Test 4.4.7.a

This sub-section reports the coding performance of HMVP applied to merge mode only with Nmrg equal to 6, 8 and 10, respectively.

Table 3. Performance of Test 4.4.7.a under Random Access Main 10

|  | Nmrg = 6 |  |  |  |  | Nmrg = 8 |  |  |  |  | Nmrg = 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.36% | -0.60% | 101% | 102% | -0.40% | -0.42% | -0.76% | 101% | 101% | -0.41% | -0.48% | -0.78% | 102% | 102% |
| Class A2 | -0.37% | -0.68% | -0.66% | 100% | 101% | -0.56% | -0.98% | -0.92% | 100% | 102% | -0.65% | -1.07% | -1.11% | 101% | 102% |
| Class B | -0.60% | -0.70% | -0.74% | 101% | 102% | -0.84% | -1.03% | -1.05% | 101% | 103% | -0.92% | -1.00% | -1.06% | 102% | 103% |
| Class C | -0.34% | -0.24% | -0.49% | 105% | 104% | -0.45% | -0.57% | -0.74% | 104% | 103% | -0.48% | -0.49% | -0.66% | 103% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.43% | -0.51% | -0.63% | 102% | 102% | -0.59% | -0.78% | -0.88% | 102% | 102% | -0.65% | -0.77% | -0.91% | 102% | 102% |

Table 4. Performance of Test 4.4.7.a under Low Delay B Main 10

|  | Nmrg = 6 |  |  |  |  | Nmrg = 8 |  |  |  |  | Nmrg = 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.39% | -0.46% | -0.37% | 100% | 101% | -0.60% | -0.64% | -0.83% | 100% | 101% | -0.64% | -0.81% | -1.19% | 101% | 101% |
| Class C | -0.10% | -0.12% | -0.40% | 100% | 103% | -0.16% | -0.24% | -0.51% | 101% | 102% | -0.23% | -0.37% | -0.62% | 102% | 103% |
| Class E | 0.07% | -0.16% | -0.24% | 101% | 103% | -0.06% | 0.12% | -0.34% | 101% | 104% | -0.03% | -0.25% | -0.02% | 102% | 104% |
| Overall | -0.18% | -0.27% | -0.35% | 100% | 102% | -0.32% | -0.32% | -0.60% | 101% | 102% | -0.35% | -0.52% | -0.71% | 102% | 102% |

## Simulation results with same merge list size

This sub-section reports the coding performance of HMVP applied to both AMVP and merge mode with Nmrg equal to 6, 8 and 10, respectively.

Table 5. Performance of Test 4.4.7.c under Random Access Main 10

|  | Nmrg = 6 |  |  |  |  | Nmrg = 8 |  |  |  |  | Nmrg = 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.44% | -0.69% | 100% | 102% | -0.45% | -0.55% | -0.72% | 100% | 103% | -0.50% | -0.62% | -0.85% | 101% | 102% |
| Class A2 | -0.55% | -0.89% | -0.84% | 99% | 102% | -0.73% | -1.10% | -1.19% | 100% | 102% | -0.79% | -1.31% | -1.35% | 101% | 102% |
| Class B | -0.78% | -0.91% | -0.96% | 99% | 103% | -0.99% | -1.18% | -1.17% | 100% | 103% | -1.08% | -1.24% | -1.31% | 101% | 102% |
| Class C | -0.48% | -0.56% | -0.73% | 100% | 105% | -0.59% | -0.63% | -0.94% | 102% | 104% | -0.62% | -0.68% | -0.92% | 103% | 106% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.58% | -0.72% | -0.82% | 100% | 103% | -0.72% | -0.89% | -1.02% | 100% | 103% | -0.78% | -0.98% | -1.12% | 101% | 103% |

Table 6. Performance of Test 4.4.7.c under Low Delay B Main 10

|  | Nmrg = 6 |  |  |  |  | Nmrg = 8 |  |  |  |  | Nmrg = 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.41% | -0.66% | -0.59% | 99% | 96% | -0.66% | -0.83% | -0.89% | 100% | 101% | -0.72% | -0.90% | -1.09% | 101% | 102% |
| Class C | -0.18% | -0.32% | -0.31% | 100% | 102% | -0.22% | -0.38% | -0.48% | 102% | 103% | -0.29% | -0.51% | -0.53% | 102% | 104% |
| Class E | -0.01% | 0.45% | 0.55% | 101% | 97% | -0.12% | 0.20% | -0.18% | 102% | 105% | 0.01% | 0.07% | -0.04% | 102% | 102% |
| Overall | -0.23% | -0.27% | -0.21% | 100% | 98% | -0.38% | -0.42% | -0.57% | 101% | 103% | -0.39% | -0.53% | -0.64% | 101% | 102% |

# Conclusions

In this contribution, the HMVP method is proposed to improve the coding efficiency of MVP while there is no line buffer increased. In addition, the computational complexity is also kept very low since the total number of merge candidate list is kept unchanged and only up to 6 HMVP candidates may be checked in the merge candidate list construction process. It is recommended to consider adoption of the second test (i.e., HMVP with constrained FIFO, table size S set to 6 and merge candidate list size set to 8) to the next version of VTM.

# Reference

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, doc. JVET-J1010, April 2018.

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).