| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0280 |
| --- | --- |

| Title: | CE3-related: Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to signal 6 MPMs and only 32 remaining modes for each coding unit. To code an intra prediction mode, an MPM flag is firstly signaled. If the MPM flag is true, then truncated unary codeword with bypass coding is used to signal the MPM index. Otherwise, 5-bit fixed length coding is used to signal the remaining mode. Simulation results reportedly show that, with almost no impact on run-time, average -0.49% and -0.16% BD-rate change is achieved on top of VTM2.0.1 for all intra (AI) and random access (RA) configuration under common test condition.

Introduction

As shown in Figure 1, there are Planar, DC and 85 angular intra prediction modes used in VTM2.0.1. The number of the available intra prediction modes is always 67 for each block. To code an intra mode, a most probable mode (MPM) list of size 3 is constructed using the intra prediction modes of two neighboring blocks. Firstly, one MPM flag is signaled to indicate whether current mode is MPM or not. Then, if MPM flag is true, truncated unary code is used to signal the MPM index, otherwise, 6-bit fix length coding is used to signal the remaining modes.

Figure 1 Intra prediction modes in VTM2.0.1

Proposed method

In this contribution, it is proposed to signal 6 MPMs and only 32 remaining modes for each coding unit. The 6 MPM modes are derived from five neighboring blocks as shown in Figure 2. The 32 remaining modes are chosen based on the angular neighboring modes in MPM list.

Figure 2: 5 Neighbours (left -L, above-A, below-left-BL, above-right-AR, above-left-AL)

To code an intra prediction mode, an MPM flag is firstly signaled. If the MPM flag is true, then truncated unary codeword with bypass coding is used to signal the MPM index. Otherwise, 5-bit fix length coding is used to signal the mode index of the remaining modes.

Experimental results

The proposed method is implemented on top of BMS2.0.1 [1] software. The simulations are performed on Linux cluster using VTM configuration with AI and RA configurations, using the common test condition described in JVET-K1010 [2].

Table 1: Coding performance of the proposed method over VTM-2.0.1

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.27% | -0.29% | 99% | 95% | -0.15% | 0.01% | 0.00% | 95% | 97% |
| Class A2 | -0.42% | -0.36% | -0.39% | 99% | 97% | -0.14% | 0.10% | 0.02% | 95% | 97% |
| Class B | -0.47% | -0.34% | -0.39% | 96% | 94% | -0.17% | -0.04% | 0.03% | 95% | 95% |
| Class C | -0.57% | -0.51% | -0.31% | 98% | 99% | -0.18% | 0.07% | -0.10% | 95% | 92% |
| Class E | -0.56% | -0.38% | -0.42% | 98% | 95% |  |  |  |  |  |
| Overall | -0.49% | -0.38% | -0.36% | 98% | 96% | -0.16% | 0.03% | -0.01% | 95% | 95% |
| Class D | -0.41% | -0.10% | -0.32% | 97% | 91% | -0.10% | -0.25% | 0.22% | 96% | 94% |
| Class F | -0.76% | -0.72% | -0.75% | 100% | 100% | -0.49% | -0.55% | -0.45% | 96% | 95% |

Conclusion

In this contribution, it is proposed to only signal 6 MPM and 32 remaining modes for each coding unit. The proposed method does not increase the encoding and decoding run time and average -0.49% and -16% BD-rate change is reportedly shown on top of VTM2.0.1 under AI and RA configurations. It is proposed to further study this proposal in CE.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).