| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0283-v2 |
| --- | --- |

| Title: | CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross, Paul Keydel, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Liang Zhao, Xin Zhao, Xiang Li, Shan Liu Yao-Jen Chang, Hui-Yu Jiang Po-Han Lin, Chi-Chang Kuo Ching-Chieh Lin, Chun-Lung Lin | Email: | firstname.lastname@hhi.fraunhofer.de {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com {morriss.yj.chang, teresa.hy.jiang} @foxconn.com {pohan, chichang, JackLin, chunlung}@itri.com |
| Source: | Fraunhofer HHI, Tencent, Foxconn, ITRI International |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3-1.1 on multi-reference line intra prediction (MRL), including Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4 which are based on MRL using explicit signaling. In all CE3-1.1 tests, two additional reference lines are used, and reference line index is signaled before intra prediction mode. In addition, Test 1.1.1 applies MRL to all angular modes, Test 1.1.2 applies MRL to even angular modes, Test 1.1.3 and 1.1.4 apply MRL to MPMs. The results reportedly show that, for All Intra configuration, Test 1.1.1 achieved -0.61% luma BD-Rate with 125% enc-time, Test 1.1.2 achieved -0.47% luma BD-Rate with 130% enc-time, Test 1.1.3 achieved -0.46% luma BD-Rate with 102% enc-time, Test 1.1.4 achieved -0.40% luma BD-Rate with 102% enc-time. There is almost no decoder run-time impact for all four CE3-1.1 tests. In addition, results for combinations of MRL test 1.1.3 with CE3 tests 3.1.2 and 3.2.1 on 4-tap intra interpolation filters are provided that show that the BD-rate gains of MRL and the 4-tap filters are additive achieving -0.95% luma BD-Rate with 103% enc-time and -1.08% luma BD-Rate with 105% enc-time.

# Introduction

Multiple reference line (MRL) intra prediction was proposed to use more reference lines for intra prediction. The index of selected reference line (mrl_idx) is signaled and used to generate intra predictor. The reference line index is signaled before intra prediction modes, and Planar and DC modes are excluded from intra prediction modes in case a nonzero reference line index is signaled. In Figure 1, an example of 4 reference lines is depicted, where the samples of segments A and F are not fetched from reconstructed neighbouring samples but padded with the closest samples from Segment B and E, respectively.

Among all related contributions in CE3-1.1 [1], the following aspects have been identified to be further studied:

- Additional reference lines (current intra-picture prediction uses the nearest reference lines, i.e., 0):

  - 2 additional lines 0,1,3 (JVET-K0051)

- Intra prediction modes used for additional reference lines:

  - Angular modes (JVET-K0051, JVET-K0277)

  - Even angular modes (JVET-K0284)

- Reference line index signaling:

  - Mode after reference line index - for reference line idx > 0, only include additional reference line modes in MPM list (JVET-K0051)

  - Mode after reference line index (MPM only) - for reference line idx > 0, only include additional reference line modes in MPM list and only signal mpm index without remaining mode (JVET-K0482, JVET-K0175)

- Restrictions

  - Block sizes, i.e., disable MRL for narrow WxH blocks with abs(log2W-log2H) > 1 and blocks smaller than 8x8 (JVET-K0051)

  - Top line of CTU restriction, i.e., disable MRL for the first line of blocks inside a CTU to prevent using extended reference samples outside the current CTU line (JVET-K0221, JVET-K0166, JVET-K0162)

  - MDIS, i.e., disable MDIS for additional line (JVET-K0166)

  - MTS, i.e., disable MTS for additional line

  - PDPC, i.e., disable PDPC for additional line

Figure 1 Example of four reference lines neighboring to a prediction block

The following tests are designed in CE3-1.1 to study the impact of each individual aspect and interaction between them:

| Test | Intra Prediction Modes | Ref. Line Index Signalling | Block Size Restr. | Top CTU Restr. | MTS Restr. | MRL_EXCLUDE_ODD_MODES | MRL_USE_ONLY_MPM | MRL_DISABLE_BLOCK_SIZES_NONZERO_LINE | MRL_DISABLE_TOP_CTU_NONZERO_LINE | MRL_DISABLE_EMT_NONZERO_LINE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | Angular Modes | Mode after index | y |  | y | 0 | 0 | 1 | 0 | 1 |
| 1.1.2 | Even Angular Modes | Mode after index | y |  | y | 1 | 0 | 1 | 0 | 1 |
| 1.1.3 | Angular Modes | Mode after index (MPM only) |  |  |  | 0 | 1 | 0 | 0 | 0 |
| 1.1.4 | Angular Modes | Mode after index (MPM only) |  | y |  | 0 | 1 | 0 | 1 | 0 |

All tests have been integrated in a common software basis with test specific macros set as specified in the table above:

https://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS/tree/CE3-1.1

# Experimental results

The above four CE3-1.1 tests are implemented and tested on top of BMS-2.0.1 software. The simulations are performed following all intra (AI) and random access (RA) configuration on Linux cluster, and the test sequences are those defined in the common test condition described in [2]. Below tables report the detailed test results for each test.

Table 1: Summary results of CE3-1.1.1 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.10% | -0.17% | 130% | 99% | -0.33% | -0.18% | -0.30% | 109% | 100% |
| Class A2 | -0.46% | -0.23% | -0.29% | 126% | 99% | -0.34% | 0.02% | -0.12% | 107% | 100% |
| Class B | -0.59% | -0.30% | -0.35% | 125% | 99% | -0.32% | -0.27% | 0.00% | 108% | 100% |
| Class C | -0.85% | -0.54% | -0.57% | 121% | 98% | -0.33% | -0.17% | -0.17% | 108% | 100% |
| Class E | -0.70% | -0.44% | -0.37% | 126% | 99% |  |  |  |  |  |
| Overall | -0.61% | -0.33% | -0.36% | 125% | 99% | -0.33% | -0.17% | -0.13% | 108% | 100% |
| Class D | -0.30% | 0.14% | -0.17% | 122% | 99% | -0.12% | -0.32% | -0.02% | 107% | 100% |
| Class F | -0.82% | -0.66% | -0.44% | 122% | 99% | -0.44% | -0.13% | -0.43% | 107% | 99% |

Table 2: Summary results of CE3-1.1.2 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.06% | -0.28% | 134% | 98% | -0.35% | -0.22% | -0.22% | 108% | 100% |
| Class A2 | -0.38% | -0.14% | -0.18% | 130% | 99% | -0.25% | -0.06% | -0.16% | 107% | 100% |
| Class B | -0.45% | -0.22% | -0.17% | 130% | 98% | -0.24% | -0.13% | -0.01% | 109% | 101% |
| Class C | -0.58% | -0.37% | -0.26% | 127% | 101% | -0.22% | -0.08% | -0.03% | 109% | 99% |
| Class E | -0.53% | -0.28% | -0.31% | 129% | 100% |  |  |  |  |  |
| Overall | -0.47% | -0.22% | -0.23% | 130% | 99% | -0.26% | -0.12% | -0.09% | 108% | 100% |
| Class D | -0.29% | 0.00% | -0.29% | 128% | 101% | -0.14% | -0.39% | -0.03% | 109% | 101% |
| Class F | -0.59% | -0.49% | -0.45% | 126% | 99% | -0.33% | -0.18% | -0.19% | 109% | 101% |

Table 3: Summary results of CE3-1.1.3 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.00% | -0.06% | 102% | 98% | -0.11% | 0.03% | 0.10% | 100% | 100% |
| Class A2 | -0.28% | -0.16% | -0.17% | 101% | 97% | -0.21% | 0.09% | -0.11% | 100% | 100% |
| Class B | -0.48% | -0.25% | -0.30% | 101% | 97% | -0.23% | -0.10% | -0.04% | 100% | 100% |
| Class C | -0.82% | -0.60% | -0.45% | 102% | 99% | -0.25% | 0.02% | 0.00% | 100% | 99% |
| Class E | -0.48% | -0.34% | -0.13% | 103% | 98% |  |  |  |  |  |
| Overall | -0.46% | -0.29% | -0.24% | 102% | 98% | -0.20% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.22% | -0.07% | -0.23% | 103% | 99% | -0.02% | -0.09% | 0.32% | 100% | 99% |
| Class F | -1.45% | -1.11% | -1.14% | 103% | 101% | -0.96% | -0.67% | -0.68% | 100% | 100% |

Table 4: Summary results of CE3-1.1.4 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.02% | 0.01% | 103% | 97% | -0.11% | 0.08% | 0.12% | 100% | 100% |
| Class A2 | -0.22% | -0.11% | -0.14% | 102% | 98% | -0.19% | 0.03% | -0.06% | 101% | 100% |
| Class B | -0.43% | -0.23% | -0.25% | 101% | 96% | -0.18% | -0.09% | 0.07% | 100% | 100% |
| Class C | -0.74% | -0.49% | -0.29% | 104% | 102% | -0.24% | -0.07% | -0.06% | 100% | 98% |
| Class E | -0.37% | -0.21% | -0.27% | 102% | 96% |  |  |  |  |  |
| Overall | -0.40% | -0.23% | -0.20% | 102% | 98% | -0.18% | -0.03% | 0.02% | 100% | 99% |
| Class D | -0.20% | 0.07% | -0.25% | 102% | 93% | -0.04% | -0.24% | 0.24% | 100% | 99% |
| Class F | -1.29% | -1.03% | -0.91% | 103% | 100% | -0.86% | -0.60% | -0.50% | 100% | 100% |

# Combination with 4-tap intra interpolation filters

In addition, when 4-tap intra interpolation filter (as being studied in CE3-3) is enabled, it is noticed that MRL provides additive or even extra gain by applying 4-tap Cubic interpolation filter (IF) on nonzero reference lines (i.e., mrl_idx > 0). Several combination tests are described in Table 5, and for each combination test, the 4-tap Cubic IF is applied to nonzero reference lines.

Table 5: Descriptions of the combination tests.

| Test | Method description |
| --- | --- |
| C1 | VTM-2.0.1 + CE3-1.1.3 + BMS-2.0.1 4-Tap IF (--Intra4Tap=1) |
| C2 | VTM-2.0.1 + CE3-1.1.3 + CE3-3.1.2 |
| C3 | VTM-2.0.1 + CE3-1.1.3 + CE3-3.2.1 |

Table 6: Results overview for combined tests

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE3-1.1.3 | -0.46% | -0.29% | -0.24% | 102% | 98% | -0.20% | -0.01% | -0.01% | 100% | 100% |
| BMS 4-tap | -0.36% | -0.39% | -0.41% | 101% | 99% | -0.16% | -0.31% | -0.30% | 100% | 99% |
| C1 | -0.90% | -0.76% | -0.78% | 104% | 98% | -0.40% | -0.31% | -0.32% | 100% | 98% |
| CE3-3.1.2 | -0.46% | -0.58% | -0.61% | 103% | 102% | -0.19% | -0.14% | -0.13% | 103% | 103% |
| C2 | -0.95% | -0.88% | -0.87% | 103% | 98% | -0.41% | -0.18% | -0.15% | 100% | 98% |
| CE3-3.2.1 | -0.59% | -0.68% | -0.69% | 102% | 101% | -0.26% | -0.43% | -0.45% | 100% | 100% |
| C3 | -1.08% | -0.99% | -0.97% | 105% | 102% | -0.49% | -0.46% | -0.45% | 101% | 100% |

Detailed experimental results for each combination test are reported in below tables.

Table 7: Summary results of C1 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.19% | -0.14% | 105% | 96% | -0.22% | -0.12% | -0.17% | 99% | 97% |
| Class A2 | -0.48% | -0.29% | -0.33% | 105% | 98% | -0.30% | -0.10% | -0.24% | 98% | 96% |
| Class B | -0.93% | -0.87% | -0.84% | 103% | 97% | -0.43% | -0.48% | -0.30% | 101% | 99% |
| Class C | -1.52% | -1.47% | -1.46% | 104% | 100% | -0.57% | -0.41% | -0.51% | 101% | 99% |
| Class E | -1.09% | -0.64% | -0.85% | 105% | 97% |  |  |  |  |  |
| Overall | -0.90% | -0.76% | -0.78% | 104% | 98% | -0.40% | -0.31% | -0.32% | 100% | 98% |
| Class D | -0.74% | -0.54% | -0.74% | 103% | 91% | -0.32% | -0.51% | -0.30% | 101% | 102% |
| Class F | -1.81% | -1.68% | -1.64% | 104% | 100% | -1.02% | -0.66% | -0.76% | 101% | 101% |

Table 8: Summary results of C2 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.10% | -0.21% | 104% | 97% | -0.16% | -0.08% | -0.03% | 99% | 97% |
| Class A2 | -0.46% | -0.31% | -0.34% | 104% | 99% | -0.34% | 0.02% | -0.08% | 98% | 96% |
| Class B | -0.94% | -0.96% | -0.90% | 102% | 98% | -0.44% | -0.29% | -0.21% | 101% | 101% |
| Class C | -1.62% | -1.55% | -1.46% | 101% | 97% | -0.59% | -0.25% | -0.22% | 101% | 99% |
| Class E | -1.35% | -1.20% | -1.23% | 102% | 97% |  |  |  |  |  |
| Overall | -0.95% | -0.88% | -0.87% | 103% | 98% | -0.41% | -0.18% | -0.15% | 100% | 98% |
| Class D | -0.75% | -0.60% | -0.95% | 102% | 92% | -0.34% | -0.36% | 0.01% | 101% | 102% |
| Class F | -1.82% | -1.66% | -1.68% | 102% | 97% | -0.96% | -0.67% | -0.72% | 100% | 101% |

Table 9: Summary results of C3 over VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.46% | -0.46% | 106% | 102% | -0.38% | -0.23% | -0.19% | 100% | 102% |
| Class A2 | -0.61% | -0.45% | -0.51% | 105% | 102% | -0.44% | -0.42% | -0.49% | 100% | 99% |
| Class B | -1.00% | -0.94% | -0.97% | 106% | 100% | -0.47% | -0.60% | -0.45% | 102% | 100% |
| Class C | -1.71% | -1.70% | -1.57% | 103% | 103% | -0.63% | -0.49% | -0.63% | 101% | 99% |
| Class E | -1.31% | -1.21% | -1.12% | 105% | 102% |  |  |  |  |  |
| Overall | -1.08% | -0.99% | -0.97% | 105% | 102% | -0.49% | -0.46% | -0.45% | 101% | 100% |
| Class D | -0.77% | -0.69% | -0.95% | 103% | 101% | -0.35% | -0.44% | -0.16% | 101% | 100% |
| Class F | -1.94% | -1.93% | -1.77% | 103% | 101% | -1.06% | -0.92% | -0.98% | 101% | 99% |

# Conclusions

Among all the tests defined in CE3-1.1 based on explicit signaling, it can be concluded that, applying MRL to MPM provides the best trade-off between coding efficiency and encoder runtime. For all intra in VTM configuration, Test 1.1.3 achieves -0.46% BD-rate Y with 102% encoder runtime. Furthermore, in Test 1.1.4, by adding further constraint on MRL, the HW concerns can be addressed without noticeably sacrificing BD-rate. It is also reported that the MRL provides additive or extra coding gain on top of 4-Tap interpolation filters. Hence it is proposed to adopt this MRL technique as tested in Test 1.1.4.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-K1023, Ljubljana, SI, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).