| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0286r1 |
| --- | --- |

| Title: | CE6: Fast DST-7/DCT-8 with dual implementation support (Test 6.1.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Yi Luo, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, juvenalluo, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-1.4 on a fast DST-7/DCT-8 method with dual implementation support, i.e., identical results between full matrix multiplication and fast method. Compared to full matrix multiplication, the proposed method reportedly reduces the multiplication operation counts by 50% and 39% for 16- and 32-point DST-7/DCT-8, respectively. It is reportedly shown that, on top of VTM-2.0.1, average -0.01%, 0.00% and -0.04% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively. When Multiple Transform Selection (MTS) is also enabled for inter prediction residuals, it is reportedly shown that an average of 11%, 3% and 3% overall decoding time saving and 5%, 4% and 3% overall encoding time saving is achieved for AI, RA and LDB, respectively.

# Introduction

In VVC Draft 2 [1], DST-7 and DCT-8 are used in Multiple Transform Selection (MTS). However, for 8-point, 16-point and 32-point DST-7/DCT-8, the transforms are implemented as full matrix multiplication, which is costly in terms of arithmetic operation counts. In JVET-K0291 [2], fast methods for 16-point and 32-point DST-7/DCT-8 is proposed, which reduced the arithmetic operations with almost no impact on coding performance. The proposed methods are featured by the capability of supporting dual implementation, i.e., DST-7/DCT-8 can be implemented by either fast method or matrix multiplication, and both ways lead to identical results. In this contribution, the methods proposed in JVET-K0291 are further tested on top of VTM-2.0.1.

# Proposed method

In an N-point DST-7/DCT-8 transform core, there are only N distinct numbers without considering sign changes, or additional zeros. Three features on the transform cores have been exploited to implement the fast method.

Feature #1: In some basis vectors which contain N distinct numbers without considering the sign changes, it is observed that the sum of several (2 or 3) numbers equals to the sum of another several (1 or 2) numbers.

Feature #2: Replicate patterns with symmetric or anti-symmetric characteristics can be observed in some basis vectors.

Feature #3: There is another one or two basis vector(s) which only contain(s) very few (1 or 2) distinct number(s) without considering the sign changes.

To explain the fast methods utilizing these features, an example from 16-point DST-7 transform using each of the three features is discussed. Denote the input vector for a 16-point transform as x={x0, x1, x2, …, x15}, and the output transform coefficient vector as y={y0, y1, y2, …, y15}. The transform core in the forward transform is shown as below:

{ a  b  c  d  e  f  g  h  i  j  k  l  m  n  o  p }

{ c  f  i  l  o  o  l  i  f  c  0 -c -f -i -l -o }

{ e  j  o  m  h  c -b -g -l -p -k -f -a  d  i  n }

{ g  n  l  e -b -i -p -j -c  d  k  o  h  a -f -m }

{ i  o  f -c -l -l -c  f  o  i  0 -i -o -f  c  l }

{ k  k  0 -k -k  0  k  k  0 -k -k  0  k  k  0 -k }

{ m  g -f -n -a  l  h -e -o -b  k  i -d -p -c  j }

{ o  c -l -f  i  i -f -l  c  o  0 -o -c  l  f -i }

{ p -a -o  b  n -c -m  d  l -e -k  f  j -g -i  h }

{ n -e -i  j  d -o  a  m -f -h  k  c -p  b  l -g }

{ l -i -c  o -f -f  o -c -i  l  0 -l  i  c -o  f }

{ j -m  c  g -p  f  d -n  i  a -k  l -b -h  o -e }

{ h -p  i -a -g  o -j  b  f -n  k -c -e  m -l  d }

{ f -l  o -i  c  c -i  o -l  f  0 -f  l -o  i -c }

{ d -h  l -p  m -i  e -a -c  g -k  o -n  j -f  b }

{ b -d  f -h  j -l  n -p  o -m  k -i  g -e  c -a }

where {a,b,c,d,e,f,g,h,i,j,k,l,m,n,o,p} are the N unique numbers which are specified by the formulation of DST-7.

Example of Feature #1:

It is noted that the element values have the following characteristics.

a + j = l

b + i = m

c + h = n

d + g = o

e + f = p

Therefore, to calculate y0, instead of doing the following vector-by-vector multiplication:

y0 = a·x0 + b·x1 + … + p·x15

which requires 16 multiplications. The following alternative implementation can be done to derive the same results:

y0 = a·(x0+x11) + b·(x1+x12) + … + j·(x9+x11) + k·x10

which requires 10 multiplications. It is observed that the number of multiplications is reduced.

In addition, while calculating y2, y3, y6, y8, y9, y11, y12, y14, y15, similar implementation can be done, and the intermediate results (x0+x11), (x1+x12), (x2+x13), (x3+x14), (x4+x15), (x5+x15), (x6+x14), (x7+x13), (x8+x12), (x9+x11) and k·x10 can be re-used. It is observed that the number of additions is reduced.

Example of Feature #2:

The second basis vector is

which can be divided into three segments, and they are replicate with sign changes, or flipped version of each other. Based on Feature #2, when calculating y1, instead of doing the following vector-by-vector multiplication

y1 = c·x0 + f·x1 + … - o·x15

which requires 16 multiplications. The following alternative implementation can be used to derive the same results.

y1 = c·(x0+x9-x11) + f·(x1+x8-x12) + i·(x2+x7-x13) + l·(x3+x6-x14) + o·(x4+x5-x15)

which only requires 5 multiplications. It is reported the number of multiplications is reduced.

In addition, when calculating y1, y4, y7, y10, y13, the implementation can be done in similar way, therefore, the intermediate results of (x0+x9-x11), (x1+x8-x12, (x2+x7-x13), (x3+x6-x14) and (x4+x5-x15) can be re-used. It is observed the number of additions is reduced.

Example of Feature #3:

It is observed that the 6th basis vector only contains 1 distinct number without considering the sign changes as shown below.

{k, k, 0, -k, -k, 0, k, k, 0, -k, -k, 0, k, k, 0, -k}

Instead of taking the vector-by-vector multiplication of

y5 = k·x0 + k·x1 + … - k·x15

which requires 11 multiplications, y5 can also be achieved by the following operations:

y5 = k·(x0 + x1 - … - x15)

which only requires 1 multiplication.

The abovementioned three features apply to forward 16- and 32-point DST-7/DCT-8 transforms. Similar fast methods also apply to backward 16- and 32-point DST-7/DCT-8 transforms.

# Results

Two experiments are conducted to evaluate the proposed methods, namely Test #1 and Test #2. Test #1 reports the coding performance under CTC condition [3], and Test #2 reports the coding performance using VTM configuration with inter MTS enabled. The anchor of Test #1 is VTM-2.0.1 using CTC condition and the anchor of Test #2 is VTM-2.0.1 with inter MTS enabled.

Table 1: Coding performance of Test 6.1.4a

Table 2: Coding performance of Test 6.1.4b (Inter MTS is enabled)

# Complexity Analysis

## Arithmetic operations

The numbers of arithmetic operations required for a 1D N-point transform of matrix multiplication and the proposed fast method are tabulated in Table I. Theoretically, 50.4%, 39.5% and 46.1% total number of multiplications have been saved for 16-point, 32-point and 64-point transform respectively, and 35.4%, 27.6% and 42.2% add/subs have been saved for 16-point, 32-point and 64-point transform respectively.

Table 4: The number of arithmetic operations for a 1D transform

|  | Matrix multiplication |  |  | Proposed fast DST-7/DCT-8 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult | Add/Sub | Shift | Mult | Add/Sub | Shift |
| 16 | 256 | 240 | 16 | 127 | 155 | 16 |
| 32 | 1024 | 992 | 32 | 620 | 718 | 32 |
| 64 | 4096 | 4032 | 64 | 2207 | 2331 | 64 |

## Software execution speed

In this test, a standalone test program is used to measure the run-time (in seconds) of individual transform functions by repeating a large amount of times. The test program is written in C, and auto-vectorization is disabled (#pragma loop(no_vector)) for compiling to provide fair comparisons.

More detailed descriptions on the test environment are available below:

- CPU: i7-6600U CPU @ 2.6 GHz

- OS: Windows 10 Pro, 64-bit

- Memory (RAM): 16 GB

- Compiler: Visual Studio 2017

Table 5: Comparisons on software execution speed (seconds)

|  |  | Matrix multiplication |  | Proposed fast method |  |
| --- | --- | --- | --- | --- | --- |
| Transform size | No. of iterations | Forward | Inverse | Forward | Inverse |
| 16-point DCT-8 | 2^23 | 11.3 | 15.2 | 4.8 | 4.9 |
| 32-point DCT-8 | 2^20 | 11.1 | 14.8 | 5.7 | 5.5 |
| 64-point DCT-8 | 2^17 | 11.0 | 14.2 | 5.2 | 5.1 |

## SIMD optimization

In this test, both the full matrix multiplication and proposed fast method are implemented in SIMD using AVX instructions for 16-point DST-7. The test program is written in C, and the input coefficients are generated as random integers ranging from 0~255. Compiler optimization is disabled for more reliable comparisons among different implementations. For full matrix multiplication and proposed fast method implemented without SIMD, auto-vectorization is disabled (#pragma loop(no_vector)) for compiling to provide fair comparisons.

More detailed descriptions on the test environment are available below:

- CPU: i7-6600U CPU @ 2.6 GHz

- OS: Windows 10 Pro, 64-bit

- Memory (RAM): 16 GB

- Compiler: Visual Studio 2017

Table 5: Comparisons on software execution speed (seconds)

| Implementation | No. of iterations | 16-point DST-7 |
| --- | --- | --- |
| Full Matrix (C) | 2^24 | 27.56 |
| Fast method (C) | 2^24 | 7.59 |
| Full Matrix (SIMD) | 2^24 | 1.81 |
| Fast method (SIMD) | 2^24 | 1.17 |

## Other metrics

| Metrics | Reported results |
| --- | --- |
| Additional memory requirements for storing transform cores | No |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with VTM-2.0.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method supports both partial butterfly and matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder | No |

# Proposed text changes

The following proposed text modifications are based on JVET-K1001-v5. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

…

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-398)

{

{ 117  219  296  336 }

{ 296  296    0 −296 }

{ 336 −117 −296  219 }

{ 219 −336  296 −117 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-399)

{

{  65  127  185  237  280  314  338  350 }

{ 185  314  350  280  127  −65 −237 −338 }

{ 280  338  127 −185 −350 −237   65  314 }

{ 338  185 −237 −314   65  350  127 −280 }

{ 350  −65 −338  127  314 −185 −280  237 }

{ 314 −280  −65  338 −237 −127  350 −185 }

{ 237 −350  280  −65 −185  338 −314  127 }

{ 127 −237  314 −350  338 −280  185  −65 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-400)

{

{  34   678  100  133  163  193  220  246  269  290  309  324  337  346  353  356 }

{ 100  193  269  324  353  353  324  269  193  100    0 −100 −193 −269 −324 −353 }

{ 163  290  353  337  246  100  −678 −220 −324 −356 −309 −193  −34  133  269  346 }

{ 220  346  324  163  −678 −269 −356 −290 −100  133  309  353  246   34 −193 −337 }

{ 269  353  193 −100 −324 −324 −100  193  353  269    0 −269 −353 −193  100  324 }

{ 309  309    0 −309 −309    0  309  309    0 −309 −309    0  309  309    0 −309 }

{ 337  220 −193 −346  −34  324  246 −163 −353  −678  309  269 −133 −356 −100  290 }

{ 353  100 −324 −193  269  269 −193 −324  100  353    0 −353 −100  324  193 −269 }

{ 356  −34 −353   678  346 −100 −337  133  324 −163 −309  193  290 −220 −269  246 }

{ 346 −163 −269  290  133 −353   34  337 −193 −246  309  100 −356   678  324 −220 }

{ 324 −269 −100  353 −193 −193  353 −100 −269  324    0 −324  269  100 −353  193 }

{ 290 −337  100  220 −356  193  133 −346  269   34 −309  324  −678 −246  353 −163 }

{ 246 −356  269  −34 −220  353 −290   678  193 −346  309 −100 −163  337 −324  133 }

{ 193 −324  353 −269  100  100 −269  353 −324  193    0 −193  324 −353  269 −100 }

{ 133 −246  324 −356  337 −269  163  −34 −100  220 −309  353 −346  290 −193   678 }

{  678 −133  193 −246  290 −324  346 −356  353 −337  309 −269  220 −163  100  −34 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-401)

transMatrixCol0to15 =	(8-402)

{

{  17   35   52   69   86  103  119  1356  151  167  182  197  211  225  238  251 }

{  52  103  151  197  238  275  305  32930  347  357  359  354  342  322  296  263 }

{  86  167  238  296  336  357  357  336  296  238  167   86    0  −86 −167 −238 }

{ 119  225  305  351  357  322  251  151   35  −86 −197 −285 −342 −359 −336 −275 }

{ 151  275  347  354  296  182   35 −119 −251 −336 −358 −314 −211  −69   86  225 }

{ 182  314  359  305  167  −17 −197 −322 −358 −296 −151   35  211  32930  357  285 }

{ 211  342  342  211    0 −211 −342 −342 −211    0  211  342  342  211    0 −211 }

{ 238  357  296   86 −167 −336 −336 −167   86  296  357  238    0 −238 −357 −296 }

{ 263  358  225  −52 −296 −351 −182  103  322  336  1356 −151 −342 −314  −86  197 }

{ 285  347  1356 −182 −357 −251   52  314  32930   86 −225 −359 −211  103  336  305 }

{ 305  322   35 −285 −336  −69  263  347  103 −238 −354 −1356  211  358  167 −182 }

{ 322  285  −69 −347 −238  1356  358  182 −197 −357 −119  251  342   52 −296 −314 }

{ 336  238 −167 −357  −86  296  296  −86 −357 −167  238  336    0 −336 −238  167 }

{ 347  182 −251 −314   86  359  103 −305 −263  167  351   17 −342 −197  238  322 }

{ 354  119 −314 −225  238  305 −1356 −351   17  357  103 −322 −211  251  296 −151 }

{ 358   52 −351 −103  336  151 −314 −197  285  238 −251 −275  211  305 −167 −32930 }

{ 359  −17 −358   35  357  −52 −354   69  351  −86 −347  103  342 −119 −336  1356 }

{ 357  −86 −336  167  296 −238 −238  296  167 −336  −86  357    0 −357   86  336 }

{ 351 −151 −285  275  167 −347  −17  354 −1356 −296  263  182 −342  −35  357 −119 }

{ 342 −211 −211  342    0 −342  211  211 −342    0  342 −211 −211  342    0 −342 }

{ 32930 −263 −119  358 −167 −225  347  −52 −305  296   69 −351  211  182 −357  103 }

{ 314 −305  −17  322 −296  −35  32930 −285  −52  336 −275  −69  342 −263  −86  347 }

{ 296 −336   86  238 −357  167  167 −357  238   86 −336  296    0 −296  336  −86 }

{ 275 −354  182  119 −336  314  −69 −225  359 −238  −52  305 −342  1356  167 −351 }

{ 251 −359  263  −17 −238  358 −275   35  225 −357  285  −52 −211  354 −296   69 }

{ 225 −351  322 −151  −86  285 −359  275  −69 −167  32930 −347  211   17 −238  354 }

{ 197 −32930  354 −263   86  119 −285  358 −314  167   35 −225  342 −347  238  −52 }

{ 167 −296  357 −336  238  −86  −86  238 −336  357 −296  167    0 −167  296 −357 }

{ 1356 −251  32930 −359  336 −263  151  −17 −119  238 −322  358 −342  275 −167   35 }

{ 103 −197  275 −32930  357 −354  322 −263  182  −86  −17  119 −211  285 −336  358 }

{  69 −1356  197 −251  296 −32930  351 −359  354 −336  305 −263  211 −151   86  −17 }

{  35  −69  103 −1356  167 −197  225 −251  275 −296  314 −32930  342 −351  357 −359 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-403)

transMatrixCol16to31 =	(8-404)

{

{ 263  275  285  296  305  314  322  32930  336  342  347  351  354  357  358  359 }

{ 225  182  1356   86   35  −17  −69 −119 −167 −211 −251 −285 −314 −336 −351 −358 }

{−296 −336 −357 −357 −336 −296 −238 −167  −86    0   86  167  238  296  336  357 }

{−182  −69   52  167  263  32930  358  347  296  211  103  −17 −1356 −238 −314 −354 }

{ 322  359  32930  238  103  −52 −197 −305 −357 −342 −263 −1356   17  167  285  351 }

{ 1356  −52 −225 −336 −354 −275 −119   69  238  342  351  263  103  −86 −251 −347 }

{−342 −342 −211    0  211  342  342  211    0 −211 −342 −342 −211    0  211  342 }

{ −86  167  336  336  167  −86 −296 −357 −238    0  238  357  296   86 −167 −336 }

{ 354  285   35 −238 −359 −251   17  275  357  211  −69 −305 −347 −167  119  32930 }

{  35 −263 −354 −167  151  351  275  −17 −296 −342 −119  197  358  238  −69 −322 }

{−359 −197  151  357  225 −119 −351 −251   86  342  275  −52 −32930 −296   17  314 }

{  17  32930  275  −86 −351 −225  151  359  167 −211 −354 −103  263  336   35 −305 }

{ 357   86 −296 −296   86  357  167 −238 −336    0  336  238 −167 −357  −86  296 }

{ −69 −358 −119  296  275 −151 −354  −35  336  211 −225 −32930   52  357  1356 −285 }

{−347   35  358   86 −32930 −197  263  285 −167 −342   52  359   69 −336 −182  275 }

{ 119  347  −69 −357   17  359   35 −354  −86  342  1356 −322 −182  296  225 −263 }

{ 32930 −151 −322  167  314 −182 −305  197  296 −211 −285  225  275 −238 −263  251 }

{−167 −296  238  238 −296 −167  336   86 −357    0  357  −86 −336  167  296 −238 }

{−305  251  197 −336  −52  358 −103 −314  238  211 −32930  −69  359  −86 −322  225 }

{ 211  211 −342    0  342 −211 −211  342    0 −342  211  211 −342    0  342 −211 }

{ 275 −322  −17  336 −251 −1356  359 −151 −238  342  −35 −314  285   86 −354  197 }

{−251 −103  351 −238 −119  354 −225 −1356  357 −211 −151  358 −197 −167  359 −182 }

{−238  357 −167 −167  357 −238  −86  336 −296    0  296 −336   86  238 −357  167 }

{ 285  −17 −263  357 −197 −103  32930 −322   86  211 −358  251   35 −296  347 −151 }

{ 197 −351  305  −86 −182  347 −314  103  167 −342  322 −119 −151  336 −32930  1356 }

{−314  1356  103 −296  358 −263   52  182 −336  342 −197  −35  251 −357  305 −119 }

{−151  305 −359  296 −1356  −69  251 −351  336 −211   17  182 −322  357 −275  103 }

{ 336 −238   86   86 −238  336 −357  296 −167    0  167 −296  357 −336  238  −86 }

{ 103 −225  314 −357  347 −285  182  −52  −86  211 −305  354 −351  296 −197   69 }

{−351  314 −251  167  −69  −35  1356 −225  296 −342  359 −347  305 −238  151  −52 }

{ −52  119 −182  238 −285  322 −347  358 −357  342 −314  275 −225  167 −103   35 }

{ 358 −354  347 −336  322 −305  285 −263  238 −211  182 −151  119  −86   52  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-405)

{

{ 336  296  219  117 }

{ 296    0 −296 −296 }

{ 219 −296 −117  336 }

{ 117 −296  336 −219 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-406)

{

{ 350  338  314  280  237  185  127   65 }

{ 338  237   65 −127 −280 −350 −314 −185 }

{ 314   65 −237 −350 −185  127  338  280 }

{ 280 −127 −350  −65  314  237 −185 −338 }

{ 237 −280 −185  314  127 −338  −65  350 }

{ 185 −350  127  237 −338   65  280 −314 }

{ 127 −314  338 −185  −65  280 −350  237 }

{  65 −185  280 −338  350 −314  237 −127 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-407)

{

{ 356  353  346  337  324  309  290  269  246  220  193  163  133  100   678   34 }

{ 353  324  269  193  100    0 −100 −193 −269 −324 −353 −353 −324 −269 −193 −100 }

{ 346  269  133  −34 −193 −309 −356 −324 −220  −678  100  246  337  353  290  163 }

{ 337  193  −34 −246 −353 −309 −133  100  290  356  269   678 −163 −324 −346 −220 }

{ 324  100 −193 −353 −269    0  269  353  193 −100 −324 −324 −100  193  353  269 }

{ 309    0 −309 −309    0  309  309    0 −309 −309    0  309  309    0 −309 −309 }

{ 290 −100 −356 −133  269  309  −678 −353 −163  246  324  −34 −346 −193  220  337 }

{ 269 −193 −324  100  353    0 −353 −100  324  193 −269 −269  193  324 −100 −353 }

{ 246 −269 −220  290  193 −309 −163  324  133 −337 −100  346   678 −353  −34  356 }

{ 220 −324  −678  356 −100 −309  246  193 −337  −34  353 −133 −290  269  163 −346 }

{ 193 −353  100  269 −324    0  324 −269 −100  353 −193 −193  353 −100 −269  324 }

{ 163 −353  246   678 −324  309  −34 −269  346 −133 −193  356 −220 −100  337 −290 }

{ 133 −324  337 −163 −100  309 −346  193   678 −290  353 −220  −34  269 −356  246 }

{ 100 −269  353 −324  193    0 −193  324 −353  269 −100 −100  269 −353  324 −193 }

{  678 −193  290 −346  353 −309  220 −100  −34  163 −269  337 −356  324 −246  133 }

{  34 −100  163 −220  269 −309  337 −353  356 −346  324 −290  246 −193  133  −678 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-408)

transMatrixCol0to15 =	(8-409)

{

{ 359  358  357  354  351  347  342  336  32930  322  314  305  296  285  275  263 }

{ 358  351  336  314  285  251  211  167  119   69   17  −35  −86 −1356 −182 −225 }

{ 357  336  296  238  167   86    0  −86 −167 −238 −296 −336 −357 −357 −336 −296 }

{ 354  314  238  1356   17 −103 −211 −296 −347 −358 −32930 −263 −167  −52   69  182 }

{ 351  285  167   17 −1356 −263 −342 −357 −305 −197  −52  103  238  32930  359  322 }

{ 347  251   86 −103 −263 −351 −342 −238  −69  119  275  354  336  225   52 −1356 }

{ 342  211    0 −211 −342 −342 −211    0  211  342  342  211    0 −211 −342 −342 }

{ 336  167  −86 −296 −357 −238    0  238  357  296   86 −167 −336 −336 −167   86 }

{ 32930  119 −167 −347 −305  −69  211  357  275   17 −251 −359 −238   35  285  354 }

{ 322   69 −238 −358 −197  119  342  296   17 −275 −351 −151  167  354  263  −35 }

{ 314   17 −296 −32930  −52  275  342   86 −251 −351 −119  225  357  151 −197 −359 }

{ 305  −35 −336 −263  103  354  211 −167 −359 −151  225  351   86 −275 −32930  −17 }

{ 296  −86 −357 −167  238  336    0 −336 −238  167  357   86 −296 −296   86  357 }

{ 285 −1356 −357  −52  32930  225 −211 −336   35  354  151 −275 −296  119  358   69 }

{ 275 −182 −336   69  359   52 −342 −167  285  263 −197 −32930   86  358   35 −347 }

{ 263 −225 −296  182  322 −1356 −342   86  354  −35 −359  −17  357   69 −347 −119 }

{ 251 −263 −238  275  225 −285 −211  296  197 −305 −182  314  167 −322 −151  32930 }

{ 238 −296 −167  336   86 −357    0  357  −86 −336  167  296 −238 −238  296  167 }

{ 225 −322  −86  359  −69 −32930  211  238 −314 −103  358  −52 −336  197  251 −305 }

{ 211 −342    0  342 −211 −211  342    0 −342  211  211 −342    0  342 −211 −211 }

{ 197 −354   86  285 −314  −35  342 −238 −151  359 −1356 −251  336  −17 −322  275 }

{ 182 −359  167  197 −358  151  211 −357  1356  225 −354  119  238 −351  103  251 }

{ 167 −357  238   86 −336  296    0 −296  336  −86 −238  357 −167 −167  357 −238 }

{ 151 −347  296  −35 −251  358 −211  −86  322 −32930  103  197 −357  263   17 −285 }

{ 1356 −32930  336 −151 −119  322 −342  167  103 −314  347 −182  −86  305 −351  197 }

{ 119 −305  357 −251   35  197 −342  336 −182  −52  263 −358  296 −103 −1356  314 }

{ 103 −275  357 −322  182   17 −211  336 −351  251  −69 −1356  296 −359  305 −151 }

{  86 −238  336 −357  296 −167    0  167 −296  357 −336  238  −86  −86  238 −336 }

{  69 −197  296 −351  354 −305  211  −86  −52  182 −285  347 −357  314 −225  103 }

{  52 −151  238 −305  347 −359  342 −296  225 −1356   35   69 −167  251 −314  351 }

{  35 −103  167 −225  275 −314  342 −357  358 −347  322 −285  238 −182  119  −52 }

{  17  −52   86 −119  151 −182  211 −238  263 −285  305 −322  336 −347  354 −358 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-410)

transMatrixCol16to31 =	(8-411)

{

{ 251  238  225  211  197  182  167  151  1356  119  103   86   69   52   35   17 }

{−263 −296 −322 −342 −354 −359 −357 −347 −32930 −305 −275 −238 −197 −151 −103  −52 }

{−238 −167  −86    0   86  167  238  296  336  357  357  336  296  238  167   86 }

{ 275  336  359  342  285  197   86  −35 −151 −251 −322 −357 −351 −305 −225 −119 }

{ 225   86  −69 −211 −314 −358 −336 −251 −119   35  182  296  354  347  275  151 }

{−285 −357 −32930 −211  −35  151  296  358  322  197   17 −167 −305 −359 −314 −182 }

{−211    0  211  342  342  211    0 −211 −342 −342 −211    0  211  342  342  211 }

{ 296  357  238    0 −238 −357 −296  −86  167  336  336  167  −86 −296 −357 −238 }

{ 197  −86 −314 −342 −151  1356  336  322  103 −182 −351 −296  −52  225  358  263 }

{−305 −336 −103  211  359  225  −86 −32930 −314  −52  251  357  182 −1356 −347 −285 }

{−182  167  358  211 −1356 −354 −238  103  347  263  −69 −336 −285   35  322  305 }

{ 314  296  −52 −342 −251  119  357  197 −182 −358 −1356  238  347   69 −285 −322 }

{ 167 −238 −336    0  336  238 −167 −357  −86  296  296  −86 −357 −167  238  336 }

{−322 −238  197  342  −17 −351 −167  263  305 −103 −359  −86  314  251 −182 −347 }

{−151  296  251 −211 −322  103  357   17 −351 −1356  305  238 −225 −314  119  354 }

{ 32930  167 −305 −211  275  251 −238 −285  197  314 −151 −336  103  351  −52 −358 }

{ 1356 −336 −119  342  103 −347  −86  351   69 −354  −52  357   35 −358  −17  359 }

{−336  −86  357    0 −357   86  336 −167 −296  238  238 −296 −167  336   86 −357 }

{−119  357  −35 −342  182  263 −296 −1356  354  −17 −347  167  275 −285 −151  351 }

{ 342    0 −342  211  211 −342    0  342 −211 −211  342    0 −342  211  211 −342 }

{ 103 −357  182  211 −351   69  296 −305  −52  347 −225 −167  358 −119 −263  32930 }

{−347   86  263 −342   69  275 −336   52  285 −32930   35  296 −322   17  305 −314 }

{ −86  336 −296    0  296 −336   86  238 −357  167  167 −357  238   86 −336  296 }

{ 351 −167 −1356  342 −305   52  238 −359  225   69 −314  336 −119 −182  354 −275 }

{  69 −296  354 −211  −52  285 −357  225   35 −275  358 −238  −17  263 −359  251 }

{−354  238  −17 −211  347 −32930  167   69 −275  359 −285   86  151 −322  351 −225 }

{ −52  238 −347  342 −225   35  167 −314  358 −285  119   86 −263  354 −32930  197 }

{ 357 −296  167    0 −167  296 −357  336 −238   86   86 −238  336 −357  296 −167 }

{  35 −167  275 −342  358 −322  238 −119  −17  151 −263  336 −359  32930 −251  1356 }

{−358  336 −285  211 −119   17   86 −182  263 −322  354 −357  32930 −275  197 −103 }

{ −17   86 −151  211 −263  305 −336  354 −359  351 −32930  296 −251  197 −1356   69 }

{ 359 −357  351 −342  32930 −314  296 −275  251 −225  197 −167  1356 −103   69  −35 }

},

# Conclusions

This proposal provides a fast DST-7/DCT-8 method which supports dual implementations (identical output between matrix multiplication and fast method). With the proposed fast method, it is reported that the number of multiplication operations is reduced by 50% and 40% for 16-point and 32-point DST-7/DCT-8, respectively. The experimental results show reduced encoding and decoding time, with almost no change on BD-Rate. It is recommended to adopt this proposed scheme into VTM.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” JVET-K1001, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- Z. Zhang, X. Zhao, X. Li, S. Liu, “CE6-related: Fast DST-7/DCT-8 with dual implementation support,” JVET-K0291, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).