| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0291-v3 |
| --- | --- |

| Title: | CE3 Related : Extended reference sample construction for longer interpolation filter in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com jin78.heo@lge.com jangwon84.choi@lge.com l.li@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In HEVC (and JEM), only a bi-linear interpolation filter was used for prediction of sub-pel position. However, in CE3-Intra prediction and mode coding, longer interpolation filters (i.e., 4-tap and 6-tap filters) are being considered. In the conventional way of reference sample construction, it is not available for the very-end predicted samples (some right bottom samples in a currently predicted block) to refer to the reconstructed samples in the neighbor blocks. In this contribution, an extension of a reference sample construction is proposed for longer interpolation filter. Specifically, the number of the reference samples is extended from the neighbor blocks. The experimental results are reportedly shown that 0.04% and 0.01% BD-rate coding gains from VTM-AI and VTM-RA respectively.

Proposed method

In HEVC and VTM, the number of reference samples from neighbor blocks is defined as 2 * width + 1 and 2 * height +1 for above reference samples and left reference samples, respectively. The length of the reference is sufficient even when the intra prediction mode has the largest angle for the right bottom sample in the predicted block, like shown in Figure 1. However, when the longer interpolation filters are applied than bi-linear interpolation filter, the number of reference samples in the current method is not sufficient. As depicted in Figure 2, when the sub-pel position is between the very end reference sample and its adjacent neighbor reference sample, it is not necessary to increase the number of the reference samples for the bi-linear interpolation, while another outer reference sample is required if the 4-tap interpolation filter is applied.

Generally, as the filter size filtSize (where filtSize > 2) increases, the number of reference samples are to be extended as follows :

Sizeref(above)=Sizeref(above)+filtSize ≫1-1

Sizeref(left)=Sizeref(left)+filtSize ≫1-1

(a)

(b)

Figure 1 Reference sample position when the intra prediction angle is large, (a) for square blocks (b) for blocks with wide-angle prediction

(a)

Figure 2 Reference samples for interpolation (a) bi-linear filter (b) 4-tap filter

Spec text

Changes are marked with orange color.

##### 8.2.4.2.1 General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable filtSize specifying the size of intra interpolation filter.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable whRatio is set equal to Min( Abs( Log2( nTbW / nTbH ) ), 2 ).

The variables refW and refH are derived as follows:

refW = ( ( nTbH > nTbW )  ?  ( nTbW + ( nTbH >> whRatio ) + Ceil( nTbH / 32 ) )  :  ( nTbW * 2 ) ) +( filtSize >> 1) −1 	(2-1)

refH = ( ( nTbW > nTbH )  ?  ( nTbH + ( nTbW >> whRatio ) + Ceil( nTbW / 32 ) )  :  ( nTbH * 2 ) ) +( filtSize >> 1) −1 	(2-2)

For the generation of the reference samples p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.2.4.2.2 is invoked with the sample location ( xTbCmp, yTbCmp ), the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.2.4.2.3 is invoked with the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

- The reference sample filtering process as specified in clause 8.2.4.2.4 is invoked with the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output.

(…)

Experimental results

The proposed method has been implemented using BMS-2.0.1 with VTM configurations as software base, and experimentally evaluated for VTM test according to the common test conditions defined in JVET-K1010 [1]. Table 1 is the result based on VTM-2.0.1, and Table 2 shows the coding gain from the proposed method based on the software that intra 4-tap interpolation filter enabled (privileged in VTM but disabled by default).

Table 1 Test results based on BMS-2.0.1 (with VTM config.)

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 AI |  |  | Over VTM-2.0.1 RA |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.18% | -0.18% | -0.16% | -0.13% | -0.19% | -0.08% |
| Class A2 | -0.14% | -0.03% | -0.12% | -0.07% | -0.01% | -0.04% |
| Class B | -0.40% | -0.42% | -0.36% | -0.20% | -0.15% | -0.03% |
| Class C | -0.64% | -0.82% | -0.78% | -0.26% | -0.19% | -0.18% |
| Class E | -0.57% | -0.66% | -0.69% |  |  |  |
| Overall | -0.40% | -0.44% | -0.43% | -0.17% | -0.14% | -0.08% |
| Class D | -0.52% | -0.39% | -0.60% | -0.27% | -0.40% | -0.16% |
| Enc Time [%] | 102% |  |  | 102% |  |  |
| Dec Time [%] | 101% |  |  | 104% |  |  |

Table 2 Test results based on BMS-2.0.1, Intra4Tap enabled (with VTM config.)

|  | All Intra Main10 – VTM configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 AI Intra4Tap=1 |  |  | Over VTM-2.0.1 RA Intra4Tap=1 |  |  |
|  | Y | U | V | Y | U | V |
| Class A1 | -0.02% | -0.11% | -0.11% | 0.02% | 0.05% | 0.07% |
| Class A2 | 0.00% | 0.12% | 0.05% | 0.01% | 0.29% | 0.22% |
| Class B | -0.04% | -0.03% | 0.00% | -0.04% | 0.18% | 0.22% |
| Class C | -0.08% | 0.03% | 0.04% | -0.01% | 0.15% | 0.35% |
| Class E | -0.08% | -0.34% | -0.15% |  |  |  |
| Overall | -0.04% | -0.06% | -0.02% | -0.01% | 0.17% | 0.22% |
| Class D | -0.01% | 0.06% | 0.02% | -0.03% | 0.27% | 0.05% |
| Enc Time [%] | 100% |  |  | 100% |  |  |
| Dec Time [%] | 100% |  |  | 100% |  |  |

.

Conclusion

In this proposal, an extension of a reference sample construction is proposed for longer interpolation filter. From the experimental results, it is reportedly shown that 0.04% and 0.01% BD rate savings are achieved from VTM-AI and VTM-RA respectively. It is recommended to consider the proposed method to be accepted for the next version of VTM, if the longer intra interpolation filter is adopted which is discussed in the CE.

References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” Document of Joint Video Experts Team (JVET), JVET-K1023, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- G. Van der Auwera, J. Heo, A. Filippov, “CE3: Summary report on intra prediction and mode coding,” Document of Joint Video Experts Team (JVET), JVET-L0023, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).