| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0298-v2 |
| --- | --- |

| Title: | CE4: Bilinear Motion Vector Prediction (Test 4.5.2, Test 4.5.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Frank Bossen Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8492 choib@sharplabs.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE4 tests 4.5.2 and 4.5.3. These CE tests have been conducted to evaluate the performance of Bilinear Motion Vector Prediction (BiMVP) proposed in JVET-K0302. The motivation for these tests is to exploit the linearity of motion vectors in high resolution sequences. In the proposal, the motion vector difference (MVD) and motion vector predictor (MVP) for list1 are derived from those of list0. This is asserted to have a benefit of saving the bits to explicitly represent MVD and MVP for list1. In addition, bits for ref_idx and ref_pred_idc are also saved by using the first reference picture and the bi-directional prediction only. The average BD Rate performance for the tests relative to VTM-2.0.1 is as follows:

| Random Access Main (versus VTM-2.0.1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Enc | Dec |
| CE 4.5.2.a | -0.41% | -0.49% | -0.59% | 106% | 99% |
| CE 4.5.2.c | -0.46% | -0.55% | -0.68% | 112% | 99% |
| CE 4.5.3.b | -0.47% | -0.58% | -0.75% | 114% | 99% |
| CE 4.5.3.c | -0.48% | -0.56% | -0.74% | 122% | 99% |

# Introduction

In the current VVC draft [3], the inter-prediction mode signals a set of syntax elements, including ref_pred_idc, ref_idx, mvp_flag and a coded motion vector difference (MVD). These syntax elements may be used to signal an arbitrary motion vector, while merge mode is used to indicate restricted motion vectors that are derived from adjacent motion vectors. In many cases, however, the parameters for list1 are similar to those in list0, as many video sequences have mainly linear motions across multiple pictures. By using the linear property of a motion vector, some redundant bits to represent the same (or similar) motions for bi-prediction can be saved. In the current VVC, for example, ref_idx, mvp_flag and coded mvd are signaled for both list0 and list1. When the current motion is linear between the backward and forward reference pictures and current picture, then ref_idx, mvp_flag and coded MVD for list1 need not to be signalled.  Furthermore, the motion vector predictor (MVP) and MVD for list1 can be derived from those parameters for list0.

# Syntax design and MV derivation process

The proposed BiMVP mode is another type of AMVP. Keeping the existing inter-prediction mode, a flag (bilateral_mode_flag) is employed to activate the BiMVP. bilateral_mode_flag equal to 1 indicates the BiMVP is applied for the current CU, while bilateral_mode_flag equal to 0 indicates the existing AMVP mode is done. When activated, BiMVP mode does not signal ref_idx and ref_pred_idc.  Instead, the first reference pictures whose ref_idx values are equal to 0 for both list0 and list are used for bi-prediction. Uni-prediction is not allowed.

When bilateral_mode_flag is equal to 1, an additional flag (symmetric_flag) is signalled to indicate whether the BiMVP for the current CU is the Symmetric BiMVP mode or the Asymmetric BiMVP mode. The Symmetric and Asymmetric BiMVP modes are explained in the following sub-sections.

Fig. 1. Syntax design for BiMVP mode

## Symmetric BiMVP mode

In the symmetric BiMVP mode, only mvp_flag and coded MVD for list0 are signaled. Then, both MV predictor (V1x, V1y) and MV difference (offset_x, offset_y) for list1 are derived from MVP = (V0x, V1y) and MVD = (offset_x, offset_y) for list0 by mirroring values. For example, when the forward MV is equal to (Vx + offset_x, Vy + offset_y), its paired backward MV is derived as equal to (-Vx - offset_x, -Vy - offset_y). Since the derived MV for list1 is exactly symmetric with MV for list1, it is called Symmetric BiMVP mode.

Fig. 2. Illustration of symmetric BiMVP

## Asymmetric BiMVP mode

In the asymmetric BiMVP mode, mvp_flag, coded MVD for list0 and mvp_flag for list1 are signaled. This means that the MVP for list1 is not derived from the MVP for list0; instead, it is explicitly indicated by signaling. For example, when the MV for list0 is equal to (V0x + offset_x, V0y + offset_y), its paired MV for list1 is set equal to (V1x - offset_x, V1y - offset_y). Note that both (V0x, V0y) and (V1x, V1y) are determined by the values of mvp_flag. Even though it is called Asymmetric BiMVP mode in this contribution, its MVD values are symmetric.

Fig. 3. Illustration of asymmetric BiMVP

# Implementation with MV compression/reconstruction process

In order to improve coding gains and reduce complexity, several implementation issues have been considered. First, the symmetric BiMVP mode is disabled in case that the current POC value is greater than the POC values of the first reference pictures for both list0 and list1. Hence, the symmetric_flag is not signaled and instead is inferred to be equal to 0 in this case. Second, BiMVP can have only a MVD value among a set of predefined MVD values in integer-pel resolution in the current implementation. Since we observed that the MVD values of BiMVP mode are not widely distributed, only representative integer-pel MVD values are allowed. In the view-point of implementation, this modification employs motion vector compression and reconstruction processes, which are intended to reduce the MVD bits and the encoding complexity.  The detailed processes of motion vector compression and reconstruction process are given in the following sub-sections:

## MV compression process

Input : uiUncompressedMVAbs (Reconstructed Absolute MVD value of X or Y) Output : uiCompressedMVAbs (Compressed Absolute MVD value of X or Y) if (uiUncompressedMVAbs < 3) uiCompressedMVAbs = uiUncompressedMVAbs else { a = log2(uiUncompressedMVAbs) if ((1 << a) == uiUncompressedMVAbs) uiCompressedMVAbs = 2*a - 1; else if (a > 1 && (3 << (a- 1)) == uiUncompressedMVAbs) uiCompressedMVAbs = 2*a; else return -1; // invalid } return uiCompressedMVAbs

## MV reconstruction process

Input : uiCompressedMVAbs (Compressed Absolute MVD value of X or Y) Output : uiUncompressedMVAbs (Reconstructed Absolute MVD value of X or Y) if (uiCompressedMVAbs < 3) uiUncompressedMVAbs = uiCompressedMVAbs; else if (uiCompressedMVAbs % 2) uiUncompressedMVAbs = 4 << ((uiCompressedMVAbs - 3) >> 1); else { uiUncompressedMVAbs = 4 << ((uiCompressedMVAbs - 3) >> 1); uiUncompressedMVAbs += uiUncompressedMVAbs >> 1; } return uiUncompressedMVAbs;

# Core Experimental Results

Each CE test was tested and evaluated with VTM-2.0.1 used as anchors. The summary results for each test is given below, while the full results are available in the attached excel file.

| Test # | Description |
| --- | --- |
| 4.5.2 | - Asymmetric BiMVP mode - a)+ Symmetric BiMVP mode |
| 4.5.3 | b) Combination of MVD compression/reconstruction and 4.5.2.a c) Combination of MVD compression/reconstruction and 4.5.2.c |

## CE4.5.2.a

## CE4.5.2.c

## CE4.5.3.b

## CE4.5.3.c

# Conclusion

The “BiMVP” mode was tested and evaluated in CE4.  The method consists of symmetric and asymmetric modes that derive list1 information directly from list0. It is demonstrated from experiments that the proposed method shows 0.41% gain with 106% encoding run time on average, and with increasing gains for higher resolution sequences.  We recommend the adoption of BiMVP mode in the next draft of VVC.

# References

[1] JVET-K1024 “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” H. Yang, S. Liu, K. Zhang; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[2] JVET-K0353 “CE4-related: Bilateral Motion Vector Prediction,” B. Choi, F. Bossen, A. Segall; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[3] JVET-K1001 “Versatile Video Coding (Draft 2),” B. Bross, J. Chen, S. Liu; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).