| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0302-v1 |
| --- | --- |

| Title: | CE4-related: History based spatial-temporal MV prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, a history based spatial-temporal merge mode using previously coded MV and temporal MV combination (H-STMVP) is proposed. Two spatial and one temporal motion vectors are averaged to form this predictor in merge candidate list. The spatial candidates come from the last two coded MVs in decoding order while the temporal candidate comes from TMVP as currently used in VVC. With the same number of merge candidates used (6 in current VTM-2), it is reported that the proposed method achieves 0.36%/0.xx% BD rate saving in RA and LB configurations, respectively.

# Introduction

A block based spatial-temporal merge mode is proposed in this document, referred as H-STMVP. Different from the method in [1], where the two spatial candidates are derived from current block’s spatial non-adjacent or adjacent neighbors, the two spatial candidates in the proposed method (H-STMVP) are from the last two coded MVs in decoder order prior to the current block.

With the three MV candidates checked (last coded MV: mv_Last1; second last coded MV: mv_Last2; TMVP: MV: mv_T), the final merge candidate mv is:

- if all three are available:

    - mv = ((mv_Last1 + mv_Last2 + mv_T) * 43) / 128

- if two candidates are available:

    - mv is the average of the two

- if only one candidate is available:

    - mv is the available one

- if no candidate is available:

    - this mode does not apply

When applicable, the proposed H-STMVP mode is placed after ATMVP candidate in the merge list.

This document reports coding efficiency of the proposed method in comparison to VTM-2.

# Simulation results

The proposed method is implemented using VTM-2.0.1. The simulation is performed using VTM common test condition [2]. The number of merge candidates used is set to be 6, which is also currently used in VTM-2.0.1. Results are shown in the following table:

Table I: coding performance comparison of the proposed method on top of VTM-2.0.1

# Conclusion

In this contribution, a history based spatial-temporal MV prediction method called H-STMVP is proposed. The last two coded MVs and a temporal MV predictor are averaged to form this predictor MV. It is recommended that the proposed method is further studied in CE.

# References

[1] T. Zhou and T. Ikai, “Spatial-temporal merge mode (non subblock STMVP)”, JVET-K0532, Ljubljana, SL, July 2018

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).