| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0305 |
| --- | --- |

| Title: | CE4-related: History Based Affine Merge Candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Seethal Paluri, Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA, 92131, U.S.A. Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 661 Bryant St, Palo Alto, CA 94301, U.S.A. | Tel: Email: Tel: Email: | 1-201-816-2000 jie.zhao@lge.com +1-650-798-3300 guichunli@tencent.com |
| Source: | LG Electronics. & Tencent |  |  |

_____________________________

# Abstract

This proposal presents a history based affine merge candidate (Affine HMVP) method for affine merge list generation. A table to store motion information of coded affine CUs is maintained at slice level, and is updated after coding an affine coded CU. Then affine HMVP candidate is applied in affine merge list generation.

Comparing to CE4.2.1 affine merge common base, by maintaining only 1 affine HMVP candidate, results show that there is 0.12% and 0.04% gain for RA and LD test conditions. By maintaining 8 affine HMVP candidates, there is 0.23% and 0.xx% gain for RA and LD test conditions. In the test, merge list size and other affine merge candidates are kept unchanged, and the affine HMVP candidate is inserted after the inherited and constructed affine merge candidate.

It is asserted that, the history based affine candidate has lower complexity than deriving inherited or constructed affine merge candidate. Therefore this history based affine merge candidate may be combined with other affine merge simplification methods, and reduce the overall complexity.

# Proposed method

The concept of the proposed history based Affine merge candidate is similar to the History Based Motion Vector Prediction (HMVP) in CE4.4.7 [1], which maintains a table for non-affine coded CU and uses the history MV in regular merge list and/or AMVP list generation.

In this proposal, a table is maintained for affine coded CUs at slice level. For each entry of Affine history table, the following motion information is stored.

- Affine control point motion vectors (CPMVs) mv0, mv1, mv2

- Inter prediction direction (L0, L1 or Bi-pred)

- Reference index of List 0 and/or List1

- affine type, whether 4-parameter and 6-parameteer affine

The memory requirement for each entry of the affine history table can be calculated as:

- CPMVs for 2 reference list: 4 bytes * 2 * 3

- Reference index for 2 lists: 1 byte * 2

- Inter Direction: 1 byte

- Affine Type: 1 byte

- Total: 4*3*2 + 1*2+1+1 = 28 bytes

Below is the total memory requirement for each affine history table size:

- Table size 1: 28 bytes;

- Table size 4: 112 bytes;

- Table size 8: 224 bytes;

The table for history affine MV information is organized as first-in-first out. When the table is full, the oldest history is pushed out. In our test, during affine merge list generation, the history affine MV is inserted after the constructed affine MV candidate and before zero MV candidate.

mv0

mv1

mv2

Fig. 1. MVs in affine coded CU

Size of the history CU is not stored.  Based on our testing, scaling the history affine MV according to the history CU size and the current CU size has very little gain.  So for simplicity, the history affine CPMVs are inserted directly into affine merge list without scaling. Same pruning process as the pruning for inherited and constructed affine merge candidates is applied.

It is asserted that, the history based affine candidate has lower complexity than deriving inherited or constructed affine merge candidate. For inherited and constructed affine merge candidate, some comparisons have to be done to derive the reference index and MVs, while for history affine MV, it does not need those derivations. In addition, it doesn’t depend on motion data line buffer for blocks adjacent to the CTU top boundary. Therefore this history based affine merge candidate may be combined with other simplification method, and reduce overall complexity.

# Experimental results

The proposed method has been implemented into CE4.2.1 affine common base [2], and tested using JVET SDR common test condition [3].

We tested maintaining the affine history table size of 1 and 8 candidates, and results are shown in the tables below. In the test, the inherited and constructed affine candidates are kept unchanged. Number of candidate in the final affine merge list is kept as 5.  As shown in the table, relative to affine common base, maintaining 1 history affine MV gives 0.12% gain for RA, and 0.04% gain for LD. Adding 8 history affine MV gives 0.23% gain for RA, and 0.xx% gain for LD.

Table 1: Experimental results of maintaining 1 Affine HMVP candidate ( random access.)

| RA | Over VTM 2.0.1 |  |  |  |  | Over VTM 2.0.1 Affine Common Base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.08% | -0.78% | -1.03% | 106% | 110% | -0.11% | -0.20% | -0.14% | 102% | 105% |
| Class A2 | -1.10% | -1.13% | -1.23% | 108% | 110% | -0.09% | -0.15% | -0.09% | 102% | 104% |
| Class B | -0.84% | -0.74% | -0.89% | 110% | 111% | -0.17% | -0.17% | -0.19% | 102% | 104% |
| Class C | -0.55% | -0.45% | -0.63% | 114% | 123% | -0.07% | 0.01% | -0.09% | 102% | 107% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.86% | -0.75% | -0.92% | 110% | 114% | -0.12% | -0.13% | -0.13% | 102% | 105% |
| Class D | -0.75% | -0.66% | -0.75% | 114% | 135% | -0.06% | 0.01% | 0.03% | 102% | 111% |

Table 2: Experimental results of maintaining 1 Affine HMVP candidate ( low delay)

| LD | Over VTM 2.0.1 |  |  |  |  | Over VTM 2.0.1 Affine Common Base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.44% | -0.40% | -0.25% | 109% | 108% | -0.06% | -0.12% | 0.39% | 105% | 110% |
| Class C | -0.38% | -0.11% | -0.42% | 110% | 110% | -0.02% | 0.27% | -0.16% | 102% | 107% |
| Class E | -0.68% | -0.42% | -0.91% | 112% | 114% | -0.04% | 0.21% | -0.72% | 102% | 107% |
| Overall | -0.48% | -0.31% | -0.47% | 110% | 110% | -0.04% | 0.09% | -0.07% | 103% | 108% |
| Class D | -0.58% | -0.41% | -0.62% | 113% | 123% | 0.02% | 0.02% | -0.02% | 102% | 113% |

Table 3: Experimental results of maintaining 8 Affine HMVP candidate ( random access.)

| RA | Over VTM 2.0.1 |  |  |  |  | Over VTM 2.0.1 Affine Common Base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.16% | -0.81% | -1.17% | 105% | 110% | -0.20% | -0.23% | -0.28% | 101% | 104% |
| Class A2 | -1.16% | -1.22% | -1.37% | 105% | 108% | -0.16% | -0.24% | -0.24% | 100% | 103% |
| Class B | -1.00% | -0.88% | -0.94% | 105% | 104% | -0.33% | -0.31% | -0.24% | 97% | 98% |
| Class C | -0.64% | -0.51% | -0.65% | 110% | 113% | -0.16% | -0.05% | -0.11% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.97% | -0.84% | -1.00% | 107% | 108% | -0.23% | -0.21% | -0.21% | 99% | 100% |
| Class D | -0.76% | -0.67% | -0.81% | 111% | 127% | -0.08% | 0.00% | -0.03% | 99% | 104% |

Table 4: Experimental results of maintaining 8 Affine HMVP  ( low delay)

| LD | Over VTM 2.0.1 |  |  |  |  | Over VTM 2.0.1 Affine Common Base |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  | #NUM! | #NUM! |  |  |  | #NUM! | #NUM! |
| Class C | -0.40% | -0.36% | -0.45% | 106% | 107% | -0.05% | 0.02% | -0.18% | 98% | 104% |
| Class E | -0.83% | -1.24% | -0.80% | 108% | 109% | -0.20% | -0.61% | -0.61% | 98% | 103% |
| Overall |  |  |  | #NUM! | #NUM! |  |  |  | #NUM! | #NUM! |
| Class D | -0.60% | -0.29% | -0.38% | 107% | 108% | 0.00% | 0.15% | 0.22% | 97% | 99% |

# Conclusion

A history based affine merge candidate is proposed. Experimental results show that by maintaining only 1 affine HMVP, there is 0.12% and 0.04% gain for RA and LD test conditions. By maintaining 8 affine HMVP, there is 0.23% and 0.xx% gain for RA and LD test conditions.

It is asserted that history based affine MV has lower complexity than generating inherited and constructed affine merge candidates. And it doesn’t have dependency to the affine motion line buffer. Therefore, we recommend to combine the history based affine candidate with other affine simplification methods, and perform further studies in CE.

# Reference

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Dong, “CE4-related: History-based Motion Vector Prediction,” JVET-K0104, Ljubljana, SI, July 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-K1024, Ljubljana, SI, July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010 Ljubljana, SI, July 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).