| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0306-v1 |
| --- | --- |

| Title: | On slices and tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia (Tampere, Finland) | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes that

- The concept of raster-scan-order slices is removed from VVC.

- The concept of tiles (as known in HEVC) is included in VVC.

- A tile grid is indicated in the PPS as in HEVC.

- A slice contains one or more complete tiles in row-major order of tiles.

- The slice header contains a tile_id value indicating the first tile in the slice in decoding order. slice_address is proposed to be removed from the slice header.

- Rather than being present after each CTU, end_of_slice_flag is present only after each tile in slice_data( ).

# Introduction

This contribution proposes specification text that is aligned with the high-level syntax design choices proposed in JVET-L0110.

This contribution is not as versatile and comprehensive as the proposal in JVET-L0114. Furthermore, JVET-L0114 takes a step towards meeting some of the design goals for tiles suggested in JVET-L0182. The proponent therefore prefers JVET-L0114 over this contribution and suggests that this contribution should only be considered if some aspects of JVET-L0114 are not acceptable for adoption.

# Proposal

This section contains the proposed changes relative to JVET-K1001-v5.

## Picture parameter set

HEVC syntax for the tile grid is proposed as a starting point, as indicated by red font below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics are identical to those in HEVC. HEVC clause 6.5.1 (coding tree block raster and tile scanning conversion process) is included with local variables colWidth and rowHeight turned to global variables ColWidth and RowHeight, respectively. In addition, the following variables are derived:

- If tiles_enabled_flag is equal to 0, NumTileColumns, NumTileRows, and NumTilesInPic are all set equal to 1.

- Otherwise, the following applies:

  - NumTileColumns is set equal to num_tile_columns_minus1 + 1.

  - NumTileRows is set equal to num_tile_rows_minus1 + 1.

  - NumTilesInPic is set equal to NumTileColumns * NumTileRows.

## Slice header

Since slice headers are aligned with tile boundaries, slice_address is replaced with tile_id that specifies the position in the tile grid.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| if( tiles_enabled_flag ) |  |
| tile_id | u(v) |
| // the remaining of slice_header( ) is unchanged |  |

tile_id is used to determine the address of the first coding tree block in the slice segment. The length of the tile_id syntax element is Ceil( Log2( NumTilesInPic ) ) bits. The value of tile_id shall be in the range of 0 to NumTilesInPic − 1, inclusive. The value of tile_id shall not be equal to the value of tile_id of any other coded slice segment NAL unit of the same coded picture. When tile_id is not present, it is inferred to be equal to 0.

The variable CtbAddrInTs, specifying a coding tree block address in tile scan, is derived as follows:

tileRow = tile_id / NumTileColumns
tileCol = tile_id % NumTileColumns
CtbAddrInTs = 0
for( i = 0; i  <  tileRow; i++ )		(x-y)
	CtbAddrInTs += PicWidthInCtbsY * RowHeight[ i ]
for( i = 0; i < tileCol; i++ )
	CtbAddrInTs += ColWidth[ i ] * RowHeight[ tileRow ]

## Slice data

The proposed changes cause CTUs to be scanned in tile scan order and the end_of_slice_flag to be present only after the last CTU of a tile.

| slice_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| CtbAddrInTs++ |  |
| CtbAddrInRs = CtbAddrTsToRs[ CtbAddrInTs ] |  |
| if( CtbAddrInTs = = PicSizeInCtbsY ¦ ¦ ( tiles_enabled_flag && TileId[ CtbAddrInTs ] != TileId[ CtbAddrInTs − 1 ] ) ) |  |
| end_of_slice_flag | ae(v) |
| end_of_slice_flag | ae(v) |
| CtbAddrInRs++ |  |
| } while( !end_of_slice_flag ) |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).